3GPP TSG-RAN WG4 Meeting #116bis			                        R4-2513130

Prague, CZ, August 13rd – 17th, 2025

Agenda item:	8.12

Source: 	CMCC

Title: 	Discussion on 6GR Testability and OTA

Document for:	Discussion

# 1. Introduction

In RAN plenary #108 meeting, the SID for 6G has been approved. 6G-related RAN4 work will officially begin at this meeting. According to the agenda released by the RAN4 chair, the 6G activities have been split into multiple parts.

With the introduction of new spectrum, new scenarios, and new requirements for 6GR, both testability and OTA requirements need to be updated accordingly. This contribution will focus on testability and OTA for future 6G work.

- Discussion

### 2.1 Testability

The first consideration is the impact of the introduction of new spectrum on testability. Currently, RAN1 has retained the following three options for the categorization of 6GR spectrum.

• Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology. • Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family. • Option 3: define separate FR for 7.125 – 24.25 GHz.

The discussion on 6G frequency categorization is still ongoing, especially the issue whether to define a new separate FR has not yet reached a conclusion.

Therefore, in terms of testability, it is more necessary to consider from the perspective of frequency characteristics. If option 2 is adopted for frequency categorization, the testability will most likely be consistent with the corresponding FR. For the frequency around 7GHz, considering that its frequency range is closer to FR1, it is recommended that its testability use FR1 conduction test as the baseline.

Proposal 1: For the testability around 7GHz, FR1 conduction test should be used as the baseline.

If option 1 or option 3 is adopted for frequency categorization, a new separate FR will be defined. In this new FR, for the frequency below 10GHz (such as the frequency around 7GHz), on the one hand, considering that its frequency range is closer to FR1, and on the other hand, since the conduction test cycle is short and cheap, it is recommended that its test still use FR1 conduction test as the baseline. For frequencies above 15GHz, such as Ku bands, the test method can consider using FR2 OTA test as the baseline.

Proposal 2: If separate FR is defined for 6GR,

- For frequency bands below [10GHz] (the frequency around 7GHz), it is suggested that the testability should take the conduction test of FR1 as the baseline.

- For frequency bands above [15GHz] (Ku bands), consider using the OTA test of FR2 as the baseline.

In addition to the new frequencies, new test cases also need to be discussed. Many testing requirements for different device types were raised during 5G, such as AIOT devices and NTN UEs. Due to the differences in device architectures and characteristics, the testing requirements for these devices in some cases cannot fully match the existing testability. However, due to the high cost of setting up the testing system and the difficulty in modifying, it is difficult to adjust the testing system for these new device types.

At the current stage of discussions in RAN and RAN1, no conclusion has been reached regarding the types of devices. However, it is foreseeable that more and more new types of devices will be proposed, such as the types mentioned in the following proposal.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set. - Whether/how to have one or more device types for eMBB or 6G IoT. - Whether/how to have othe device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, Al agents, collaborative robots, etc. - Whether/how to explicitly standardize device types. - Ensuring forward compatibility. - Minimizing/avoiding potential market fragmentation. Note: the terminology “device types” subject to further discussion and possible refinement.

In 6G day1, the establishment of test cases should take into account the supported devices types, And the testing methods should be as closely aligned with the characteristics of the device as possible., such as A-IoT devices and NTN UEs.

Proposal 3: New test case should be considered in testability of 6G day1, e.g. A-IoT device, NTN UE.

In addition to considering the new test cases, the updates of testing instruments are also changing the testing requirements. Many testing requirements have been in use for a long time, and some of them are limited by the testing environment and cannot be updated. Some testing requirements were actually not used as references in actual applications. During this period, which is the initial stage of 6G, it is the best time to revisited the testing requirements.

Proposal 4: Legacy testability should be revisited in 6G day1.

For AI/ML, one important topic is generalization. The goal of generalization is to verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintained under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. In 5G-A discussion on testing environment/framework for AI/ML, both static and non-static scenarios/configurations are dicsussed. Finally, due to limited timeline and workload, static conditions are used as baseline. In detail, RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability (environment differs in each test but not changing dynamically during the test). There are some drawbacks for generalization based on static test. Firstly, the test with fixed configurations/channel conditions cannot reflect the real situation. Secondly, to guatantee the generalization performance of AI/ML model in real-world, multiple tests are needed wjoch increase the test burden. Above issues can be solved by adopting dynamic test. Dynamic test, i.e. environment changes dynamically during the test, is more realistic. And the dynamic test could help in reducing the number of tests. It is proposed to study dynamic test for AI/ML in 6G.

Proposal 5: for AI/ML, it is proposed to consider dynamic test (i.e. environment changes dynamically during the test) in 6GR.

### 2.2 OTA

For OTA test, the alignment with the deployment scenarios is an important reference. Therefore, when designing 6G OTA tests, more attention should be paid to the gap between the existing tests and the current network deployment scenarios， such as the dynamic channel model in the MIMO OTA testing. In previous MIMO OTA tests, the channel model parameters and the design of multipath were often fixed, which did not match the actual MIMO deployment scenarios.

Rel-19 conducted some research on dynamic test methodologies and eventually completed TR38.762. In the environmental conditions part of R19 TR38.762, the signal power takes into account the full UMa/UMi path loss model, and the normalized signal power with the effect of the gNB antenna is modeled. The dynamic channel model is defined at different positions of the testing device. The related research of Rel-19 has basically completed the design of the dynamic test methodology, but considering the time limit, some work still needs to be further improved, such as the performance metrics have not been fully specified.

Observation 1: Rel-19 conducted research on the dynamic test methodology for MIMO OTA.

Proposal 6: Dynamic test methodologies in MIMO OTA testing should be further studied and supported in 6G day1.

# 3. Conclusions

Proposal 1: For the testability around 7GHz, FR1 conduction test should be used as the baseline.

Proposal 2: If separate FR is defined for 6GR,

- For frequency bands below [10GHz] (the frequency around 7GHz), it is suggested that the testability should take the conduction test of FR1 as the baseline.

- For frequency bands above [15GHz] (Ku bands), consider using the OTA test of FR2 as the baseline.

Proposal 3: New test case should be considered in testability of 6G day1, e.g. A-IoT device, NTN UE.

Proposal 4: Legacy testability should be revisited in 6G day1.

Proposal 5: for AI/ML, it is proposed to consider dynamic test (i.e. environment changes dynamically during the test) in 6GR.

Observation 1: Rel-19 conducted research on the dynamic test methodology for MIMO OTA.

Proposal 6: Dynamic test methodologies in MIMO OTA testing should be further studied and supported in 6G day1.

# 4. Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)