3GPP TSG-RAN WG4 Meeting #116bis											R4-2513135

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.11

Source:	CMCC

Title:	Discussion on 6G sensing

Document for:	Discussion

- Introduction

In RAN#108 meeting, 3GPP requirements for 6G Radio for new services are approved to be developed in SID for 6G[1].

- （9）Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

According to chairman’s guideline, the scope of sensing/ISAC(Integrated Sensing and Communication,ISAC) includes RAN4 related PHY functions and procedure, RF, coexistence and testability. This contribution gives proposals on 6G RAN4.

- Discussion

- 2.1	use case and performance metric

Before detailed requirements and simulation evaluation discussion, the target use cases should be identified. ISAC is aimed to provide sensing services for low-altitude economy, traffic surveillance, intrusion detection while also assisting communication services. In RAN #109 meeting, it was agreed that the 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. These use cases shall be considered for RAN4 study.

Proposal 1: for RAN4 study, it is proposed to consider use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

For NR, usually we use throughput as performance metric for RF requirements evaluation. For sensing, now RAN plenary is discussing detailed KPIs and RAN4 is suggested to follow the performance metric discussion and select the proper metric from evaluation/testing point of view. Based on RAN plenary #109 meeting discussion, following list our view on 6G ISAC KPI, detailed are listed in [2].

Table 1 Metrics for sensing

| Metric | Definition |
| --- | --- |
| Miss detection probability | Miss detection probability is the probability that a true target is not associated with a detected target. From the aspect of evaluation, it is defined as Pmd=n=0N−1DnMnN Where, - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object - Mn is the number of true targets in the drop n. - N is total number of drops with at least one target per drop |
| False alarm probability | Definition Type 1 (no target dropped in simulation area): False alarm probability is defined as the probability that an object is detected when there is no target present in simulation area is considered a false alarm. From the aspect of evaluation, it is defined as Pf1=n=0N−1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. Definition Type 2 (targets dropped in simulation area): False alarm probability is defined as the probability that an object is detected but not associated with any true targets in the simulation area is considered as a false alarm. From the aspect of evaluation, it is defined as Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - K is number of drops (N) |
| Horizontal/Vertical Positioning Accuracy | Horizontal/vertical positioning accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical position estimation error. From the aspect of evaluation, the horizontal/vertical position estimation error is the norm of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target. |
| Velocity Accuracy | Velocity accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. From the aspect of evaluation, the velocity estimation error is the norm of the difference between the estimated velocity and the corresponding true velocity of a sensing target. |
| Sensing Capacity Note 1 | Sensing capacity is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with 95% probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing. |
| Max sensing service latency | Max sensing service latency is the time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. |
| Refreshing rate | Refreshing rate is the rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. |
| Note 1: An intuitive evaluation methodology for sensing capacity can be found in Annex A. |  |

Table 2 KPIs on sensing

| Use case | Positioning Accuracy |  | Velocity Accuracy [m/s] | Max sensing service latency [ms] | Refreshing rate [Hz] | Sensing Capacity [per sector] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal [m] | Vertical [m] |  |  |  |  |  |  |
| UAV detection | 10 | 10 | 1 | 1000 | 1-10 | 100 | 1 | 1 |
| UAV tracking | 1 | 1 | 1 | 100-1000 | 1-10 | 100 | 0.1-1 | 1 |
| Automotive detection and tracking | 1 | N/A | 1 | 1000-5000 | 10 | 100 | 1 | 3 |
| Human detection | 1 | N/A | N/A | 1000 -5000 | 10 | 100 | 5 | 2 |
| Note 1: Metrics for tracking can be different with detection. And metrics for tracking should be evaluated independently. |  |  |  |  |  |  |  |  |

Proposal 2: It is proposed to take Table 1 as stating point for metric discussion for sensing.

Proposal 3: It is proposed to take Table 2 as stating point to discuss KPI on sensing.

  - 2.2	co-existence analysis

According to 5G experience, the legacy 5G spectrum will be further re-farmed to 6G in future. During a long period, 5G and 6G will co-exist in the same area. RAN4 should at least analyze the co-existence between 5G legacy network and 6G sensing network. Besides, the co-existence between 6G sensing and 6G normal BS may also be needed which based on the frame structure and detailed interference avoidance scheme, i.e. whether 6G normal BS slice the time unit when sensing BS transmit

In 5G, only TRP mono-static sensing is supported. Thus, the performance and use cases are limited. In 6G, RAN plenary has agreed to study six sensing modes including both mono-static and bi-static with both TRP and UE involved to explore more use cases and to further improve sensing performance.

- TRP mono-static

- TRP-TRP bistatic

- TRP-UE DL

- UE-TRP UL

- UE-UE bistatic

- UE mono-static.

Now there is no down-selection of above 6 sensing modes. For all above cases, interference cancellation or spatial isolation analysis is needed for the receiver of sensing signal which should be in RAN4 scope like what we have done for SBFD and A-IoT items. Besides, the co-existence analysis between 5G and 6G is necessary.

Proposal 4: it’s RAN4’s scope for self-interference cancellation/spatial isolation analysis and the adjacent carrier co-existence simulation for following scenarios

- Between 5G legacy network and 6G sensing network

- Between 6G normal network and 6G sensing network based on detailed interference avoidance scheme.

  - 2.3	BS RF analysis

- NR defined three BS types

  - 1-C BS which is applicable mainly for less than 2GHz

  - 1-H BS which is the major BS types for FR1 larger than 2GHz.

    - The OTA part for 1-H BS only include EIRP and EIS requirements to reduce testing workload.

  - 1-O BS which only include OTA requirements and is not the major BS types among the world due to high testing workload

- For ISAC BS, narrower beam-width will have better detection/tracking accuracy. OTA requirements are more important with narrower beam-width. It’s better for RAN4 to further analyze the possibility of more OTA requirements for 1-H type BS and the trade-off between accurate testing and testing workload. From our side, we can further discuss the possibility of EVM and ACS as OTA requirements for 1-H.

- Proposal 5: RAN4 to further discuss the possibility of introduce more OTA requirements for 1-H. On the other side, OTA requirements numbers are limited to reduce testing workload.

  - 2.4 measurement

- According to the agreements in RAN #109 meeting, 6GR should study the sensing modes including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. Since UE is involved for some sensing modes, the impact on UE measurement for sensing need to be considered.

- How to perform measurement is pending on RAN1/2 design, e.g.the reference signal design. From RAN4 point of view, some high level issues can be considered. One consideration is measurement framework. One option is to take measurement for positioning as baseline, since most likely the horizontal/vertical position will be one of the performance metrics for sensing. However, other measurement may also be needed considering new metrics, e.g. velocity.

- Proposal 6: it is proposed to study the impact on UE measurement for the sensing modes involving UE.

- Conclusion

In this contribution, we provide our view on 6G sensing use cases and related KPIs.

Proposal 1: for RAN4 study, it is proposed to consider use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Proposal 2: It is proposed to take Table 1 as stating point for metric discussion for sensing.

- Proposal 3: It is proposed to take Table 2 as stating point to discuss KPI on sensing.

Proposal 4: it’s RAN4’s scope for self-interference cancellation/spatial isolation analysis and the adjacent carrier co-existence simulation for following scenarios

- Between 5G legacy network and 6G sensing network

- Between 6G normal network and 6G sensing network based on detailed interference avoidance scheme.

- Proposal 5: RAN4 to further discuss the possibility of introduce more OTA requirements for 1-H. On the other side, OTA requirements numbers are limited to reduce testing workload.

- Proposal 6: it is proposed to study the impact on UE measurement for the sensing modes involving UE.

- Reference

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108Prague, Czech Republic, June 9-13, 2025.

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108Prague, Czech Republic, June 9-13, 2025.

- RP-252088

- Minimum requirements related to technical performance for IMT-2030 radio interface(s), 2 July 2025

- 3GPP TR 22.137, 2024

- 3GPP TR 22.870, 2025

- Annex A (informative)

Evaluation methodology for sensing capacity

| Step 1 | Initialize evaluation parameters. Decide the number of targets dropped. |
| --- | --- |
| Step 2 | Implement Mont-Carlo system-level simulations with the given number of targets. Generate sensing transmit signal. Obtain sensing received signal and perform signal processing to estimate the interested parameters of all targets. |
| Step 3 | Count the cases that all targets fulfil QoS requirements (depending on use case). Calculate the probability that all targets fulfil QoS requirements. |
| Step 4 | If necessary, update the number of targets dropped and repeat Step.1 to Step.3, until the probability that all targets fulfil QoS requirements equalling to 95%. |

- Annex B (informative)

Requirements from TR 22.870

Table 7.4.6-1:	Performance requirements of sensing results for low-altitude UAV supervision

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |
| Low-altitude UAV supervision | UAV intrusion detection | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | UAV trajectory tracking | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |

Table 7.6.6-1:  Performance requirements for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （note 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 vehicles Per 1000 m x 24 m ≤100 pedestrian per 1000 m x 24 m (note 1) | Static/moving, up to 5 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1 m/s | ≤85 vehicles per 1000 m x 24 m (note 2) | Moving, up to 120 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.17.6-1:	Performance requirements for Safe and Economic UAV transport

| Scenario | Sensing service availability [%] | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing Resolution |  | Max sensing service latency [ms] | Refreshing Rate [s] | Missed Detection [%] | False Alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safe and economic UAV transport | 99.999 | 99.99 | ≤ 50 | 50 | 5 | 5 | 10 | 0.5 | ≤ 100 | 0.1 - 1 | 0.1% (notes 2, 3) | 1% (note 2) |
| NOTE 1: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 2: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 3: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. |  |  |  |  |  |  |  |  |  |  |  |  |