3GPP TSG-RAN WG4 Meeting #116bis                      R4-2513142                                                                                                       Prague, CZ, 13th – 17th October 2025

Source:	China Telecom

Title:	Discussion on UE requirements for 6Rx UE with interference

Agenda Item:	7.5.4

Document for:	Discussion

Introduction

In RAN #109 meeting, the new WID on NR demodulation performance: Phase6 was approved [1], with the following objectives for the UE requirements for 6Rx UE with interference:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In this contribution, our views on UE requirements for 6Rx UE with interference is provided.

Discussion

In the following table, we have summarized the key parameters used for MMSE-IRC requirements defined for 2/4Rx UE in Rel-17:

Table 1. Key parameters used for Rel-17 MMSE-IRC requirements for 2/4Rx UE

- PDCSH requirements with inter cell interference - Receiver type: - MMSE-IRC - Network type: - Synchronized network only - Cover both HomNet and Hetnet scenarios with the following applicability rule [2] - ‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’ - Interference profile: INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet - INR1 = 7.58dB for HetNet - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Test and PDSCH configuration - TDLC300-100, MCS 13, 2x2 ULA Low, rank1 for HomNet - TDLA-30-10, MCS 13, 2x2 ULA Low, rank1 for HetNet - PDCSH requirements with intra cell inter user interference - Receiver type: - MMSE-IRC - 1 co-scheduled UE with random 16QAM symbols - Rank configuration: - Both rank 1+1 and rank 2+2 are covered - Precoding method: - Target UE: Random precoding - Co-scheduled UE: - Random precoding for MMSE-IRC tests with rank 1+1; - Orthogonal precoding with the target UE for MMSE-IRC tests with rank 2+2 - Test and PDSCH configuration for the target UE - TDLC300-100, MCS 13, 2x2 and 2x4 ULA Low for MMSE-IRC tests with rank 1+1 - TDLA30-10, MCS 13, 4x4 ULA Low for MMSE-IRC tests with rank 2+2 - DMRS port allocation between target and co-scheduled UEs: - Same DMRS group for rank 1+1 test, i.e., {1000} for target and {1001} for co-UE - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE

In addition, RAN4 has also defined enhanced 8Rx demodulation requirements with interference with the following scopes and parameters.

- PDCSH requirements with inter cell interference - Receiver type, Network type, Interference profile and other parameters are same as 2/4Rx UE cases - PDSCH configuration for the serving cell - TDLA-30-10, MCS 17, 2x8 ULA Low, rank2 for HomNet - TDLA-30-10, MCS 13, 4x8 ULA Low, rank4 for HetNet - PDCSH requirements with intra cell inter user interference - Receiver type, co-scheduled UE number and other parameters are same as 2/4Rx cases - Rank configuration: Rank 2+2 - Precoding method: - Target UE: Random precoding - Co-scheduled UE: Orthogonal precoding - Test and PDSCH configuration for the target UE - TDLA30-10, MCS 17, 4x8 ULA Low - DMRS port allocation between target and co-scheduled UEs: - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE

General

UE type, Test applicability rule and release independent

MMSE-IRC is the basic receiver assumption for all 6Rx UE. Therefore, the new requirements for 6Rx MMSE-IRC receiver should also be mandatory requirements for all 6Rx UEs without additional test applicability rule.

It is clearly captured in the WID that the new 6Rx UE requirements with interference apply for both handheld and FWA UEs. However, within the discussion of the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’, it is undecided whether to define one set or two sets of requirements for handheld and FWA UEs. Therefore, we propose to postpone this issue until there is clear decision in WI ‘NR_ENDC_RF_Ph4-Perf’,

Proposal 1: The new requirements for 6Rx MMSE-IRC receiver should be mandatory for all 6Rx UEs without additional test applicability rule.

Observation 1: It is undecided whether to define one set or two sets of requirements for handheld and FWA UEs in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Proposal 2: FFS how to cover both handheld and FWA UEs until there is clear agreements in Rel-19.

In addition, it is unclear whether there will be release independent rules for the basic 6Rx requirements. We propose to further discuss the release independent issue.

Proposal 3: RAN4 should further discuss the release independent issue after there is conclusion in Rel-19.

PDCSH requirements with inter cell interference

Test scenarios and interference profile

For the test scenario, we propose to align with the existing 2/4/8Rx cases:

- Cover both HomNet and Hetnet scenarios with the following applicability rule: - ‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’

As this WI targets to define requirements for 6Rx UE (both handheld and FWA), we propose to reuse the existing INR values for Rel-17 2/4Rx and Rel-19 8Rx to avoid extra system level simulation work.

- INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet - INR1 = 7.58dB for HetNet - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s)

Proposal 4: Cover both HomNet and Hetnet scenarios with the following applicability rule: If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Proposal 5: Reuse the same interference power levels (i.e., INR values) defined for Rel-17 2/4Rx and Rel-19 8Rx for 6Rx requirements with inter cell interference.

PDSCH configuration for the serving cell

As a start point, we propose to reuse the same configuration for 8Rx requirements:

For the antenna correlation, we propose to follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Proposal 6: Reuse the same PDSCH configurations for 8Rx requirements

- PDSCH configuration for the serving cell - TDLA-30-10, MCS 17, 2x6, rank2 for HomNet - TDLA-30-10, MCS 13, 4x6, rank4 for HetNet

Proposal 7: For the antenna correlation, follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

PDCSH requirements with intra cell inter user interference

For MU-MIMO requirements for 6Rx, we propose to use the existing test parameters for 8Rx as a start point.

Proposal 8: Use the existing test parameters for 8Rx as a start point as shown below:

- PDCSH requirements with intra cell inter user interference - 1 Co-scheduled UE number - Rank configuration: Rank 2+2 - Precoding method: - Target UE: Random precoding - Co-scheduled UE: Orthogonal precoding - Test and PDSCH configuration for the target UE - TDLA30-10, MCS 17, 4x6, FFS on the antenna correlation - DMRS port allocation between target and co-scheduled UEs: - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE

Wideband CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

We believe CQI reporting requirements with inter-cell interference is critical to verify UE performance under interference scenario. Moreover, RAN4 has already defined CQI requirements with interference for all 2/4/8Rx UEs. Therefore, no matter how RAN4 will design the CQI reporting tests for basic 6Rx, we do see the necessity to define CQI reporting requirements with inter-cell interference.

Proposal 9: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference.

Conclusion

Proposal 1: The new requirements for 6Rx MMSE-IRC receiver should be mandatory for all 6Rx UEs without additional test applicability rule.

Observation 1: It is undecided whether to define one set or two sets of requirements for handheld and FWA UEs in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Proposal 2: FFS how to cover both handheld and FWA UEs until there is clear agreements in Rel-19.

Proposal 3: RAN4 should further discuss the release independent issue after there is conclusion in Rel-19.

Proposal 4: Cover both HomNet and Hetnet scenarios with the following applicability rule: If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Proposal 5: Reuse the same interference power levels (i.e., INR values) defined for Rel-17 2/4Rx and Rel-19 8Rx for 6Rx requirements with inter cell interference.

Proposal 6: Reuse the same PDSCH configurations for 8Rx requirements

- PDSCH configuration for the serving cell - TDLA-30-10, MCS 17, 2x6, rank2 for HomNet - TDLA-30-10, MCS 13, 4x6, rank4 for HetNet

Proposal 7: For the antenna correlation, follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’.

Proposal 8: Use the existing test parameters for 8Rx as a start point as shown below:

- PDCSH requirements with intra cell inter user interference - 1 Co-scheduled UE number - Rank configuration: Rank 2+2 - Precoding method: - Target UE: Random precoding - Co-scheduled UE: Orthogonal precoding - Test and PDSCH configuration for the target UE - TDLA30-10, MCS 17, 4x6, FFS on the antenna correlation - DMRS port allocation between target and co-scheduled UEs: - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE

Proposal 9: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference.

Reference

RP-252957, WID: NR demodulation performance: Phase 6, Apple, RAN#109, Sep 2025