3GPP TSG RAN4 Meeting #116bis																R4-2513144

Prague, Czech Republic, 13th – 17th Oct, 2025

Title: 	6GR System parameters

Agenda Item:	8.3

Source: 	Nokia

Document for:	Approval

# 1				Introduction

Under 6G system parameters topic following items are to be discussed during the Q4 2025 [1].

Waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types.

In this paper we give our initial view for each of the topics.

# 2	Discussion

## 2.1	Waveform

August RAN1#122 made the following agreement on waveform:

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

While additions to the above RAN1 agreement on 6G waveforms maybe done in the future, CP-OFDM for DL and UL as well as DFT-s-OFDM for UL should be used as RAN4 assumption for system parameters.

Support CP-OFDM waveform for downlink

Support CP-OFDM and DFT-s-OFDM for uplink

For further waveform related work in RAN1, RAN4 should discuss and agree on the PA modelling approach that RAN1 should use, and provide the information to RAN1 with a liaison statement:

Ask RAN1 to evaluate waveforms with a PA modelling approach and provide the related PA model to RAN1

## 2.2	Modulation

August RAN1#122 made the following agreement on modulation:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

While additions to the above RAN1 agreement on 6G modulations maybe done in the future, QPSK-to-1024QAM for downlink, and QPSK-to-256QAM with CP-OFDM and pi/2 BPSK-to-256QAM with DFT-s-OFDM for uplink should be used as RAN4 assumption for system parameters.

Support QPSK-to-1024QAM modulation for downlink

Support QPSK-to-256QAM with CP-OFDM and pi/2 BPSK-to-256QAM with DFT-s-OFDM for uplink

## 2.3	Numerology

The numerology for 6G should in principle follow some of the NR structure since interworking between RATs are expected. To define numerology, it is important to keep on mind that Subcarrier spacing, Channel bandwidth (CBW) and the required FFT size are connected to each other.

August RAN1#122 made the following agreement on subcarrier spacings, confirmed by RAN#109:

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Table 2.3-1 below illustrates the SCS options according to the related agreement made in RAN1 #122 above, with a proposal for the maximum CBW (Channel Bandwidth) and the required FFT size (power of two) for different frequency ranges. In the following we elaborate on the justifications for each choice of combination of SCS and maximum CBW sizes. A goal is to define SCS (for PxSCH and PxCCH) for each 6GR scenario as a cell and band-specific parameter.

Table 2.3-1: Subcarrier spacing and Maximum CBW Scenarios for 6G

| FR | Range | SCS (kHz) | Max CBW (MHz) | Required FFT size (assuming power of 2)** |
| --- | --- | --- | --- | --- |
| FR1 | Sub 6GHz (410 MHz – 6.425 GHz) | 15 (FDD) | 100 | 8192 |
|  |  | 30 (TDD) | 200 | 8192 |
|  | Around 7GHz (6.425 – 8.4 GHz) | 30* | 400 | 16384 |
| New FR(s) |  |  |  |  |
|  |  | 60* |  | 8192 |
|  | Around 15GHz† (8.4 - 24.25 GHz) | 30* | 400 | 16384 |
|  |  | 60* |  | 8192 |
|  |  | 120* |  | 4096 |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 800 | 8192 |

- * Select one SCS per sub-FR

- ** This is an implementation issue

† This range can be divided into several sub-Ranges, each with its own SCS.

### 2.3.1. FFT size

The FFT size itself as such is an implementation issue. However, choosing FFT sizes as powers of two enables faster, more energy-efficient, and easier-to-optimize implementations in both software and hardware. Furthermore, it provides a good FFT utilization for scenanrios with 2x ∙100 MHz. Finally it should be noted that even though the FFT size as such is an implementation issue, the 6GR scenario with the maximum number of RBs/cell (and the related maximum number of REs/cell) can be seen as a parameter indicative of the maximum FFT size for 6GR. For example, from RAN1 & RAN2 point of view, 275 RBs/cell is the maximum in NR. This corresponds to 3300 REs and can be made with 4k FFT. As larger maximum channel bandwidths for a given SCS is to be supported for 6GR, there is a need to increase the required FFT size for 6GR compared to the 4k FFT of NR, as can be seen from Table 2.3-1 above. Overall, we suggest adopting 8k FFT as baseline operation. But it should be noted that 16k FFT would be required in case 400MHz channel bandwidth should be supported also with 30kHz SCS.

8k FFT is a baseline for 6GR.

16k FFT needed to support 400 MHz CBW with 30 kHz for the appropriate bands.

### 2.3.2. SCS

As had been visible from the discussions during RAN1#122, clearly the discussions about SCS are specific for different ranges of carrier frequencies.

For Sub 6GHz (410 MHz - 6.425 GHz), we propose this range to remain aligned with current NR deployments using 15kHz SCS for FDD and 30kHz SCS for TDD aligned with the RAN1#122 agreement. This is reasonable given that MRSS requires that SCS is aligned with NR. We don’t see a need for any additional SCS to be supported for sub 6GHz.

For Around 7GHz (6.425 – 8.4 GHz), the RAN1#122 decision still leaves it open if 30 or 60kHz SCS should be applied. Both 30 and 60 kHz SCSs have their benefits for the 7 GHz range (TDD-only) scenario:

- 30 kHz: Better coverage, longer CP length, better co-existence with TDD on Sub 6GHz

- 60 kHz: Improved phase noise robustness, support for higher speeds with given reference signal overhead, lower latency​

At present, we prioritize supporting 30 kHz SCS for this range. Having the same SCS for both Sub- 6GHz (TDD) and Around 7GHz range is preferred not only from coexistence point of view, but also from complexity reasons. For instance, when considering inter-band CA between the 7 GHz range and Sub 6GHz, aligning both to the same SCS (30 kHz) simplifies many functionalities, e.g. HARQ and scheduling operations as well as power control. Furthermore, as shown in [3], 30 kHz SCS provides a robust performance for wide range of different parameter settings such as MCS, delay spread and speed of UE. We therefore show the 30 kHz SCS for around 7GHz in bold in Table 2.3-1 to indicate our current preference.

For Around 15 GHz (8.4 - 24.25 GHz), [30 60 120] kHz SCSs can be studied based on the RAN1 agreement cited above. We see that further discussion and evaluations are needed on the SCS targets, for around 15GHz range. For example division of the Range into several sub-Ranges is needed, before determining the applicable SCS for each sub-Range, where e.g. 30 kHz might be a good choice for the lower portion of this range. Similarly, applying 120 kHz SCS for the upper portion of the Around 15GHz range would allow for synergies with FR2-1 deployments and implementations.

For FR2-1 (24.25 - 52.6 GHz), the RAN1 agreed 120kHz SCS is aligned with current NR deployments in this range.

Consider SCS values shown in Table 2.3-1 as bold for RAN4 evaluations for Sub-6GHz, Around 7GHz and FR2-1.

Further studies in RAN1 for the frequency range around 15 GHz (8.4 - 24.25 GHz) will be needed on the applicable SCS including a potential further split of this frequency range to align the operation with the Around 7GHz band for the lower part and FR 2-1 for the upper part of this frequency range.

## 2.4	Channel bandwidth

These topics are discussed in the following section maximum CBW, minimum CBW and CBW sizes options between minimum and maximum.

### 2.4.1	Maximum CBW

As presented in Table 2.3-1 our proposal for the maximum CBW (Channel Bandwidth) for different frequency ranges is

- Sub 6GHz: compared to NR we propose to increase the maximum FFT size from 4096 to 8192, which allows the support of 100MHz CBW with 15 kHz SCS and 200MHz CBW with 30 kHz SCS.

- Around 7 GHz, around 15 GHz: We see 400 MHz CBW as a relevant scenario not only for Around 15 GHz, but also for Around 7 GHz range. There are several reasons to support 400 MHz as the maximum (instead of 200 MHz):

- Spectrum: total amount of spectrum in Around 7GHz range is almost 2GHz (i.e. 700 MHz in Upper 6 GHz, and 1275 MHz in above 7.125 GHz, respectively)

- Performance: support for a wider CBW is a performance issue. When compared to intra-band CA with the same aggregated bandwidth, wideband carrier is better in all metrics – e.g. UL coverage, throughput, control channel overhead, complexity.

- Forward compatibility: When 400 MHz CBW is needed, it should be defined it in the first 6G release.

- FR2-1: One difference compared to NR is that we propose to increase the maximum FFT size from 4096 to 8192, which brings the maximum CBW size to the desired level of improvement compared to 5G NR.

For Sub 6GHz (410 MHz-6.425 GHz), support up to 200 MHz Channel bandwidth for TDD and up to 100 MHz Channel bandwidth for FDD, respectively.

For Around 7 & 15 GHz (6.425 to 24.25 GHz), support up to 400 MHz Channel bandwidth.

For FR2-1 (24.25 - 52.6 GHz), support up to 800 MHz Channel bandwidth.

### 2.4.2	Minimum CBW

The following was agreed in RAN1#122, agenda item 11.1 and further endorsed in RAN#109.

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

In 5G NR, the initial access channels in DL (SS block and CORESET#0) define the bottleneck scenario for the minimum channel bandwidth (CBW) size. Until Release 17 the minimum CBW size was 5 MHz, and it turned out to be challenging to build support for the 3 MHz CBW in Release 18.

It is reasonable to assume that the initial access design will be a bottleneck for the minimum CBW​ also in 6G. Hence, we need to ensure availability of the second (narrowband) SSB/CORESET0 design for channel bandwidths below 5MHz. The design for second SSB/CORESET0 could be, for example, mapping/rate matching to 12 (or 15) RBs, or puncturing from 24 to (12 or) 15 RBs (i.e. puncturing according to Rel-18). This should be applied for predefined scenarios with specific sync raster points in specific bands and FDD with 15kHz SCS​.

The minimum CBW (Channel Bandwidth) requirements of 5G NR should be reused for 15 and 30 / 60kHz SCS in FR1 and 30 / 60 kHz in the new FR(s) otherwise, i.e. defined by 25 RBs. For 120 kHz SCS for higher bands, we suggest increasing the minimum CBW to 100MHz to allow for a large number of SSB beams by supporting 240kHz SCS SSB.

Table 2.4.2-1 below shows our proposal for minimum CBW for different SCSs.

Table 2.4.2-1: Minimum Channel bandwidth size for 6G

| SCS (kHz) | 15 (FDD) |  | 30 (TDD) | 60 (TDD) (if supported) | 120 (TDD) |
| --- | --- | --- | --- | --- | --- |
| Min. CBW (MHz) | 3* | 5 | 10 | 20 | 100 |

*	Supported for certain bands and sync raster points only

Consider values shown in Table 2.4.2-1 as minimum CBW for different 6GR scenarios

### 2.4.3	CBW size options between maximum and maximum

In NR the options between maximum and maximum channel bandwidth started out with an intentionally limited number of options and a clear objective to have certain step sizes. However, in the duration of NR more and more options were added which have complicated the adaptation of these since they originally were not intended to be introduced. Hence, a different methodololy for options between maximum and maximum channel bandwidth is discussed further in Section 2.9.

## 2.6	#Rx and #Tx

5G was envisioned with two receivers as was the case with LTE. Currently four Rx is mandatory for only a selected bands with frequency of operation higher than 2GHz, n7, n38, n41, n48, n77, n78, n79, and n104 to name a few. However, with the advent of higher frequency bands, foldable form factors and new device technologies, the number of receivers that can fit in the eMBB device category has increased. Thus, the default number of receievrs envisioned for 6GR should be higher than two/four and must be updated. Further, with the upcoming spectrum allocation in upper 6GHz range, it would be hard to maintain desired coverage with a minimum of two/four receivers only. Thus, for these new bands at least eight receivers should be the baseline for eMBB devices. Given the larger footprint and power budget for the FWA devices, we propose to a possibility to have 16 recievers for FWA devices.

Currently 5G standard support TX chains up to 8TX. However, RAN4 has specified requirements only up to 4TX. Work items are made with an assumption that UEs are supporting up to 3TX and FWA type of devices can support up to 4TX. For 6G with introduction of a new band in 7GHz it is envisioned that number of UL MIMO layers may increase.

Consider four receivers as baseline for 6GR, at least for all the bands re-farmed from 5G.

Consider eight receivers for new 6GR bands for eMBB UE. For FWA devices, consider 16 receivers as baseline for new 6GR bands.

Consider UE devices supporting TX chains up to 4

## 2.7.6	Synchronization signal and raster

### 2.7.1	Initial access and synchronization maitenance

As we discussed in our RAN1 contribution [4], 6GR design for initial access should consider drivers from network energy saving perspective and enabling network to support different types of cells while considering UE complexity and different UEs like eMBB and LPWA devices. Periodical always-on signaling like periodical SS/PBCH transmissions in 5G contribute to network energy consumption. 5G was able to enhance network energy efficiency from 4G by decreasing PSS/SSS periodicity four times and replacing CRS transmitted in every subframe with periodical TRS transmitted more sparsely in time. For 6G it is expected that time domain footprint of the always-on signals and channels related to initial access can be further reduced.

6GR to further reduce the time-domain footprint of always-on signals and channel related to initial access compared to 5G NR, aiming at increased energy savings.

It is important to consider from first release what is the minimum channel bandwidth envisioned to be supported in 6GR to avoid needing in future to adapt the initial-access related signals to reduced bandwidths as also mentioned in 2.4.2..

### 2.7.2	Channel raster

For NR bands up to 2.4 GHz refarmed from 4G, 100 kHz channel raster is introduced so that the compatibility with the legacy RATs is maintained. As 100 kHz raster cannot always support UE specific channel bandwidth configured within the SIB1 carrier bandwidth unless their number of RBs are both odd or both even. Thus, the enhanced channel raster 10 kHz is introduced to resolve this even/odd issue in Rel-18. UE is mandated to support the enhanced channel raster from Rel-18.

For NR bands above 2.6 GHz for both FR1 and FR2, SCS based raster is introduced to support flexible wideband operation.

The compatibility with NR channel raster would be required for 6G to support efficient multi-radio spectrum sharing (MRSS), therefore the compability of 6G channel raster with NR channel raster shall be remained for NR refarming bands.

The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them.

### 2.7.3	Sync raster

When UE is powered on or is in out-of-coverage, UE makes a cell search to find a sync signal transmitted from a network which UE can camp on. If no preferred network or previously registerd network is found, UE may go into network scan to exhaustively search for any cell among all the possible frequencies supported by UE. This may cost a lot of search time and consumes a lot of power until UE can get into low power idle mode.

Therefore, the NR sync raster is defined as such that there is a minimum number of entries per band to minimize UE’s initial access efforts. This design concept of NR sync raster is described in TR 38.817-01 clause 4.3.

As RAN1 is still discussing the design of 6G SS/PBCH, it may be early for RAN4 to discuss the 6G sync raster design in detail. However, the following design principles in the NR sync raster can be considred.

- The numer of sync raster entries is as small as possible.

- SSB is not in a fixed position within the SIB1 carrier bandwidth and thus one SSB position can support multiple SIB1 carrier bandwidth configurations.

A range of channel raster that one SSB position can support depends on the minimum channel bandwidth of the band and the bandwidth of SS/PBCH as discussed in TR 38.817-01 clause 4.3.1.1. The range gets wider (i.e., the minimum separation of sync raster points gets larger), if the SS/PBCH bandwidth is smaller and the minimum channel bandwidth is larger. In NR Rel-15, 5 MHz is the mimum channel bandwidth and 20 PRB (3.6 MHz) is the size of SS/PBCH for 15 kHz SCS. This has made 1.2 MHz sync raster cluster (of 3 SSB) separation for 100 kHz channel raster and 1.44 MHz sync raster sepration for SCS based raser for FR1 bands.

When 3 MHz channel bandwidth is introduced later in Rel-18 for some of FR1 bands, a specific SS/PBCH design is introduced and more dense sync raster (i.e., 600 kHz sync cluster separation) than legacy channel bandwidths is introduced. Requiring 3 MHz channel bandwidth to all frequency bands would cause lots of burden in initial access, specific sync raster design may still be needed.

The sync raster design is further discussed according to the progress of SS/PBCH design and the minimum channel bandwidth requirement. For bands requiring ~3 MHz CBW, specific raster design may still be needed for 6GR as in NR.

### 2.7.4	Channel spacing

The nominal channel spacing defined for NR has caused some misunderstanding among those who plan their network deployment due to its complex definition. The benefit of the current definition is that the nominal channel spacing is the same among multiple numerologies so that aggregated carriers are all contiguous or non-contiguous for all the deployed numerologies. The mixed numerology is deployed in none of 5G commercial networks; thus, the nominal channel spacing formula has turned out unnecessarily complex. It is proposed to simplify the nominal channel sapcing without considering simultaneous mixed numerologies.

The nominal channel spacing for 6G shall be simplified without considering simultaneous mixed numerologies.

## 2.8	Spectrum utilization

### 2.8.1	Spectrum utilization

The maximum transmission bandwidth configuration for FR1 in TS 38.101-1 is shown in the following table, including the latest addition of 7 MHz channel bandwidth.

Note: 60 kHz scs may not be specified for 6GR.

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 35 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | N/A | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Spectrum utilization of some NR bandwidths looks slightly more conservative than its next smaller and larger channel bandwidths, such 7 MHz and 45 MHz in 15 kHz SCS, for example. RAN4 needs to study 6GR SU but , NR spectrum utilization can be considered as the starting point for 6G for majority of existing NR channel bandwidths.

NR spectrum utilization can be considered as the starting point for 6G for the majority of existing NR channel bandwidths.

### 2.8.2	Minimum guard band

NR minimum guard bands are calculated using the equation, (CBW x 1000 (kHz) - RB value x SCS x 12) / 2 - SCS/2 as described in TR 38.817-01 clause 4.5.2.1. CBW is the channel bandwidth in MHz, RB value is the maximum transmission bandwidth presented above. SCS is the subcarrier spacing. The minimum guard bands for FR1 are shown in the following table and figure.

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 342.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | N/A | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

One issue we have observed in some NR network deployments is that the minimum guard band of UE specific channel bandwidth is larger than the minimum guard band of SIB1 channel bandwidth (i.e., the system bandwidth configured in BS), even though the UE specific channel bandwidth is smaller than the SIB1 channel bandwidth.

For example, the guard band for 30 MHz is larger than 40 MHz in the above table (where the line in the figure has negative slope). UE transmitting with full 40 MHz SIB1 channel bandwidth needs 552.5 kHz (for 15 kHz SCS) as minimum guard band to meet the emission requirement defined for the 40 MHz channel bandwidth. However, UE configured with UE specific channel bandwidth of 30 MHz aligned at the edge of the 40 MHz SIB1 channel bandwidth require a wider guard band of 592.5 kHz and therefore the emission requirements at the edge of the channel (i.e., SIB1 carrier bandwidth) is not guaranteed in the current standards.

To resolve this issue one network had to tweak the centre frequency of the 40 MHz BS channel (i.e., SIB1 carrier bandwidth) by shifting 40 kHz to secure an enough guard band for the UE specific channel bandwidth placed at the edge. The exceptional BS channel raster was introduced to NR band n28 in the legacy BS specifications.

This characteristic is against UE radio implementations and is just an artifact coming out of the above equation that the minimum guard band is simply derived from the maximum transmission bandwidth configuration. Technically, the guard band to meet emissons requirement such as SEM and spurious emissions should be smaller if the transmission bandwidth is smaller. Such a tweak to the channel raster should never be needed. Therefore, it is proposed to define the minimum guard band such that it increases always monotonically with the channel bandwidth. The minimum guard band for a given SCS and channel bandwidth shall never be larger than the minimum guard band of all the larger channel bandwidths for the same SCS.

The minimum guard band for a given SCS and channel bandwidth shall never be larger than the minimum guard band of all the larger channel bandwidths for the same SCS.

## 2.9	Irregular/Flexible channel bandwidth

### 2.9.1	Background for Flexible BW

In 5G the applicable FR1 channel bandwidths are listed in tables in 38.104 for BS and 38.101-1 for UE (it should be noted RAN4 starts to define requirements also on 6MHz channel bandwidth in Rel-20 NR). In principle all channel bandwidths are mandatory for a UE unless indicated by note 4 that the channel bandwidth is optional. In practice all UEs may not support all channel bandwidths but from 3GPP point of view those UEs are not compliant to specification.

Table 5.3.5-1 Channel bandwidths for each NR band 38.101-1 for UE

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n26 | 15 | 34 | 5 | 10 | 15 | 20 | 25 | 3 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
| n90 | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.9.1-1: UE spec

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1 38.104

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n1 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n26 | 15 | 3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
| n90 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |

Table 2.9.1-2: BS spec

In first 5G release, Rel-15 the list of possible channel bandwidths was not as comprehensive as it is now. For UE, 3, 35, 45, 70 and 90 MHz channel bandwidths were not specified at all. For BS, 70 and 90 MHz bandwidths were part of the initial release but 3, 35 and 45 bandwidths were not. However, 70 MHz and 90 MHz were added by CAT-F correction CRs already during later phase of Rel-15. Addition of 90 MHz bandwidth required also some RAN2 changes to signalling in Rel-15.

Then 3, 35 and 45 MHz had dedicated WIs to add the support. Adding 3 MHz in Rel-18 was very complicated as it required even RAN1 changes due to PBCH and PDCCH redesign (=puncturing support) and RAN4 needed to define the new sync raster entries. 3 MHz is still optional even in Rel-19 for some frequency bands.

In Rel-19 RAN4 has a basket WI called “Adding channel bandwidth(s) support to existing NR bands and CA/ENDC combinations in REL-19” to add missing channel bandwidths that have become relevant to some operators to an existing band. These channel bandwidths are listed in WID for FR1: {3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz}. Note that 3 MHz is only possible for bands below 1 GHz. Furthermore, in Rel-19 there is also a dedicated WI to add support for 7 MHz for both UE and BS “NR_FR1_7MHz_BW”.

In addition to above work which is visible in UE and BS specifications there has been a lot of effort done under a study item on how to handle irregular channel bandwidths which are not multiple of 5 MHz and are smaller than 20 MHz. These are typically spectrum chunks that have become available when previous radio systems such as TV are not using the spectrum anymore. This study is captured in TR 38.844 [2].

From this TR 38.844 we can find a figure 8-1 below. This figure shows one way to achieve irregular channel bandwidth without major specification changes. In this case BS operates with irregular CBW (13 MHz in figure 8-1) and UEs operate as normal UE using legacy bandwidths (10 MHz in figure 8-1).

Figure 8-1: Irregular channel bandwidth

Overlapping CA: the irregular bandwidth is handled by two overlapping component carriers (CCs) with NR channel bandwidth defined in Rel-17. It is network responsibility to prevent collisions between the different component carriers.

Overlapping UE channel BW from network perspective: network supports the irregular bandwidth (either by a single carrier or by two overlapping carriers) while each UE operates in an existing lower regular NR channel bandwidth

### 2.9.2	Flexible bandwidth for 6G

As we have discussed in previous chapter it is a major effort to add new channel bandwidths to specification after the initial release. This is natural as it requires UE and BS re-designs and updates on multiple specifications in RAN1, RAN2 and RAN4.

Therefore, it would be very beneficial if all channel bandwidths between minimum and maximum channel bandwidth were possible from specification perspective and those would be mandatory for an eMBB UE from day 1 of 6G.. The channel bandwidth granularity could increase with the increasing CBW size. Support for the new CBW sizes with increased granularity should be a mandatory requirement for all 6GR UEs for entire channel bandwidth range for 6G i.e. from 3 MHz to 200 MHz for the new native 6G bands in 6–7 GHz range and optionally supported even for 200–400 MHz channel bandwidths if these are specified for new 6G bands. In case of LPWA UEs the similar flexibility would be applied but the maximum channel bandwidth would be narrower than for eMBB UE.

It is acknowledged that the maximum bandwidths in question are very wide which would mean that if granularity is 1 MHz then there would be 198 possible channel bandwidths and if granularity is 1RB then number of possible channel bandwidths is further increased. This potentially means that testing efforts and design efforts would increase although the way the specifications are written can possibly be do so that these issues are mitigated. Nevertheless, it may not be possible to have full flexibility due to increase on testing and design work on channel bandwidths for full 3 – 200 MHz channel bandwidth range. If flexibility needs to be constrained, then the channel bandwidths could be split into ranges which have different granularity. One possibility is presented below.

Table 2.9.2-1: Different bandwidth granularity ranges

|  | Range1* | Range 2 | Range 3 | Range 4 |
| --- | --- | --- | --- | --- |
| Channel bandwidths | CBW ∈ {3, 5, 6,…, 20} MHz | 20 < CBW ≤ 50 MHz | 50 < CBW ≤ 200 MHz | 200 < CBW ≤ 400 MHz |
| Granularity | 1 MHz | 5 MHz | 10 MHz | 20 MHz |
| Optional vs. mandatory | mandatory | mandatory | mandatory | Mandatory or optional |

*) Aligned with minimum CBW size in Table 3.2.2-1.

Range 1 in Table 2.9.2-1 i.e. 3,5,6-20 MHz are the original LTE channel bandwidths and represent the irregular channel bandwidth cases in 5G where smallest granularity is needed. These bands are typically if not entirely, all sub 1 GHz FDD bands and have been re-farmed from other radio systems such as 2G, TV or GSM-R services. Having reasonably high granulariy for the narrowband scenarios would enable optimal utilization of the most valuable frequency assets without the added complexity and reduced performance introduced by the overlapping CA approach.

Range 2 in Table 2.9.2-1 i.e. 20-50 MHz channel bandwidths are marked with 5 MHz granularity. These bandwidths are typically normal MNO spectrum and have been sold or licenced with minimum of 5 MHz chunks. Another more flexible option would have Range 2 to go up to 100 MHz. The difference to 5G even with more flexible option 2 is not as big as with range 1 if one compares with tables 2.9.1-1 and 2.9.1-2. 5G specs are however lacking 55, 65, 75, 85 and 95 MHz channel bandwidths compared to 6G proposal in table 2.9.2-1. Maybe bigger difference to 5G would be that all channel bandwidths in table 2.9.2-1 should be mandatory from day 1. One can say that there can be irregular bandwidth options also above 20 MHz CBW. However, these are relatively rare cases (c.f. <20 MHz CBW). And on the other hand, as the CBW size increases the relative size of uncovered portion reduces (e.g. 30/33 MHz =91% vs. 18/15 MHz= 83%)

Range 3 in Table 2.9.2-1 would go beyond current 5G specs having channel bandwidth of more than 100 MHz and up to 200 MHz. Possible granularity could be 10 MHz. This is important range as the intent is to specify atleast 200 MHz as minimum maximum channel bandwidth to the UE at least for native 6G bands around 6-7 GHz which makes 6G UE more capable compared to 5G UE.

Range 4 in Table 2.9.2-1 reflects the discussions on maximum possible FR1 channel bandwidth for 6G. In this context 400 MHz is often quoted as maximum.

RAN4 to discuss how to make channel bandwidth definition more flexible and future proof in 6GR compared to NR

## 2.10	Device types

The 6GR SID outlines several distinct objectives related to device types. For instance: Target scalable and forward compatible design for diverse device types” [RAN1, RAN2, RAN3, RAN4], “Study UE RF capabilities considering different device types and implementations” [RAN4]. The set of complementary objectives related to different categories of UEs is collectively aiming to achieve an integrated solution in 6GR that supports a framework for device types that is extensible and efficient, and one that overcomes issues faced in previous generations.

5G didn’t introduce UE categories in the first release, targeting eMBB (with hight data rates) as a mainstream use case. The absence of specific categories initially (in Rel-15) made it challenging to manage smooth introduction of a new UEs with distinct capabilities, for which high data rate wasn’t key characteristic. Namely, RedCap devices with limited capabilities meant for specific use cases were introduced into the market and standard didn’t offer handy procedures to exclusively integrate these devices for differentiated handling other than defining discting set of different capabilities. Lack of solid reference and baseline led to proceeding in standard with devices requirements and capabilities scattered across different domains (RAN1, RAN4, RAN2), with challenging identification of a UEs, making it not strighforward to recognize actual support of a baseline and/or complete set of device capabilities.

In the interest of progressing the related objectives, RAN1#122 agreed to:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

In addition, RAN#109 has endorsed the two RAN1#122 agreements on RF and Baseband bandwidth (as referred in section 3.2), guiding further that RAN WGs should continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Therefore, the required RAN1 and RAN4 coordination on several aspects is already emerging due to generic purpose to design 6GR, such as: waveform, modulation initial access, minimum spectrum allocation, minimum 6G channel bandwidth, etc.

For a scalable design for diverse device types, the coordination needs to specifically consider how to characterize 6G devices to determine whether they differ significantly enough to justify defining few distinct device types. For instance, in 5G, eMBB devices were originally (Rel-15) required to support by default 4Rx in most bands, while RedCap devices were originally (Rel-17) only required to support 2Rx or 1Rx (Rel-18). Such characteristics impact both the initial access and RF/PHY characteristics, coverage planning for deployments and UE configuration/capabilities. However, both: 5G eMBB and 5G RedCap devices support default initial NR access scheme, and some versions of 5G eMBB and 5G RedCap devices can even support the same number of RF chains (Rel-15 eMBB device with fallback to 2Rx for power savings and Rel-17 RedCap device) or power class (PC3). On the other hand, highly performing devices (CPEs) became a unique use case with distinct RF and power characteristics, that stand out from all the other devices.

To ensure 6G enables proper and scalable device classification, there are several criteria to consider, which span across RAN1, RAN2 and RAN4, as shown in the following table:

| Criteria |  |  | Device example | Device type |
| --- | --- | --- | --- | --- |
| HW and RF Properties (RAN1 and RAN4) | Grouping ranges of similar properties under a single device type | - No. of RF chains (Rx, Tx) - No. of antennas - Half-duplex/Full-duplex - Single-band vs multiple bands - Bandwidth (flexibility or limitations) | LPWA | Type X |
| Peak data rate (RAN1/RAN2/RAN4) |  | - Max data rate - Modulation (flexibility or limitations) - Minimum performance requirements and feature test applicability |  |  |
| Delay/Processing requirements (RAN1/RAN2/RAN4) |  | - Processing delay constrains vs. relaxation |  |  |
| Energy/Power savings related requirements (RAN1/RAN2/RAN4) |  | - Energy savings support (e.g. WUS) - Power class |  |  |
| Other factors (RAN1/RAN2/RAN4) |  | - Traffic or deployment type (e.g. LPWA, IoT) |  |  |

Scalable design for device types in 6GR requires joint efforts among RAN1, RAN4 and RAN2.

There can be different factors considered for classifying devices, which include fundamental PHY and RF characteristics e.g.: CBW, RF chains, number of antennas, minimum MIMO layers, power consumption, UE processing time, baseband requirements, RF/RRM/DMD requirements, characteristics such as latency, data rate, and energy efficiency.

RAN4 has a key role in defining RF, RRM/DMD minimum performance requirements, Tx/Rx antennas, half-duplex vs, full-duplex, power class and processing requirements Tx/Rx antennas.

RAN4 to study the necessary RF characteristics and attributes for different devices i.e. minimum spectrum allocation, smallest maximum supported RF and Baseband bandwidth, number of RF chains, antennas, HD vs. FD applicability, and power class.

As noted in RAN#109 (in [RP-282873, ”Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)”, Apple, September 2025], the 6G need for at last 6G eMBB and 6G IoT device types “seem to be converging”, “although there are different views regarding whether there should be one or multiple eMBB (or 6G IoT) device types”. Thus, to determine whether assumed 6G devices differ significantly enough to justify defining few distinct device types, we propose to address in the beginning the few examples of devices (eMBB, FWA and mIoT) and identify their evolution to 6G:

RAN4 to study the necessary RF/RRM characteristics to better understand the needs for 6G smartphones (eMBB), Fixed Wireless Access (FWA) devices and minimum capability devices like massive IoT devices.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: Support CP-OFDM waveform for downlink

Proposal 2: Support CP-OFDM and DFT-s-OFDM for uplink

Proposal 3: Provide RAN1 with a PA modelling approach to be used in further waveform related studies in RAN1

Proposal 4: Support QPSK-to-1024QAM modulation for downlink

Proposal 5: Support QPSK-to-256QAM with CP-OFDM and pi/2 BPSK-to-256QAM with DFT-s-OFDM for uplink

Proposal 6: 8k FFT is a baseline for 6GR.

Observation 1: 16k FFT needed to support 400 MHz CBW with 30 kHz for the appropriate bands.

Proposal 7: Consider SCS values shown in Table 2.3-1 as bold for RAN4 evaluations for Sub-6GHz, Around 7GHz and FR2-1.

Observation 2: Further studies in RAN1 for the frequency range around 15 GHz (8.4 - 24.25 GHz) will be needed on the applicable SCS including a potential further split of this frequency range to align the operation with the Around 7GHz band for the lower part and FR 2-1 for the upper part of this frequency range.

Proposal 8: For Sub 6GHz (410 MHz-6.425 GHz), support up to 200 MHz Channel bandwidth for TDD and up to 100 MHz Channel bandwidth for FDD, respectively.

Proposal 9: For Around 7 & 15 GHz (6.425 to 24.25 GHz), support up to 400 MHz Channel bandwidth.

Proposal 10: For FR2-1 (24.25 - 52.6 GHz), support up to 800 MHz Channel bandwidth.

Proposal 11: Consider values shown in Table 2.4.2-1 as minimum CBW for different 6GR scenarios

Proposal 12: Consider four receivers as baseline for 6GR, at least for all the bands re-farmed from 5G.

Proposal 13: Consider eight receivers for new 6GR bands for eMBB UE. For FWA devices, consider 16 receivers as baseline for new 6GR bands.

Proposal 14: Consider UE devices supporting TX chains up to 4

Proposal 15: 6GR to further reduce the time-domain footprint of always-on signals and channel related to initial access compared to 5G NR, aiming at increased energy savings.

Proposal 16: The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them.

Proposal 17: The sync raster design is further discussed according to the progress of SS/PBCH design and the minimum channel bandwidth requirement. For bands requiring ~3 MHz CBW, specific raster design may still be needed for 6GR as in NR.

Proposal 18: The nominal channel spacing for 6G shall be simplified without considering simultaneous mixed numerologies.

Proposal 19: NR spectrum utilization can be considered as the starting point for 6G for the majority of existing NR channel bandwidths.

Proposal 20: The minimum guard band for a given SCS and channel bandwidth shall never be larger than the minimum guard band of all the larger channel bandwidths for the same SCS.

Proposal 21: RAN4 to discuss how to make channel bandwidth definition more flexible and future proof in 6GR compared to NR

Observation 3: Scalable design for device types in 6GR requires joint efforts among RAN1, RAN4 and RAN2.

Observation 4: There can be different factors considered for classifying devices, which include fundamental PHY and RF characteristics e.g.: CBW, RF chains, number of antennas, minimum MIMO layers, power consumption, UE processing time, baseband requirements, RF/RRM/DMD requirements, characteristics such as latency, data rate, and energy efficiency.

Proposal 22: RAN4 to study the necessary RF characteristics and attributes for different devices i.e. minimum spectrum allocation, smallest maximum supported RF and Baseband bandwidth, number of RF chains, antennas, HD vs. FD applicability, and power class.

Proposal 23: RAN4 to study the necessary RF/RRM characteristics to better understand the needs for 6G smartphones (eMBB), Fixed Wireless Access (FWA) devices and minimum capability devices like massive IoT devices.

# 4	References

[1] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

[2] TR 38.844, Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.

[3] R1- 2506753, Frame Structure and Numerology in 6G Radio Air Interface, RAN1#122bis, Nokia

[4] R1-2505125, Nokia Views on 6G Radio Air Interface, RAN1#122bis, Nokia