3GPP TSG-RAN WG4 Meeting#116bis		R4-2513148

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	8.4

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G UE RF

Document for:	Discussion

Introduction

In RAN4#116 meeting, a Plan [1] about RAN4 6G guidance for information was provided by RAN4 chair. In this contribution, we want to provide some initial views on 6G UE RF.

6G topics; Scope; Scope in Q4’2025 and Q1’2026. System parameters; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. General RF and UE RF; non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues; non-spectrum and non-AI UE RF, spectrum aggregation，UE RF related coverage and energy efficiency. BS RF; BS RF, coexistence study, BS RF energy efficiency, duplexing, MSR; non-spectrum and non-AI BS RF, MSR, coexistence study. Spectrum; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey. RRM; RAN4 driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM related coverage and energy efficiency, other PHY signal/channel/procedure related RRM; RAN4 driven non-AI RRM topics,. Demod; RAN4 driven non-AI demod topics, MIMO operation, channel coding, multi TRP, demod related coverage and energy efficiency, other PHY signal/ channel/procedure related demod; RAN4 driven non-AI demod topics. AI; RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI; RAN4 driven use cases. 5G and 6G spectrum sharing; multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G; Topics, if identified, which are not RAN1/2 dependent. Sensing; PHY functions and procedure, RF, coexistence and testability; Topics, if identified, which are not RAN1/2 dependent. OTA and testability; Testability methodology framework and key assumptions, including both AI and non-AI features; Testability methodology framework and key assumptions, including both AI and non-AI features. RAN4 operation efficiency; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects.

Discussion

Tx requirements

- Power class

For NR FR1, power class definition which is applied in single carrier, UL MIMO, CA, ENDC and Tx diversity. The step of power class is 3dB whatever the scenario. With the increased power class, especially above for PC1.5, it is more difficult for UE implementation to achieve a 3dB step. With the increase of power, the efficiency of PA will gradually decrease. The UE may waste more than 2x power consumption with the power class increase 3dB. In our understanding, maybe we can set 2dB step or other values are more appropriate, especially for higher power classes, whether to keep 3dB step for power class needs to be further studied.

Proposal 1: Whether to keep 3dB step for power class or alter other value (e.g., 2dB) for higher power class can be further studied in 6GR.

In NR, for CA/DC, the total power class should be defined. RAN2 defines the capability of higherPowerLimit-r17 to achieve the full power for CA/DC, which can make every carrier/band realize the real max output power. Meantime, the capability can be reported by UE-powerclassPerBandPerBC-R17. Therefore, in our understanding, there is no need to define the total power class for CA/DC in RAN4.

Proposal 2: Remove total power class for CA can be considered in 6G.

- MPR requirements

The motivation of MPR is to achieve better performance, which is related to Tx EVM, ACLR and SEM. At the same time, on top of meeting better performance requirements, better coverage to achieve with MPR reduction is expected. On one hand, improving MPR is important. On the other hand, we should take no additional more complex UE implementation as baseline. In NR, BWP concept is used in RAN1, however, in RAN4 spec, MPR is only related to CBW rather than BWP. In our understanding, we can apply BWP concept to RAN4. The R19 MPR reduction conclusions can be a starting point to apply BWP to reduce MPR.

Proposal 3: MPR reduction can be studied in 6GR, we can take no additional more complex UE implementation as baseline. BWP concept can be applied to RAN4, R19 MPR reduction conclusion can be a starting point.

Rx requirements

In RAN4 spec, delta RIB,c and MSD for band combinations are huge requirements and huge workload for spec. In Rel-19, the WI about HPUE for CA/DC to simplify MSD requirements is concluded. This WI approach is to simplify the MSD requirements of PC2 1TX, PC2 2TX and PC1.5 2TX based on PC3 MSD to add delta MSD. Then the MSD requirements are queried by Look up Table. Therefore, we can reuse this approach to simplify MSD requirements in 6G.

In addition, a concept of band group is popular in 6G. Based on the concept, the number of band combinations will obviously decrease, the CA requirements can be simplified, such as MSD, and the test burden can be reduced by RAN5. The concept of band group is that bands with different frequency bands are combined, switched within groups and aggregated between groups. Such as sub-1GHz can be group1, n1, n3, n39, etc. can be group2, n7, n41, etc. can be group3. Within one band group, we can choose one band by switching. In this one band group, requirements of these bands can be same. Inter band groups can be band combinations. In our understanding, the concept of band group can be studied in 6G.

Proposal 4: To simplify Rx requirements, such as MSD, these aspects can be studied in 6G.

- Reuse the approach in NR REL-19 of simplifying MSD requirements.

- The concept of band group can be studied.

Spectrum aggregation

From spectrum perspective, FR3 (7.125-24.25GHz) is identified as 6G new band. In addition, existing frequency bands including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz) should also be supported. Multiple diverse spectrum resources are shown in Figure 1. In order to maximize the utilization of the 6G new bands and existing frequency bands, an efficient and simple spectrum utilization and operations is critical for 6GR day 1.

Figure1: candidate spectrum resources for 6GR

According to the RAN#109 meeting, there are two proposals agreed, these two proposals are shown as follow.

Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

For proposal 1, how to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. It is observed that there are so many fragmented spectrum which have smaller bandwidth especially in low FR1 bands, to meet the high data rate requirement of 6G, multiple non-continuous carriers with small bandwidth can be aggregated to achieve one wider bandwidth.

If these multiple carriers are all managed as independent cells, the spectrum utilization efficiency will be low due to multiple cell management overhead and common signalling overhead. Thus, aggregating multiple fragmented carriers as a single cell can be studied to save common channels/signals overhead, simplify cell management and achieve fast carrier switching/activation. At least multiple intra-band carriers can be considered using this MCSC (Multi-Carrier Single Cell) mechanism. For MCSC, from RAN4 perspective, shared RF and/or shared BB for fragmented spectrums can be considered to be UE architecture assumption. We can leverage Rel-19 RAN4-led Fragmented carrier SI as starting point. The SI can be extended to inter-band scenario (e.g., span ≤100MHz or 200MHz). In addition, Rel-19 and Rel-20 RAN4-led LB-LB CA switching can be leveraged. LB-LB CA switching can be extended to dual FDD DL CA and support dynamic switching.

Proposal 5: Regarding spectrum aggregation, Rel-19 Fragmented carrier SI can be extended to inter-band scenario and Rel-19/20 LB-LB CA switching can be extended to dual FDD DL CA and dynamic switching to be starting point to be studied in 6GR.

Conclusions

In this paper, we provide our initial views on 6G UE RF, we have made the following proposals.

Proposal 1: Whether to keep 3dB step for power class or alter other value (e.g., 2dB) for higher power class can be further studied in 6GR.

Proposal 2: Remove total power class for CA can be considered in 6G.

Proposal 3: MPR reduction can be studied in 6GR, we can take no additional more complex UE implementation as baseline. BWP concept can be applied to RAN4, R19 MPR reduction conclusion can be a starting point.

Proposal 4: To simplify Rx requirements, such as MSD, these aspects can be studied in 6G.

- Reuse the approach in NR REL-19 of simplifying MSD requirements.

- The concept of band group can be studied.

Proposal 5: Regarding spectrum aggregation, Rel-19 Fragmented carrier SI can be extended to inter-band scenario and Rel-19/20 LB-LB CA switching can be extended to dual FDD DL CA and dynamic switching to be starting point to be studied in 6GR.

- Reference

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair. TSG-RAN4#116 meeting