3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513152

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	4.2.7.3

Document for:	Approval

- Introduction

In RAN4#116 meeting, the big CR [1] for TS 38.108 on SAN requirements of the NR NTN Ku band was agreed. The CP-OFDM signal was defined as ACS interfering signal for FR1-NTN Ku-band, which is different from DFT-s-OFDM as ACS interfering signal for FR1-TN. So the ACS interferer offset for DFT-s-OFDM type of interfering signal in TS 38.104 cannot be reused for FR1-NTN Ku-band ACS requirement.

In this contribution, we provide our views on ACS interferer offset for CP-OFDM type of interfering signal for FR1-NTN Ku-band.

- Discussion

ACS interferer offset for CP-OFDM interfering signal

For TN ACS requirement in TS 38.104, the type of interfering signal for ACS requirement is DFT-s-OFDM signal, but for NTN ACS requirement for FR1-NTN Ku-band, the type of interfering signal for ACS requirement is agreed as CP-OFDM signal as in Table 10.5.1.2a-2 in big CR[1] copied as below:

Table 10.5.1.2a-2: OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | ±2.5075 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 35 | ±9.4625 |  |
| 50 | ±9.4625 |  |
| 70 | ±9.4675 |  |
| 100 | ±9.4675 |  |

Because for some CBW and SCS, the RBs for CP-OFDM is different from that for DFT-s-OFDM, e.g. the RBs for CP-OFDM for 20MHz CBW and 15kHz SCS is 106, while the RBs for DFT-s-OFDM for  20MHz CBW and 15kHz SCS is 100.

Proposal 1: For CP-OFDM type of interfering signal for ACS requirement for FR1-NTN Ku-band, the RBs for 20MHz CBW and 15kHz SCS should adopt 106 instead of 100.

For 20MHz 15kHz SCS 106 RBs CP-OFDM NR signal as interfering signal for ACS requirement for FR1-NTN Ku-band, the OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz in Table 10.5.1.2a-2 in big CR is not applicable.

Observation 1: For 20MHz 15kHz SCS 106 RBs CP-OFDM NR signal as interfering signal for ACS requirement for FR1-NTN Ku-band, the OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz in Table 10.5.1.2a-2 in big CR is not applicable.

For DFT-s-OFDM type of interfering signal for ACS requirement, referring to section 7.4.1 of TR 38.817-2[2], the interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz)  can be calculated by the following equation(1).

(1)

Where,

is the channel bandwidth of the interfering signal in MHz.

is required frequency offset increment in MHz.

(2)

is subcarrier spacing of the interfering signal in Hz.

(3)

is nominal frequency offset in MHz, which is equal to half of the sum of the channel bandwidth of the wanted signal and the channel bandwidth of the interfering signal, shown as following equation.

(4)

is channel bandwidth of the wanted signal in MHz.

is required frequency offset reduction, because the number of RBs of DFT-s-OFDM is reduced relative to  that of OFDM, shown as following equation.

(5)

is RB number of OFDM NR signal  for subcarrier spacing of the interfering signal.

is RB number of DFT-s-OFDM NR signal  for subcarrier spacing of the interfering signal.

But for CP-OFDM type of interfering signal for ACS requirement, frequency offset reduction due to DFT-s-OFDM is needed and is zero.

Using above equation (1), the ACS interferer frequency offset for DFT-s-OFDM and CP-OFDM type of interfering signal for ACS requirement are shown in table 2-1.

| Frequency range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Type of interfering signal | BWWanted(MHz) | SCS(Hz) | BWInterfer（MHz) | FInterfer(MHz) | BWInterferer/2+FInterfer | NRB,OFDM | NRB,DFT-s-OFDM | FDFT-s-OFDM | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-NTN Ku-band | 25 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs | 25 | 15000 | 20 | 0.0075 | 10.0075 | 106 | 100 | 0.54 | 9.4675 |
|  | 35 |  | 35 | 15000 | 20 | 0.0025 | 10.0025 | 106 | 100 | 0.54 | 9.4625 |
|  | 50 |  | 50 | 15000 | 20 | 0.0025 | 10.0025 | 106 | 100 | 0.54 | 9.4625 |
|  | 70 |  | 70 | 15000 | 20 | 0.0075 | 10.0075 | 106 | 100 | 0.54 | 9.4675 |
|  | 100 |  | 100 | 15000 | 20 | 0.0075 | 10.0075 | 106 | 100 | 0.54 | 9.4675 |
| FR1-NTN Ku-band | 25 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 106 RBs | 25 | 15000 | 20 | 0.0075 | 10.0075 | 106 | N/A | 0 | 10.0075 |
|  | 35 |  | 35 | 15000 | 20 | 0.0025 | 10.0025 | 106 | N/A | 0 | 10.0025 |
|  | 50 |  | 50 | 15000 | 20 | 0.0025 | 10.0025 | 106 | N/A | 0 | 10.0025 |
|  | 70 |  | 70 | 15000 | 20 | 0.0075 | 10.0075 | 106 | N/A | 0 | 10.0075 |
|  | 100 |  | 100 | 15000 | 20 | 0.0075 | 10.0075 | 106 | N/A | 0 | 10.0075 |

Based on above analysis, we propose,

Proposal 2: To adopt the following ACS interferer frequency offsets for CP-OFDM type of interfering signal for FR1-NTN Ku-band.

Table 10.5.1.2a-2: OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | ±2.5075 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±10.0075 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 106 RBs |
| 35 | ±10.0025 |  |
| 50 | ±10.0025 |  |
| 70 | ±10.0075 |  |
| 100 | ±10.0075 |  |

Based on above proposals, A draft CR[2] for TS 38.108 for correction on ACS interferer frequency offset for FR1-NTN Ku-band is provided for approval.

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: For CP-OFDM type of interfering signal for ACS requirement for FR1-NTN Ku-band, the RBs for 20MHz CBW and 15kHz SCS should adopt 106 instead of 100.

Observation 1: For 20MHz 15kHz SCS 106 RBs CP-OFDM NR signal as interfering signal for ACS requirement for FR1-NTN Ku-band, the OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz in Table 10.5.1.2a-2 in big CR is not applicable.

Proposal 2: To adopt the following ACS interferer frequency offsets for CP-OFDM type of interfering signal for FR1-NTN Ku-band.

Table 10.5.1.2a-2: OTA ACS interferer frequency offset for SAN type 1-O operating above 10 GHz

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | ±2.5075 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±10.0075 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 106 RBs |
| 35 | ±10.0025 |  |
| 50 | ±10.0025 |  |
| 70 | ±10.0075 |  |
| 100 | ±10.0075 |  |

Based on above proposals, A draft CR[2] for TS 38.108 for correction on ACS interferer frequency offset for FR1-NTN Ku-band is provided for approval.

- References

- R4-2511234, Big CR to TS 38.108 the SAN requirement of the NR NTN Ku band, CHTTL, RAN4#116

- R4-2513153, Draft CR for 38.108, On SAN requirement of the NR NTN Ku band, CATT, RAN4#116bis