3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513182

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	BS RF requirements for 4.9GHz new band

Agenda item:	5.11.3

Document for:	Approval

- Introduction

In RAN#108 meeting, a new WI was established for 4.9GHz new band [1] and revised in RAN#109 in [2]. The objective is as below. In this contribution, we focus on BS RF requirements on this new band.

A one quarter study phase to identify and document coexistence requirements for the new band: - Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. - Identify adjacent spectrum protection requirements - Identify any potential emission limits more stringent than the normal BS and UE emission masks - Identify other technologies operating in the 4940-4990 MHz band - Identify potential coexistence mechanism(s) - Frequency division sharing - Geographic separation - TDD synchronization - Identify any potential emission limits more stringent than the normal BS and UE emission masks Following the completion of the study phase, the core objectives of this WI are: - Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths: SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. 15; 10; 15; 20; 25; 30; 40; 50. 30; 10; 15; 20; 25; 30; 40; 50. 60; 10; 15; 20; 25; 30; 40; 50. - Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band. - Address potential BS and UE co-existence issues, if any. - Specify band specific BS, repeater and IAB emissions requirements if needed - Specify UE additional emission limits and A-MPR if needed - Specify the BS, repeater and IAB RF requirements. - Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible. - Update the related technical specifications to include support for the new band - CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

- Discussion

2.1 Tx requirements

OBUE

4.9GHz new band should be added to the list of bands for Wide Area cat A and cat B option 1 limits for bands above 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2, as n79. For FCC regional requirements, it is already mentioned in TS 38.104 in a general way, so no need further update for regional requirements from BS side.

-----------------------------------TS 38.104---------------------------------

For BS operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n48, n50, n54, n65, n66, n70, n74, n75, n77, n78, n79, n90, n92, n94, n109, n110, n111, basic limits are specified in table 6.6.4.2.1-2 and 6.6.4.2.1-2a:

Table 6.6.4.2.1-2: Wide Area BS operating band unwanted emission limits for above 3 MHz channel bandwidth (NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

-----------------------------------TS 38.104---------------------------------

For BS operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n48, n50, n65, n66, n70, n75, n77, n78, n79, n90, n92, n94, n109, n111, basic limits are specified in tables 6.6.4.2.2.1-2:

Table 6.6.4.2.2.1-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

-----------------------------------TS 38.104---------------------------------

6.6.4.2.5.1	Limits in FCC Title 47

In addition to the requirements in clauses 6.6.4.2.1, 6.6.4.2.2, 6.6.4.2.3 and 6.6.4.2.4, the BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

-----------------------------------TS 38.104---------------------------------

Spurious emission

4.9GHz new band should be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1 and colocation spurious emissions limits, table 6.6.5.2.4-1.

-----------------------------------TS 38.104---------------------------------

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 or n111. |
| NR Band n111 | 4940 – 4990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n111 or n79. |

-----------------------------------TS 38.104---------------------------------

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| NR Band n111 | 4940 – 4990 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

2.2 Rx requirements

The RX requirements for BS are band agnostic, so there is so no impact on Rx requirement.

- Conclusion

This contribution provides BS RF requirements for 4.9GHz new band and the proposal is as below:

Proposal: It is proposed above changes listed in this contribution.

- References

- RP-251887 New WID on Introduction of NR TDD 4.9GHz Band

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation