3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513188

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on RRM core maintenance for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	4.25.3

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 and Part2 were approved [1] and the WI NR_NTN_Ph3 was completed on time.

However, there are still remaining issues have been solved. This document will further discuss the RRM impact for multiple SMTCs with different periodicities and give our understanding and proposals.

- Discussion

Two different SMTC periodicities (with different offsets) in one frequency layer

In the last meeting, RAN4 discussed the related issues about multiple SMTCs with different periodicities for downlink coverage enhancement, and the following agreements were reached [1]:

## [Online # 3] Issue 0-B: Two different SMTC periodicities (with different offsets) in one frequency layer Agreement (online): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - No requirements if the SMTC periodicity is smaller than the actual SSB periodicity of the cells associated with the SMTC - For Trs in Handover requirement: - Trs is the SMTC corresponding to the physical cell ID of the target cell - Further discuss the proposal “or the longer SMTCconfigured in the measObjectNR having the same SSB frequency and subcarrier spacing.” - For Trs in RRC Connection Release with Redirection requirement: - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. - Kp and Kgap in RRC Connected mode intra/inter-frequency measurement requirement - The SMTC period in the requirement is the one used by the cell being identified Agreement (ad-hoc): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Ttarget_cell_SMTC_period in paging interruption requirements: - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement: - To be confirmed in maintenance phase: - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

When two different SMTC periodicities (with different offsets) in one frequency layer are configured, there were still some issues left, which includes Trs in handover requirement and RRC Connection Release with Redirection requirement, Kmulti_SMTC related calculation, etc.

In last RAN2 meeting, RAN2 sent the LS on additional agreements on SMTC enhancements to RAN4 to further clarify their agreements, and the LS is copied as following [2]:

For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. RAN2 kindly request RAN4 to take above agreement into account when designing RAN4 RRM requirement for SMTC enhancement and provide feedback if needed.

For STMC enhancements, there is two points highlighted for RAN4, the first is the applicable scenario especially for FR2, the second is two different SMTC periodicities is applicable for idle/inactive/connected mode not only connected mode.

Regarding frequency range, from RAN4 perspective, if FR2 is supported, the further scenario limitation should be considered since it is so hard to support FR2 for different satellites due to FR2's characteristics of narrow beam, small coverage area, and high handover frequency, etc., so the limitation to single satellite is needed.

Proposal 1: For STMC enhancements, if FR2 is supported, RAN4 to consider the further scenario limitation is needed since it is so hard to support FR2 for different satellites, and limitation to single satellite is needed.

- Trs in Handover requirement

For Trs in handover requirement, in R15, RAN4 has already introduced two SMTC configurations, and the following description of Trs in handover requirement is copied as below [3]:

Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

For ‘Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell’, it is not clear how to choose smtc1 or smtc2 according to the physical cell ID of the target cell. Considering that PCI list is configured for smtc2 (according to TS 38.331[4]), if the PCI of the target cell is in the PCI list, it is unclear whether Trs will follow smtc1 or smtc2 from delay requirements perspective. However, it is a legacy issue not a special issue for NTN.

Observation 1: In R15, RAN4 has already introduced two SMTC configurations, but it is not clear how to choose smtc1 or smtc2 according to the physical cell ID of the target cell.

SSB-MTC ::= SEQUENCE { periodicityAndOffset CHOICE { sf5 INTEGER (0..4), sf10 INTEGER (0..9), sf20 INTEGER (0..19), sf40 INTEGER (0..39), sf80 INTEGER (0..79), sf160 INTEGER (0..159) }, duration ENUMERATED { sf1, sf2, sf3, sf4, sf5 } } SSB-MTC2 ::= SEQUENCE { pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need M periodicity ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare 1} } SSB-MTC3-r16 ::= SEQUENCE { periodicityAndOffset-r16 CHOICE { sf5-r16 INTEGER (0..4), sf10-r16 INTEGER (0..9), sf20-r16 INTEGER (0..19), sf40-r16 INTEGER (0..39), sf80-r16 INTEGER (0..79), sf160-r16 INTEGER (0..159), sf320-r16 INTEGER (0..319), sf640-r16 INTEGER (0..639), sf1280-r16 INTEGER (0..1279) }, duration-r16 ENUMERATED {sf1, sf2, sf3, sf4, sf5}, pci-List-r16 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need M ssb-ToMeasure-r16 SetupRelease { SSB-ToMeasure } OPTIONAL -- Need M } SSB-MTC4-r17 ::= SEQUENCE { pci-List-r17 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need M offset-r17 INTEGER (0..159) } SSB-MTC5-r19 ::= SEQUENCE { pci-List-r19 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need M periodicity-r19 ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1} OPTIONAL, -- Need M offset-r19 INTEGER (0..159) OPTIONAL -- Need M }

In our view, there is a specific target cell in handover, and the physical cell ID of the target cell is always clear for UE. If the UE has not been provided with an SMTC configuration for the target cell, if companies have the common understanding for the legacy requirements regarding the above issue, then the legacy principle can be reused and Trs is the SMTC corresponding to the physical cell ID of the target cell, otherwise it is better to use maximum SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 2: For Trs in handover requirement, if the UE has not been provided with an SMTC configuration for the target cell,

- If companies have the common understanding for the legacy requirements regarding the issue in the above observation, then the legacy principle can be reused and Trs is the SMTC corresponding to the physical cell ID of the target cell.

- Otherwise, Trs is maximum SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- Trs in RRC Connection Release with Redirection requirement

For RRC Connection Release with Redirection requirement, it is different from handover since the physical cell ID is not clear enough. In TS 38.331, physical cell ID is not provided in RAN2 signalling design, the signalling design for RRCRelease message is copied as follows [4] and PCI is not configured in RRCRelease message:

RedirectedCarrierInfo ::= CHOICE { nr CarrierInfoNR, eutra RedirectedCarrierInfo-EUTRA, ... } CarrierInfoNR ::= SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, smtc SSB-MTC OPTIONAL, -- Need S ... }

Therefore, according the agreement in the last meeting, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 3: For Trs in RRC Connection Release with Redirection requirement, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- The definition of TSMTC/TSMTC,i

For the RRC Re-establishment delay requirements, the definition of TSMTC/TSMTC,i is copied as follows [3]:

- TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. - TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

The definition of TSMTC/TSMTC,i is related to the periodicity of the SMTC occasion. When configuring two different SMTC periodicities (with different offsets) in one frequency layer. For intra-frequency carrier and inter-frequency carrier, the periodicity of the SMTC occasion is not clear enough. Considering that PCI is clear according to TS 38.331[4], so it is the same issue as Trs in handover requirements.

ReestabUE-Identity ::= SEQUENCE { c-RNTI RNTI-Value, physCellId PhysCellId, shortMAC-I ShortMAC-I }

Therefore, if companies have the common understanding for the legacy requirements, then the legacy principle can be reused and TSMTC/TSMTC,i is the SMTC corresponding to the physical cell ID of the target cell. Otherwise, the maximum SMTC can be used in the definition.

Proposal 4: For TSMTC/TSMTC,i in RRC Re-establishment delay requirements,

- If companies have the common understanding for the legacy requirements, then the legacy principle can be reused and TSMTC/TSMTC,i is the SMTC corresponding to the physical cell ID of the target cell.

- Otherwise, the maximum SMTC can be used in the definition.

- Kmulti_SMTC/K_satellite for idle/inactive/connected mode

For configuring two different SMTC periodicities (with different offsets) in one frequency layer, since the existing calculation for Kmulti_SMTC/K_satellite didn’t consider the multi SMTC periodicities scenario, so at least the calculation for Kmulti_SMTC/K_satellite for idle/inactive and connected mode neighbor cell measurements need to be discussed.

Firstly, for the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion, so we believe that the definition of overlapping need to be further clarified. A simple method is to reuse the same method in RRC_IDLE mode for deriving Kmulti_SMTC/K_satellite as following [3]:

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

Observation 2: For the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 5: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions.

Then, the scenario becomes very complicated, for the calculation of Kmulti_SMTC/K_satellite, different SMTC occasions in one SMTC may have different overlap cases, to define the minimum requirements, we support to use the maximum K, that is, for each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Proposal 6: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

- Conclusion

This document discussed the issues about RRM core maintenance and the following observations and proposals are presented:

Proposal 1: For STMC enhancements, if FR2 is supported, RAN4 to consider the further scenario limitation is needed since it is so hard to support FR2 for different satellites, and limitation to single satellite is needed.

Observation 1: In R15, RAN4 has already introduced two SMTC configurations, but it is not clear how to choose smtc1 or smtc2 according to the physical cell ID of the target cell.

Proposal 2: For Trs in handover requirement, if the UE has not been provided with an SMTC configuration for the target cell,

- If companies have the common understanding for the legacy requirements regarding the issue in the above observation, then the legacy principle can be reused and Trs is the SMTC corresponding to the physical cell ID of the target cell.

- Otherwise, Trs is maximum SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 3: For Trs in RRC Connection Release with Redirection requirement, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 4: For TSMTC/TSMTC,i in RRC Re-establishment delay requirements,

- If companies have the common understanding for the legacy requirements, then the legacy principle can be reused and TSMTC/TSMTC,i is the SMTC corresponding to the physical cell ID of the target cell.

- Otherwise, the maximum SMTC can be used in the definition.

Observation 2: For the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion.

Proposal 5: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions.

Proposal 6: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

- Reference

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN2

- 3GPP TS 38.133, NR Requirements for support of radio resource management

- 3GPP TS 38.331, NR Radio Resource Control (RRC) protocol specification