3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513191

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on NTN testing for NGSO

Source: 	CATT

Agenda item:	6.6.3.2

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the WF on RRM requirements for NR_IoT_NTN_req_test_enh  was approved [1]. In this contribution, we will discuss and provide our views on test cases for NSGO channel model.

- Discussion

The new TE-emulated channel model with the following updated aspects were introduced for NGSO demod testing.

- New propgation delay model

- Initial ephemeris information parameters and initial UE location parameters

According to WID, the RRM test cases under the new TE-emulated channel model shall be discussed, and the WID is copied as follows[2]:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Issue 1-1: Applicatility of the new TE-emulated channel model for RRM test cases

In the last meeting, RAN4 discussed the applicatility of the new TE-emulated channel model for RRM test cases, and the WF is copied as below[1]:

WF: FFS on - The new NGSO channel model in Rel19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133.

Some companies supported to use new TE-emulated channel model for other test cases beside the UE timing tests, and we can generally understand the motivation. However, we prefer to follow the scope of the WID to only conclude uplink timing test cases, if the scope is extended to all of RRM requirements, the time budget for this WI  performance is likely insufficient. So, we suggest focusing on uplink timing aspects and the new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133.

Proposal 1: The new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133 according to the WID.

Issue 1-2: Applicatility of the new TE-emulated channel model for pre-R19 UE

In Rel-19, the new TE-emulated channel model is introduced. However, whether the pre-R19 UE shall be tested if the new channel model avaiable need to be considered as well. The WF is copied as follows [1]:

WF: FFS on - Proposal 1: - For the pre-R19 UE, if the new TE-emulated channel model defined in Rel19 available for DUT, the test cases defined before Rel19 can be tested under these new channel model instead of the static channel. - Proposal 1a: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

In our view, the pre-R19 UE should originally pass the pre-R19 test cases with corresponding test configuration. If the pre-R19 UE shall be tested with the new channel model avaiable, which means the pre-R19 UE need to be tested with R19 test cases with R19 test configurations.

Observation 1: If the pre-R19 UE shall be tested with the new channel model avaiable, which means the pre-R19 UE need to be tested with R19 test cases with R19 test configurations.

Issue 1-3: Testing requirements

For testing requirements, the candidate proposals are copied as below [1]:

WF: FFS on: - Proposal 1 (Samsung): - For the timing advance test, FFS on whether addition margin should be added in the test requirements. - Proposal 2 (Samsung, Huawei): Clarify that how to calculated the requirements for NR NTN timing test: - Option 1a (Samsung): NTA,adjUE in the test requirements is calculated by UE itself. - Option 1b (Huawei): NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

For transmit timing test, NTA,adjUE is used to determine the ideal UL timing, we think that it is calculated by the TE in the test, and clarification like option1b for assumption of  TE calculation is ok for us.

For the timing advance test, we also think that RAN4 need to discuss whether addition margin should be added in the test requirements.

Observation 1: If the pre-R19 UE shall be tested with the new channel model avaiable, which means the pre-R19 UE need to be tested with R19 test cases with R19 test configurations.

Proposal 2: For the timing advance test, FFS on whether addition margin should be added in the test requirements.

Proposal 3: NTA,adjUE is calculated by the TE in the test, and clarification like option1b for assumption of  TE calculation is ok for us.

- Conclusion

This document discussed the issues for NTN testing for NGSO and presented following observation and proposals:

Proposal 1: The new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133 according to the WID

Proposal 2: For the timing advance test, FFS on whether addition margin should be added in the test requirements.

Proposal 3: NTA,adjUE is calculated by the TE in the test, and clarification like option1b for assumption of  TE calculation is ok for us.

- Reference

[1]	R4-2512342, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi

[2]    RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung