3GPP RAN WG4 Meeting #116bis	R4-2513219

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on RRM performance requirements for BM and positioning

Source: 	CATT

Agenda item:	6.11.6

Document for:	Discussion

Introduction

RAN4 discussed the performance part of beam management in Rel-19 AI/ML for NR air interface in previous meetings without much progress. In this contribution, we will continue to present our views on relevant issues.

Discussion

Performance requirement for BM

For AI/ML based beam management, the test target depends on the way of how the performance requirements are defined. And in RAN4#114bis meeting, RAN4 have achieved much progress about test metrics/KPIs [1][2]:

RAN4#114bis: Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2 RAN4#116: Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

In this contribution, we will focus on how to define performance requirements based on the above agreed test metrics/KPIs. According to the above agreements, there are two sets of test metrics/KPIs, i.e., beam ID prediction accuracy and RSRP prediction accuracy. The former one only applies for the case where UE reports predicted beam ID only, and the latter one applies for the case where UE reports both predicted beam ID and the predicted RSRP.

Observation 1: Different performance requirements need to be separately defined for beam ID prediction and RSRP prediction.

When UE reports both predicted beam ID and the predicted RSRP, test equipment needs to verify the RSRP accuracy against specified absolute/relative RSRP accuracy requirements. However, a main motivation to introduce AI/ML into beam management is to reduce the measurement effort and air interface overhead. Therefore when RAN4 define the  performance requirements for absolute/relative RSRP accuracy, different levels of overhead reduction need to be taken into account for a given SINR. For example, predicting 64 beams based on measuring 8 beams or 16 beams respectively, leading to 87.5% and 75% overhead reduction, and different performance requirements are defined for both cases. Now assuming that the reduction ratio is 0, which means SetB is the same as SetA, and then prediction is performed. In this case, the performance requirements of AI/ML based BM shall not be worse than the legacy requirements given the same SINR, i.e., no performance degradation is expected.

Next assuming that reduction ratio increases, whether lower RSRP results are allowed is the key point in this issue. If the legacy accuracy values are reused for all overhead reduction ratios, it means only one set of performance requirements will be defined and it is exactly the same as the legacy requirements. Option 2 means that higher RSRP results will be defined, but it is not the main purpose of this feature which is to reduce overhead rather than improving performance. Option 3 is that lower RSRP results are allowed. Considering different overhead reduction ratios, larger margin values can be defined for higher overhead reduction ratios. For instance, ±1 dB margin can be defined for overhead reduction ratio of 50% and ±2 dB margin for 75%.

Proposal 1: For AI/ML based beam management with both predicted RSRP and beam ID reported, two options can be considered for defining performance requirements:

- Option 1: The legacy L1-RSRP performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy L1-RSRP performance requirements are relaxed with margins, and larger margin values are used for higher overhead reduction ratios.

When UE reports predicted beam ID only, test equipment verifies the successful rate for the correct prediction. After the values of K, N and x are determined, the result of each test is just a successful rate, and therefore RAN4 can set a lower bound for this successful rate. One issue is whether to use K, N, x and SINR as side conditions of performance requirement. In our understanding, the requirement is the minimum performance requirement and the successful rate, which is a number from 0 to 1, is not very sensitive to these conditions. Hence, we suggest defining a lower bound for the successful rate as performance requirement regardless of the values of K, N, x and SINR.

Proposal 2: For AI/ML based beam management with predicted beam ID reported only, RAN4 can define a lower bound for the successful rate as performance requirement regardless of the values of K, N, x and SINR.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Observation 1: Different performance requirements need to be separately defined for beam ID prediction and RSRP prediction.

Proposal 1: For AI/ML based beam management with both predicted RSRP and beam ID reported, two options can be considered for defining performance requirements:

- Option 1: The legacy L1-RSRP performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy L1-RSRP performance requirements are relaxed with margins, and larger margin values are used for higher overhead reduction ratios.

Proposal 2: For AI/ML based beam management with predicted beam ID reported only, RAN4 can define a lower bound for the successful rate as performance requirement regardless of the values of K, N, x and SINR.

References

[1] R4-2505105, WF on the requirements for AI/ML air interface normative work, Qualcomm Incorporated, RAN4#114bis.

[2] R4-2511890, WF AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.