3GPP RAN WG4 Meeting #116bis	R4-2513223

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on the RRM core requirements of LBCA enhancements

Source: 	CATT

Agenda item:	7.8.4

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved a new work item on enhancement of low NR band CA via switching to enable dual downlink case [1]. The objectives are reproduced here:

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

RAN4 will start to discuss this WI from RAN4#116bis meeting and in this contribution we will present our views of the impacts of enhanced LB-CA on RRM core requirements.

Discussion

Impacts on RRM requirements

According to the WID, the same deployment constraints as Rel-19 LB-CA WID are assumed, and the requirements defined in RRM and RF and the semi-static switching mechanism should be reused as much as possible. The new things here are more band combinations are considered for support of this dual downlink case, Case 3. The example band combinations are shown below as specified in TS 38.101-1, which are all FDD+SDL configurations:

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL19 |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n67 | N/A | 738 MHz – 758 MHz | SDL19 |

In order to identify the impacts of switching between Case 1 and Case 3, we would like to analyse the difference between Case 2 and Case 3. In Figure 1, the only difference between {case 1, case 2} switching and {case 1, case 3} switching is when UE stays on SDL carrier, UE can still perform DL receptions from FDD DL carrier. Therefore compared with {Case 1, Case 2} switching, {Case 1, Case 3} switching has less impacts on RRM requirements.

Figure 1 - Difference between {case 1, case 2} switching and {case 1, case 3} switching

In Rel-19 LB-CA WI, RAN4 defined the below new RRM requirements because UE cannot perform DL receptions from both PCell and SDL SCell simultaneously:

(1) SCell activation/deactivation procedures;

(2) RLM/BFD/CBD/L1-RSRP/LI-SINR measurements;

(3) L3 intra-frequency measurement with MG;

(4) TCI state switch delay and pathloss reference signal switch delay.

In Rel-20 enhanced LB-CA WI where dual DL reception is enabled, there should be no measurement issues on either PCell or SDL SCell and therefore most of new RRM requirements defined for Rel-19 LB-CA do not apply, such as RLM/BFD/L1-RSRP/L1-SINR measurement requirements. Meanwhile, network knows the time duration during which UE cannot perform UL transmission according to the switching pattern, and therefore there is no UL issues either.

Observation 1: With dual DL reception capability, most of new RRM requirements defined for Rel-19 LB-CA do not apply.

In addition, RAN4 also defined the time duration required for switching pattern activation/deactivation procedures and in our understanding these requirements and be reused.

Proposal 1: RAN4 to reuse the requirements for switching pattern activation/deactivation procedures.

Conclusions

This contribution discussed the impacts of LBCA on RRM performance requirements and some proposals are provided:

Observation 1: With dual DL reception capability, most of new RRM requirements defined for Rel-19 LB-CA do not apply.

Proposal 1: RAN4 to reuse the requirements for switching pattern activation/deactivation procedures.

References

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei), RAN#109.