3GPP TSG-RAN WG4 Meeting #116bis				                                           R4-2513229

Prague, Czech, October 13th – October 17th, 2025

Title:	Discussion on 6G AIML based RAN4 use cases

Source:	CATT

Agenda Item:	8.9

Document for:	Discussion

1 Introduction

In RAN#108 meeting, the study item on 6G radio was approved and it was updated in RAN#109 meeting [1].

According to the SID, the following agendas are given for RAN4#116bis to collect contributions:

8 6G study item 8.1 Feature lead summary and conclusions 8.2 General aspects and others 8.3 System parameters 8.4 General RF and UE RF 8.5 BS RF and coexistence 8.6 Spectrum 8.7 RRM 8.8 Demodulation 8.9 AI 8.10 Spectrum sharing 8.11 Sensing/ISAC 8.12 Testability and OTA 8.13 RAN4 operation efficiency

In this contribution, we will present our views on potential RAN4-led AI/ML use cases in 6G radio.

2 Discussion

As artificial intelligence (AI) and machine learning (ML) have been developing rapidly, they are applied in more and more industries to improve efficiency and reduce operation cost, such as AI-assisted driving in industrial areas and intelligent robots/drones in factories. Hence, it seems necessary to introduce AI/ML functions into the design of 6G wireless communication system. In Rel-18, 3GPP RAN firstly studied AI/ML based use cases and sequentially in Rel-19 spec some physical mechanisms and performance requirements are specified for AI/ML based CSI prediction, beam management and positioning. However, the convergence of AI/ML and wireless communication system started in Rel-18 in NR, which was quite late since the framework of NR system was already stable and was not easy to be optimized for adapting AI/ML-assisted use cases. In 6G radio, AI/ML functions should be considered at the very beginning of the new framework design.

From RAN4 perspective, it is worthy of considering AI/ML in each technical scope as much as possible, i.e., in RRM, RF and Demod scope.

AI/ML based RRM

In legacy RATs including NR, radio resource management (RRM) highly relies on the interworking between base stations and UEs, in which case the amount of measurement and reporting performed by UEs is quite high, consuming much UE power and also leading to high overheads for both measurement configuration and reporting in air interface.

Observation 1: In legacy RATs, DL/UL scheduling highly depends on the UE measurement reports.

Leveraging the AI/ML capabilities, 6G base stations, which have fixed locations, can model the wireless environment in the proximity area to themselves by learning from the daily information they obtain during communications. Meanwhile, the AI/ML model can continuously develop and update itself to adapt to a changing environment and thereby polish the model of wireless environment. Based on this model of wireless environment, 6G base stations can schedule UEs with optimized schemes, instead of highly relying on lots of measurement results reported by UEs. Ideally, one AI/ML model can be used to assist in the scheduling decisions of both downlink and uplink directions.

In addition, it is possible that new RRM requirements/performance metrics can also be defined based on the outputs of this AI/ML model.

Observation 2: Outputs abstracted by AI/ML models from wireless environment help reduce the overheads of measurement configuration and UE measurement reporting in the air interface.

Furthermore, abstraction of the wireless environment, i.e., the outputs of AI/ML model, in different time instances may be different in busy time duration, or they could be quite stable during some idle time from a system perspective, in which case AI/ML models deployed in base stations do not need frequently online/offline retraining or online fine-tuning. Base stations can still implement both downlink and uplink scheduling with stable schemes based on the output results of the AI/ML models with non-degraded or even improved system-level performance. Meanwhile, base stations can stop the online retraining or fine-tuning of AI/ML models to save energy, thereby reducing the power consumptions in the 6G radio. Therefore we suggest studying the potential usages of AI/ML based wireless environment abstraction in the scenario of data scheduling and network power saving in the 6G radio study in RAN4.

Observation 3: Outputs abstracted by AI/ML models from wireless environment could be beneficial to reduce the network power consumption while maintaining or improving the system-level performance.

Proposal 1: RAN4 to study the abstraction of the wireless environment by base station AI/ML models which can be used to facilitate data scheduling and network power saving in 6G radio.

AI based RF

Regarding the harmonization of AI/ML and RF, one potential use case is to select an suitable configuration from all UE supported capability combinations, e.g., band combination and power class, for all UEs served by this base station based on certain prediction outputs of AI/ML models, while achieving an optimized system-level performance. The metrics/KPIs for evaluating the performance of each UE capability combination by AI/ML model can be further studied.

Proposal 2: RAN4 to study the AI/ML models harmonized with RF, which help network to configure UEs with a suitable UE capability combination based on outputs of AI/ML models while achieving an optimized system-level performance.

Another potential use case is AI/ML based PA non-linearity compensation which has been discussed in RAN1 in last meeting. According to the RAN1 summary [2], this use case seems better to be discussed in RAN4 and therefore we would like to elaborate the motivation of this use case [3] in this contribution.

When RF power amplifiers operate in near maximum output (within nonlinear region), both amplitude distortions and phase distortion increase, leading to severe adjacent channel interference and elevated error vector magnitude. These issues force the system to reduce power levels, sacrificing the overall performance. It is difficult for conventional methods to solve this problem with balanced complexity and performance. To overcome this issue, AI/ML-based compensation techniques can be considered to improve the link performance.

Proposal 3: RAN4 to study the AI/ML based PA non-linearity compensation.

AI based Demod

For AI/ML based Demod, we think it can be studied with a lower priority, considering that the current receivers have provided a very good demodulation performance and therefore this is no much space for further performance improvement using AI/ML models. But when advanced receivers emerge in the future, AI/ML based demodulation can be studied to reduce the UE power consumption and maintain or even significantly improve the Demod performance.

Proposal 4: RAN4 to deprioritize the study of AI/ML usage in Demod unless advanced receivers emerge.

3 Conclusion

In this paper, we provided our views on potential RAN4 driven AI/ML use cases in 6G radio and the following observations and proposals are given:

AI/ML based RRM

Observation 1: In legacy RATs, DL/UL scheduling highly depends on the UE measurement reports.

Observation 2: Outputs abstracted by AI/ML models from wireless environment help reduce the overheads of measurement configuration and UE measurement reporting in the air interface.

Observation 3: Outputs abstracted by AI/ML models from wireless environment could be beneficial to reduce the network power consumption while maintaining or improving the system-level performance.

Proposal 1: RAN4 to study the abstraction of the wireless environment by base station AI/ML models which can be used to facilitate data scheduling and network power saving in 6G radio.

AI/ML based RF

Proposal 2: RAN4 to study the AI/ML models harmonized with RF, which help network to configure UEs with a suitable UE capability combination based on outputs of AI/ML models while achieving an optimized system-level performance.

Proposal 3: RAN4 to study the AI/ML based PA non-linearity compensation.

AI based Demod

Proposal 4: RAN4 to deprioritize the study of AI/ML usage in Demod unless advanced receivers emerge.

- References

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

- R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung (Moderator), RAN1#122.

- R1-2505298, Discussion on AI/ML in 6GR interface, CATT, CAICT, RAN1#122.