3GPP TSG-RAN WG4 Meeting #116-bis	                                                                                       R4-2513233

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for IoT NTN support PC 1.5

Agenda item:	7.6.3

Document for:	Approval

- Introduction

In RAN #109 meeting, a new WID [1] for NTN and IoT NTN phase 2 has been approved. The work item will focus on the further enhancement of NTN HPUE and HD-FDD VSAT. This paper is initially shares some views on RF requirements for IoT NTN support PC 1.5.

- Discussion

In RAN #109 meeting, a new WID for IoT NTN support PC 1.5 has been approved, as shown below:

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

The WID is aimed to specify RF requirements for IoT-NTN PC 1.5 HPUE. Considering that the coexistence study of PC 1.5 HPUE has been completed, and the corresponding ACLR requirements has been agreed upon, we believed that the existing agreements could be directly applied, and therefore no additional ACLR evaluation required. Meanwhile, several RF requirements, such as nominal MOP (and tolerance), have been agreed upon in Rel-19. These agreements can also be applied directly.

Proposal 1: For IoT-NTN PC 1.5 HPUE RF requirements, the agreements discussed and reached in Rel-19 can be applied without additional re-evaluation.

For the RF requirements that have not been fully evaluated in Rel-19, such as MPR/A-MPR, it may be necessary to continue the evaluation work to determine specific requirements. However, the evaluation assumption has been thoroughly discussed in Rel-19 and with fruitful conclusions. For instance, the MPR=0 reference point has been discussed and reached agreements in Rel-19 for NB IoT based IoT-NTN HPUE, and such methodology could be applied.

Proposal 2: For IoT-NTN PC 1.5 HPUE RF requirements, the evaluation assumption have been agreed in Rel-19 could be applied without additional discussion.

Considering that the IoT-NTN PC 1.5 HPUE will be introduced by 1Tx architecture, and will operate in band 256/255, it is obvious that such IoT-NTN HPUE will operate in HD-FDD mode. Referring to the RSD study during Rel-19 NTN HPUE work item and previous TN HPUE ( such as TR 38.861 and TR 38.896), for 1 Tx HD-FDD device, RSD does not seem to be necessary, as there is almost no interference between its uplink and downlink.

Proposal 3: For IoT-NTN PC 1.5 HPUE, no RSD expected for band 256 and 255.

Regarding the SAR related issues of IoT-NTN PC 1.5 HPUE, there has been a lot of attention in Rel-19, but the final conclusions are not very clear. However, considering that the HPUE type indicated in the WID is NB-IoT based, and the conclusion of Rel-19 shows that duty cycle or other new schemes have been precluded, it can be expected that the SAR issue can adopt the relevant conclusions of Rel-19.

Proposal 4: For IoT-NTN PC 1.5 HPUE, the SAR related agreements in Rel-19 could be adopted.

- Conclusion

This contribution provides our initial views on RF requirements for IoT-NTN PC 1.5 HPUE. The following proposals are concluded as follows:

Proposal 1: For IoT-NTN PC 1.5 HPUE RF requirements, the agreements discussed and reached in Rel-19 can be applied without additional re-evaluation.

Proposal 2: For IoT-NTN PC 1.5 HPUE RF requirements, the evaluation assumption have been agreed in Rel-19 could be applied without additional discussion.

Proposal 3: For IoT-NTN PC 1.5 HPUE, no RSD expected for band 256 and 255.

Proposal 4: For IoT-NTN PC 1.5 HPUE, the SAR related agreements in Rel-19 could be adopted.

- References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN #109