3GPP TSG-RAN WG4 Meeting #116-bis	                                                                                       R4-2513234

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for Ku band HD-FDD VSAT

Agenda item:	7.6.4.1

Document for:	Approval

- Introduction

In RAN #109 meeting, a new WID [1] for NTN and IoT NTN phase 2 has been approved. The work item will focus on the further enhancement of NTN HPUE and HD-FDD VSAT. This paper is initially shares some views on RF requirements for HD-FDD VSAT.

- Discussion

In RAN #109 meeting, a new WID for HD-FDD VSAT has been approved, as shown below:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note: Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

The WID is aimed to introducing NTN HD-FDD for Ku band in phase 1. Firstly, the applicability of HD-FDD VSAT requirements should be discussed. Considering the demands of stakeholders for HD-FDD VSAT, the introduction of the HD-FDD architecture is actually mainly aimed at reducing the implementation difficulty of the NTN VSAT, which does not imply a reduction in the NTN VSAT capability. Meanwhile, considering that the requirements defined for Ku band in FR1 are all OTA requirements, while currently OTA requirements do not include content of RedCap. Therefore, when defining requirements for HD-FDD VSAT, only the impact caused by the HD-FDD feature should be considered, rather than regarding HD-FDD VSAT as a RedCap device by default.

Proposal 1: For NTN VSAT operating in Ku band, only the HD-FDD related requirements should be specified.

Meanwhile, the new IE and capabilities related to HD-FDD for both SAN and HD-FDD VSAT should be defined. Since the IE and capabilities related to HD-FDD in the current spec are coupled with RedCap, it is necessary to define new IE and capabilities that are decoupled from RedCap to ensure the proper functioning of HD-FDD VSAT. From the perspective of SAN, at least a new HD-FDD allowed IE and capabilities decoupled from RedCap needs to be defined. Otherwise, SAN will not be able to indicate that the HD-FDD VSAT can operate in the cell, causing the HD-FDD VSAT to be barred. From the perspective of HD-FDD VSAT, a similar approach could also be considered. Therefore, the following IE and capabilities should be defined:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [halfDuplexVSAT-Allowed] The presence of this field indicates that the cell supports half-duplex FDD VSAT. | Band | NO | FDD only | NO |
| [halfDuplexFDD-TypeA-VSAT] Indicates whether the VSAT only supports half-duplex operation for FDD in the indicated band(s). | Band | NO | FDD only | NO |

Proposal 2: Send LS to RAN2 for the added UE capabilities and IE to configure the HD-FDD VSAT.

The following table shows the NTN VSAT RF requirements of Ku band in FR1 numerology need to be defined:

Table 2.1 NTN VSAT RF requirements for HD-FDD VSAT operating in Ku band FR1 numerology

| NTN VSAT Tx/Rx requirement | Proposals for HD-FDD VSAT operating in Ku band FR1 |
| --- | --- |
| 9.2.1 NTN VSAT maximum output power | Specification impact. Small antenna size will decrease the gain → TRP and EIRP impact |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | No specification impact. |
| 9.3.3 Transmit ON/OFF time mask | For NTN VSAT in Ku band with FR1 numerology, the transient period is already 10us → No specification impact. |
| 9.5.1 Occupied bandwidth | No specification impact. |
| 9.5.2 Out of Band Emissions | No specification impact. |
| 9.5.3 Spurious Emissions | No specification impact. |
| 9.6.2 Antenna performance | Specification impact. Small antenna size will decrease the gain |
| 9.7 Additional regional requirements indicated by NS | No specification impact. |
| 10.3 OTA reference sensitivity level | Specification impact. Small antenna size will decrease the gain, however for HD-FDD VSAT, the Tx and Rx may use single antenna panel. → reevaluate the EIS |
| 10.4 Maximum input level | Specification impact. To introduce new reference measurement channel for HD-FDD VSAT |
| 10.5 Adjacent channel selectivity | Specification impact. To introduce new OCNG pattern for HD-FDD VSAT |
| 10.6 Blocking characteristics | Specification impact. To introduce new OCNG pattern for HD-FDD VSAT |
| 10.8 Receiver antenna off-axis performance | Specification impact. Small antenna size will decrease the gain |

Proposal 3: To use the proposals in Table 2.1 for NTN VSAT RF requirements for NTN HD-FDD VSAT operating in Ku band.

- Conclusion

This contribution provides our initial views on RF requirements for HD-FDD VSAT. The following proposals are concluded as follows:

Proposal 1: For NTN VSAT operating in Ku band, only the HD-FDD related requirements should be specified.

Proposal 2: Send LS to RAN2 for the added UE capabilities and IE to configure the HD-FDD VSAT.

Proposal 3: To use the proposals in Table 2.1 for NTN VSAT RF requirements for NTN HD-FDD VSAT operating in Ku band.

- References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN #109