3GPP TSG RAN WG4 Meeting #116bis	R4-2513270

Prague, CZ, 13th – 17th, Oct. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Views on 6G UE RF aspect

# Agenda item:	8.4

# Document for:	Approval

# 1 Introduction

The new SID[1] was approved in RAN #108 meeting, and the work for RAN4 will start in RAN4 #116bis meeting.

In this contribution, we give some initial views on the 6GR UE RF parameters.

# 2	Discussion

## 2.1 UL coverage

### 2.1.1 5G NR UL coverage

The 5G NR was designed to meet vastly increased traffic demands, including significantly higher uplink data requirements. Early 5G deployments revealed that while mid-band (e.g. 3.5GHz TDD) spectrum provides large downlink capacity, the uplink can become a bottleneck due to limited device transmit power and fewer antennas on UE.

Uplink signals are typically weaker (especially at cell edges or indoors) because mobile devices must abide by strict power limits (to manage battery life, interference, and safety).

In Rel-15, some uplink boosting features were included such as ENDC, SUL and CA. EN-DC allows a 5G NR UE to transmit on LTE band simultaneously or fallback to LTE uplink in poor NR coverage, leveraging lower-frequency LTE spectrum to extend uplink coverage. Similarly, SUL combination permits pairing a high NR band with a low SUL band, improving cell-edge UL coverage on 5G. These techniques mitigated early 5G uplink limitations but also had drawbacks, e.g. in EN-DC the UE’s second transmitter is tied to LTE, preventing NR from using both antennas for true UL-MIMO on the NR carrier.

In Rel-16, another some additional uplink enhancements were introduced to unlock better 5G UL performance. A key example is Uplink Transmit (Tx) Switching, a dynamic mechanism to coordinate two UE transmit antennas across carriers. This allows, for instance, an NR CA UE to switch its second Tx between low frequency NR and high frequency NR as needed, so that during high frequency NR uplink transmissions both antennas can be used (enabling 2x2 MIMO on NR) and when high frequency NR UL is not needed, one antenna can revert to low frequency NR.

In Rel-17, some further uplink enhancements were introduced, such as intra-band CA+ UL MIMO which combines both advantages of these two technical. It can not only increase the total transmitted power, but also enable higher data rates at the cell edge with robust diversity gains. In addition to that, HPUE UL CA features were introduced and also for inter-band UL CA, high power limits capability (i.e. higherPowerLimit-r17) was introduced to enable UE can transmit higher power considering the PA full power capability.

In Rel-18, there was coverage enhancement WID led by RAN1, which aims to further increase the UL coverage by increasing UE power high limit for CA and DC and reducing MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation. The former one supports more CC in the UL inter-band CA and can sum of the power of each band. The latter one supports FDSS w/o spectrum extension for QPSK and pi/2 BPSK for DFT-s-OFDM to further reduce the PAPR, consequently it can achieve 1dB MPR power boost for PC3 and 0.5dB MPR power boost for PC2, which means the UL maximum output power can increase. In addition to that, 3Tx UL inter-band CA was supported which combines the advantages from HPUE and UL MIMO to further enhance the UL coverage. For single band, high order UL MIMO(i.e. 4Tx) was supported, although it is for FWA form factor, 4Tx UL MIMO can provide superior coverage compared to 2Tx UL MIMO primarily through more advanced beamforming (Spatial Multiplexing) and higher order diversity gains. This results in a stronger, more focused signal that can reach further and is more resilient to signal fading.

In Rel-19, Power boosting and/or MPR reduction mechanism was introduced, where an extended channel bandwidth solution was raised with the aim of reducing MPR in outer and edge RB allocation through relaxing out-of-band emission domain. This mechanism converts outer and edge RB allocation to inner RB allocation and achieves about 1 ~ 2dB MPR reduction to further improve UL coverage.

Observation 1. There were many mechanisms introduced to increase UL coverage in 5G NR.

### 2.1.2 6GR UL coverage consideration

For 6GR, UL coverage is also a vital objective according to the SID, especially for 6G deployments in at least around 7 GHz, the existing 5G mid-band (~3.5GHz) site grid was re-used and targeting comparable coverage to 5G mid-band. However, due to the high propagation loss for the high frequency range, there exists large gap for the UL coverage between around 7 GHz and existing 5G mid-band (~3.5GHz). In our initial test in the filed, we found there exists ~15dB gap between around 7 GHz and 3.5GHz. Thus it is quite important to discuss how to further improve the UL coverage consideration for 6GR.

In our view, for 6GR UL coverage consideration from RAN4 perspective, some aspects of large array UL MIMO, coherent UL MIMO, low PAPR waveform, relax requirement such as EVM/MPR to increase the transmission output power can be considered:

- Large array UL MIMO/TxD: Large Array UL MIMO can fundamentally improve uplink coverage with the benefits of power combining, array gain such as spatial multiplexing gain and diversity gain etc. This "array gain" directly improves the SNR. The base station can receive a signal than a single antenna ever at the edge of the cell, where the signal is weak, the high SNR allows for the use of higher order modulation schemes (e.g., 256QAM instead of QPSK), resulting in faster upload speeds. In 5G NR, 4Tx is only for FWA form factor and PC1.5 is supported in Rel-19 and PC1(or PC1b) is supported in Rel-20. However, for handheld UE, only PC1.5 via 2Tx with 2*26dBm is supported. To increase the UL transmission power, large array UL MIMO/TxD with high capability PA should be taken into account for handheld UE, especially for around 7GHz.

- Coherent UL MIMO: Coherent UL MIMO occurs when multiple antennas on a user device transmit the same signal in a time-synchronized and phase-coherent manner. This allows the base station to see the multiple transmissions as one powerful, combined signal. Coherent UL MIMO requirements were defined in 5G NR but it was never be implemented in the field. For 6GR, we think it is reasonable to enable Coherent UL MIMO feature.

- Low PAPR waveform(FDSS): In 5G NR, the FDSS w/o spectrum extension was introduced to reduce the PAPR for QPSK and pi/2 BPSK, and FDSS w/o is only for DFT-s-OFDM. With FDSS help, the signal in the frequency domain can be smoothed which directly reduces the time-domain peaks. This results in a lower PAPR. Lower PAPR means the UE’s PA can transmit higher power and operate at much more efficiently. It can be driven closer to its maximum power without distortion. This means more effective radiated power is sent towards the base station.

For 6GR, it’s reasonable to further consider this direction, including high order modulations, and CP-OFDM as well.

- Relax EVM/MPR requirements: For high order modulations such as 64QAM and 256QAM, EVM is the domain factor for the MPR requirements, with the relaxation of Tx EVM requirements for 64QAM and 256QAM and advanced BS receiver, the enhancement of UE output power can be achieved and the uplink coverage for high modulation order can be improved accordingly. This direction was discussed for several meetings when RAN4 discussed the Rel-20 WID. For 6GR, it was already confirmed reduce UE MPR cause with relaxed EVM will be discussed in 6G study.

More details can be found in section 2.2 and 2.3.

Proposal 1: To consider the following aspects for 6GR coverage enhancement in RAN4:

- Large array UL MIMO

- Coherent UL MIMO

- Low PAPR waveform (FDSS) for high order modulation and CP-OFDM

- Relax EVM/MPR requirements

Of course, there are some other aspects discussing in RAN1 which can also be considered for 6GR coverage enhancement.

## 2.2	UE RF capabilities

In this section, we provide some initial considerations on UE RF capabilities including considering device types and implementations such as PA capability, power class and coherent UL MIMO.

### 2.2.1 PA capability

As we know, the PA maximum output is usually higher than the maximum output power at antenna connector defined by power class due to the RFFE insertion loss. For example, 27dBm@PA output power for 23dBm@antenna connector with 4dB RFFE loss. With the developing of state of the art, high PA maximum output (such as 31dBm) with reasonable size have already been available.

Usually, companies use the power equals to power class to describe PA capability in the discussion for simplification. In the past discussion, several PA configuration were included, such as the PA configurations are 2*26dBm for PC1.5 TDD, 2*23dBm for 2Tx UL MIMO, or 2*23dBm+2*26dBm for HPUE 4Tx UL MIMO. In this case, the PA capability somehow decide how much power or what is the power class would be in RAN4 discussion for handheld UE and/or FWA form factor.

For 5G commercial mainstream handheld UE, due to the size limitation, usually only up to 2Tx/2PA for single band is supported to balance the performance and cost. Also for a UE, some bands can share a common Tx/PA which is illustrated in Fig. 2.2.1-1, where the frequency for these bands are close.

Fig 2.2.1-1. Bands share a common Tx/PA

Therefore, in order to support high UL transmission power, enhance one PA capability in Tx chain is more important than increasing PA/Tx chains numbers for handheld UE. That is why all the current 4Tx HPUE topics are all for FWA form factor.

For 6GR, we think RAN4 should study and support PC1.5 1Tx TDD/FDD band with 1*29dBm PA for handheld UE, this issue was proposed in RAN #109 meeting when companies discussed the objectives for Rel-20 but it is only for FWA form factor in the end, and this work is to start in 2026 Q4. Moreover, further study 2Tx TDD/FDD high band with 2*29dBm for handheld UE.

Proposal 2: To study 1*29dBm PA to support PC1.5 1Tx for handheld UE in 6GR.

### 2.2.2 Power class

In 5G NR, there are PC1, PC1.5, PC2, PC3 and PC5 are introduced. In the beginning of 5G NR study, RAN4 only focus on PC3 study and later, HPUE feature are introduced not only for single band (including UL MIMO), but also for band combination.

The step size between two adjacent power class is 3dB, however, the maximum output power for PA capability may not always 3dB step since different implementation of RFFE. For example when the PA maximum output power is 27dBm, the RFFE insertion loss can be 2dB, then 25dBm maximum output power can be achieved. However, there is no power class equal to 25dBm.

In addition to that, up to 4Tx UL MIMO is supported in 5G/5G-adv although it is for FWA form factor. The supported PA power configurations for each power class for TN FDD and TDD are summarized in table 1.

Table 2.2.2-1. PA power configurations for TN power class

|  | PC3 | PC2 | PC1.5 | PC1 or PC1bis |
| --- | --- | --- | --- | --- |
| TDD | 1T | 1T, 2T | 1T, 2T, 4T | 2T, 4T |
|  | 1*23 | 1*26, 2*23 | 1*29, 2*26, 4*26 | 2*29, 4*26 |
| FDD | 1T | 1T, 2T | 2T |  |
|  | 1*23 | 1*26, 2*23 | 2*26 |  |
| Note: FWA UE only |  |  |  |  |

Usually the UE maximum output power would be restricted by its power class. In other words, the PA full power capability would be constrained artificially. For example, the PA capability in PC3 2Tx UL MIMO would be constrained, and another example is 2*29dBm PA can ideally achieve 32dBm maximum output. However, there is no 5G NR power class equal to 32dBm. Moreover, in the future there may 8Tx UL MIMO supported for FWA form factor, and if 26dBm PA is used in each Tx RF chain, then the maximum output power would ideally achieve up to 35dBm. However, there is no 5G NR power class equal to 35dBm.

In 5GR, the RF requirements were defined only for symmetric UL MIMO/TxD were considered in the specification. In the symmetric UL MIMO/TxD, the power at each connector is the same. However, during the discussion, asymmetric UL MIMO/TxD were also mentioned such as in Rel-18 NR_ENDC_ RF_FR1_enh2 WID, 4x23dBm, 4x26dBm and 2x23dBm + 2x26dBm were also considered for 4Tx PC1.5. If considering asymmetric UL MIMO/TxD, we think it would more complicated by using the existing 5G NR power class framework.

For 6GR, to solve the above issues, we think the single band power class framework should reflect the actually PA capability, especially for UL MIMO. One straightforward way is to introduce more power classes but it would cause many fragment power classes introduced which would make complexity for the requirements definition. Some other way may need to be discussed such as to use the power per antenna connector or per PA for the real transmission output power, and if UE supports UL MIMO, then the real transmission output power would be sum of the real power per antenna connector.

Observation 2. For single band, the current power class definition in 5G may constrain the UE maximum output power, especially for UL MIMO.

Observation 3. Introduce too many power classes would cause power class fragment and complexity for the requirements definition.

Proposal 3. For 6GR, to discuss the power class framework to reflect the PA capability for single band, especially for UL MIMO.

In addition, as we mentioned in section 2.1.2, UL coverage is a important thing for 6GR consideration, especially for around 7GHz. In current 5GR, the power class 3 (i.e. 23dBm) is a default power class no matter for low band or high band. However, we think the HPUE single band implementation is quite mature in 5G NR commercial UE even for PC2 FDD band RedCap UE, 1*26dBm PA have already been considered only when RAN4 define the related RF requirements. Therefore, we think at least for high TDD band such as around 7GHz in 6GR, high power class like PC2 can be used as a default power class.

Proposal 4. To consider high power class like PC2 as a default power class for high TDD band such as around 7GHz in 6GR.

For HPUE inter-band band combination in 5G NR, high power limits capability (i.e. higherPowerLimit-r17) was introduced to enable UE can transmit higher power considering the PA full power capability. For example, PC2 inter-band CA with 23+26dBm PA configuration can transmit 27.8dB (i.e. 23+26dBm) maximum output power with the IE higherPowerLimit-r17. Noted that the high power limits capability is only applied to inter-band band combination, it is not applied to pure intra-band contiguous/non-contiguous CA.

Observation 4. In NR, the IE higherPowerLimit-r17 can enable higher maximum output power for inter-band combination, but it is not applied to pure intra-band contiguous/non-contiguous CA.

For 6GR, we think the PA capability for each CC should be reflected well, instead of using an optional IE in 5G NR to enable higher maximum output power for inter-band combination. In the other word, we think it is reasonable that sum of the power of each band is mandatory to be supported.

Another issue for inter-band CA band combination in 5G is the UL maximum output power is restricted by the band combination power class when all CCs in one UL band are deactivated, UL configuration become single UL band transmission. In 5G NR, there are two following cases:

1. DL CA with single carrier UL configured

2. Any ULCA with only one CC activated or scheduled but others deactivated or not scheduled,

For the case 1, it was solved in 5G NR where HPUE applicability notes do not restrict power class indication for single configured UL CC with DL CA. However, for case 2, the transmission power for the single UL band transmission would still be limited by the band combination power class although it can transmit higher power. This issue was discussed for ~2years in Rel-18/19 but the consensus is only to include the single configured UL CC with DL CA as exception in the specification.

Observation 5. For UL CA configuration in NR, the transmission power is restricted by the band combination power class.

In 6GR power class framework, we think this 5G left-over issue should continue discussing in 6GR study phase. We think the restriction of the band combination power class should be released. To achieve the above goals in 6GR, to achieve these goals, one of an enhanced method is to remove the band combination power class. In this way, the transmission output power for UL CC is sum of the power class of each band or CC, and when ULCA with only one CC activated or scheduled but others deactivated or not scheduled, the UL transmission output power is decided by the power of the current transmitting CC.

Proposal 5: For CA power class framework in 6GR, the restriction of the band combination power class should be released in the power class framework discussion.

Proposal 6: To discuss the necessity of band combination power class in 6GR.

### 2.2.3 Coherent UL MIMO

Coherent transmission allows the system to create a virtual, focused beam of energy pointed toward the base station. Consequently, the base station effectively sees a single, much stronger signal. A stronger signal can travel farther and better penetrate obstacles like walls, directly extending the range and coverage area of the device.

In 5G NR, the coherent UL MIMO requirements were introduced from Rel-15 for both FR1 and FR2 as follow, and meanwhile there are some conditions defined which shall be met within the specified time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted (from TS38.101-1) Difference of relative phase error; Difference of relative power error; Time window. 40 degrees; 4 dB; 20 msec. - UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config - UE remains in DRX active time (UE does not enter DRX OFF time) - No measurement gap occurs - No instance of SRS transmission with the usage antenna switching occurs - Active BWP remains the same - EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PScell or SCell(s))

However, it was hardly implemented in the filed due to some below challenges:

- Stringent requirements must be met in both the Baseband (digital processing) and the RF (analog hardware) parts of the transceiver, i.e. RF chains between RFIC are well synchronized (e.g. shared PLL and shared Qgen circuit).

- The baseband must know the channel, calculate the correct precoding weights, and synchronize the data streams to set the stage for coherent transmission,

- The RF hardware must preserve the precise phase and amplitude relationships set by the baseband. Any unknown drift or mismatch between the transmit chains such as timing misalignment caused by PA, front-end filter/combiner during the OEM assembling will destroy coherence.

- Interaction between Baseband and RF, such as calibrate phase and amplitude imbalances of each RF chain, shall be needed.

- Coherent UL MIMO has been introduced from Rel-15 specification, however due to the following potential implementation problems, UL coherent MIMO is not very successfully commercialized yet in the .

- RF chains between RFIC are well synchronized (e.g. shared PLL and shared Qgen circuit)

- Additional timing misalignment caused by PA, front-end filter/combiner during the OEM assembling etc.

An example overview for coherent UL MIMO implementation with the DPD link as alternative feedback link for internal calibration can be found in Figure 2.2.3-1.

- Figure 2.2.3-1. Illustration example for coherent UL MIMO implementation

For 6GR spectrum around 7GHz, it is desirable to re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments and targeting comparable coverage to 5G mid-high spectrum (~3.5GHz). However, the UL coverage for the spectrum around 7GHz would be smaller than the 5G mid-high spectrum (~3.5GHz) due to the more severed propagation loss caused by the higher frequency.

In the Figure 2.2.3-2, we provide some information for the UL coverage comparison between 3.5GHz and around 7GHz with or without coherent transmission.

- (a) cell edge throughput

- (b) Average throughput

- Figure 2.2.3-2. Average throughput and cell edge throughput for UL coherent MIMO

- We can observe that:

- 1. For the cell-edge, UPT gain would be rapidly increased with coherent transmission. With the increasing of the Tx/Rx number, the UPT gain will also increase.

- 2. For the average throughput, similar tendence can be found.

- Observation 6: Besides for increase of #. Tx in a UE, UL coherent transmission and corresponding high-resolution UL codebook become critical for boosting up UL transmission, especially for cell-edge UE (50~100% UPT gain).

Hence, 6GR should enable coherent UL MIMO feature can further improve the UL coverage.

Proposal 7: Enable Coherent UL MIMO feature for 6GR.

- Actually, the coherent UL MIMO objective was included in Rel-16 FR2 WID NR_RF_FR2_req_enh2. RAN4 spent several meetings to discuss the UL gaps for self-calibration and monitoring for the purposes UE Tx power management (i.e. MPE) and Coherent uplink MIMO. Unfortunately, the Coherent uplink MIMO objective was removed form the latest WID due to there were no final consensus achieved in the end. In later releases, there were no following up work related to coherent UL MIMO anymore.

- Nevertheless, there were some intermediate agreements achieved. Especially for the gain and feasibility of UL gap for UL coherent MIMO in the WF R4-2119962. Later that, the details for UL gap were discussing like UL gap length, gap pattern, gap indication and so on. But unfortunately, there are no final outcomes. In addition to that, the Measurement point for difference of relative phase/power error for UL coherent MIMO are captured in both TS38.101-1 and TS38.101-2.

- In our opinion, UL gaps for coherence calibration to meet the relative phase and power requirement for coherent MIMO might be one feasible way similar as antenna calibration within AAS BS for the accurate beamforming steering since UE may not always do the calibration due to limitation of hardware. In this case, UE needs UL gap to conduct the internal calibration before PUSCH transmission to do the phase error adjustment.

- For 6GR, we think the UL gap for coherence calibration could be further discussed for FR1 to enable coherent UL MIMO feature for UL coverage improvement purpose.

Proposal 8: To meet the relative phase and power requirement for coherent UL MIMO, UL gap for coherence calibration could be further studied in 6GR if necessary.

## 2.3	UE RF requirements improvement and/or simplification

### 2.3.1 MPR/A-MPR improvement

According to the 6GR SID[1], 6G is required to enhance UL coverage and cell-edge performance. It is no doubt that higher uplink transmit power is helpful on uplink coverage enhancement, and some uplink coverage improvement topics were discussed in 5G NR. However, UE is power-limited, which means UE is not allowed to always transmit maximum output power because of the limitations of ACLR, SEM, IBE and EVM requirements. Power backoff is an efficient way adopted by a UE to address the emissions and distortions caused by PA non-linearity characteristic in order to guarantee UE can meet the emission and EVM requirements. Especially for high-order modulation with CP-OFDM waveform, the PAPR is high and the UE output power needs to be backed off a lot, more power back-off means more coverage is shrunk.

Therefore, MPR/A-MPR improvement can be one of the critical methods to enhance uplink coverage in 6GR study.

As discussed in RAN1#122 meeting, RAN1 agreed to take 5G NR CP-OFDM and DFT-s-OFDM waveform as 6GR waveform basis and consider enhancements/modifications on CP-OFDM and DFT-s-OFDM to further reduce the PAPR of the uplink signal. Further reduced PAPR allows efficient utilization of PAs and provides possibility to reduce MPR/A-MPR.

Agreement in RAN1#122 meeting: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded.

Observation 7: RAN1 agreed to take 5G NR CP-OFDM and DFT-s-OFDM as 6GR waveform basis and consider enhancements/modifications on CP-OFDM and DFT-s-OFDM to further reduce the PAPR of the uplink signal, which provides possibility to reduce MPR/A-MPR.

In the beginning of 5G NR FR1 discussion, some of the RF parameters assumptions like PA model and IQ image/carrier leakage in MPR/A-MPR studies were reused from LTE.

On the one hand, MPR and A-MPR requirements are derived from simulation based on 1dB PA calibration point and reference waveform, i.e., “1dB MPR for waveform with CBW=20MHz, SCS=15kHz, QPSK, DFT-s-OFDM, 100RB0”. With the experience of some topics at the later stage of 5G NR, the LTE PA model is not suitable anymore, and some companies have already used their own PA model assumptions for the MPR/A-MPR requirements evaluation. Frankly speaking, the MPR/A-MPR requirements of high order modulations a little bit high in the specification, especially for the high power UE. This PA calibration point may not be feasible for 6GR MPR evaluation if we want to reduce MPR.

Besides, in contrast to the maximum 100MHz CBW in FR1 of 5G NR, 6GR is going to introduce wider CBW (at least 200MHz) support for 6G deployments at least above 2GHz, around 7GHz[1]. PA non-linearity could be very different due to non-negligible memory effect caused by wider CBW. Moreover, the PA model in LTE phase was based on low frequency range, it would not appropriate for higher frequency range such as round 7GHz. Therefore, we think new PA models for 6GR MPR evaluation should be discussed.

On the other hand, MPR simulation assumptions in 5G NR are -28dB IQ image and -28dBc carrier leakage, which are also reused from LTE. However, in the reality design, the practical IQ image and carrier leakage can both achieve -35dB/dBc which are far better values than the existing assumptions. With the experience of FR2 256QAM discussions, better IQ image and carrier leakage can contribute to better MPR requirements, especially for high-order modulation whose EVM is highly impacted by IQ image and carrier leakage.

Observation 8: PA model/calibration point, reference waveform, IQ image and carrier leakage in 5G NR are reused from LTE, which may not be feasible for 6GR.

Proposal 9: New PA models as well as improved IQ image and carrier leakage requirements should be discussed for 6GR MPR /AMPR evaluation.

With all of the analysis above, we see great potential to improve MPR/A-MPR requirements on top of 5G NR MPR/A-MPR. Therefore, we did some preliminary simulation based on improved IQ image, carrier leakage and PA linearity. Take the MPR evaluation for 16QAM and 64QAM with DFT-s-OFDM and CP-OFDM shown in Figure 2.3.1-1 and Figure 2.3.1-2 as an example.

Figure 2.3.1-1  MPR of 16QAM with DFT-s-OFDM and CP-OFDM

Figure 2.3.2-2  MPR of 64QAM with DFT-s-OFDM and CP-OFDM

Although the simulation results are not that accurate because first of all RAN4 needs to align simulation parameters and assumptions, we can see great potential to reduce MPR values and infer some possible directions for MPR/A-MPR improvement: First, compared to 5G NR, the MPR values could be further reduced. Second, the overall RB allocation range could be redivided, at least inner RB allocation range could be further enlarged.

- Proposal 10: To discuss how to redivide the overall RB allocation range for 6GR MPR on the top of the 5G NR MPR RB regions.

### 2.3.2 Tx EVM improvement

When evaluating the MPR for high-order modulation, such as 256QAM, we can find that the limiting factor of MPR is EVM. In order to satisfy 3.5% EVM requirement of 256QAM, much power backoff is needed to mitigate the distortion caused by PA non-linearity and the UL coverage is very limited. Currently in 5G NR, some advanced signal processing techniques to mitigate nonlinear distortions such as DPD have been mature and well-implemented.

Poor EVM requires high SNR. In short, higher-order modulation would significantly reduce UL coverage and cell edge performance due to the UL transmit power would be low and also it demands high SNR. Once a UE moves away from the NW, then the path loss would increase, the high order modulation may drop to maintain a reliable connection.

Thus, in order to leverage the high order modulation benefit, relax EVM restriction to relax MPR would In terms of the discussion, the key point for relaxing Tx EVM is to use more effective mechanism to compensate the Tx non-linear distortion. The mechanism includes AI (artificial intelligence) based BS receiver and non-AI based BS receiver.

This topic was also discussed in 5G-Adv and 6GR, and it should focus on high order modulation such as 64QAM and 256QAM. The scope for 5G-Adv will be further discussed under 6GR agenda item. Also, it is clear that 5G-Adv solution should not be AI based and there should not be any impact to RAN1, and for 6GR, AI-based BS receiver will be assumed.

No matter for 5G-Adv and 6GR Tx EVM relaxation, we think the common issue is that the non-linearity issue from UE transmitter side should be investigated first including the appropriate PA models including memory effect caused by wider channel bandwidth, IQ image, carrier leakage etc; In addition to that, the BS implementation complexity should also be taken into account.

Due to this topic may also be discussed in 5G R20 without AI-based solution, it would be desirable that the Tx mode including PA model, IQ image/carrier leakage, CFR, DPD would be same for 5G R20 and 6GR to reduce the workload.

Proposal 11: The evaluation for relaxing Tx EVM should priority high order modulation.

Proposal 12: Non-linearity issue including PA model, IQ image/carrier leakage, CFR, DPD for UE transmitter side should be investigated for Tx EVM relaxing.

### 2.3.3 Configured transmitted power simplification

In TS38.101-1, configured transmitted power requirements (a.k.a Pcmax) are defined as equations with a number of parameters associated with lots of descriptions for the parameters.

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

Several features are included in the above equation, such as ΔPPowerClass is for duty cycle SAR solution, ΔPPowerBoost is for power boosting to enhance the UL coverage, ∆TRxSRS is for SRS antenna switching xTyR cases, and ΔTIB,c is for a band in different band combinations.

For ∆TRxSRS, the value mainly come from the average of values proposed by different companies, and for 6Rx/8Rx feature, the values are too high which may not reflect the realistic implementation. Even this requirement was never verified/tested in the filed although it existed in the specification for a long time. In Rel-19, companies spent almost one release to discuss how to solve the SRS insertion loss imbalance due to high ∆TRxSRS values are defined in the specification but no consensus at all. We think this issue should be solved anyway in 6GR. Otherwise, it may not make sense to include it in the equation.

Observation 9: ∆TRxSRS requirement was not verified and the SRS insertion loss imbalance was not solved in 5G NR.

Another aspect is the duty cycle description for ΔPPowerClass and ΔPPowerBoost. The ΔPPowerClass is used to the maximum output fallback when the P-max is singled and lower than the power class or duty cycle is not met. For the duty cycle for HPUE TDD band, it seems it was never used in filed according to the previous discussion. One of the main reason was that the exact evaluation period is not clear in the text like ‘...the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame)’, it would make misalignment between NW and UE since NW have no idea what the exact evaluation period is for duty cycle evaluation. For PC2 FDD band, there was no duty cycle SAR solution introduced in the specification, instead of using the P-MPR solution.

Observation 10: Duty cycle SAR solution was not implemented in 5G NR according to the previous discussion.

Overall, we think the requirements are a bit too complicate to understand well, and these exists some unsolved issue as mentioned as above. We are wondering if it is feasible to simplify the configured transmitted power requirement, as well as to solve issues of ∆TRxSRS and duty cycle SAR solution.

Proposal 13: To simplify the configured transmitted power requirement if possible, as well as to solve issues of ∆TRxSRS and duty cycle SAR solution.

### 2.3.4 The boundary between out-of-band emissions and spurious emissions domain

Regarding the definition of the boundary between out-of-band emissions and spurious emissions domain, ITU-R SM.329 was used as a reference since UTRA was specified in 3GPP. The rule in SM.329 is that “ the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission”. In 3GPP, it specifies the boundary between NR out of band and general spurious emission domain as BWchannel + 5 for above 3MHz channel bandwidth and 6MHz for 3MHz CBW respectively. It can be observed that only OOB boundary defined for 3MHz and 5MHz CBW conforms to 250% of necessary bandwidth rule. For channel bandwidth more than 5MHz, OOB boundary is stricter than 250% of necessary bandwidth.

For Rel-19 power boosting and/or MPR reduction, it aims to specify the power domain enhancement for further MPR reduction for NR single carrier and NR intra-band UL CA. Since power enhancement is restricted by out-of-band emission requirements, e.g., ACLR, SEM and SE requirements, this scheme achieves MPR reduction in outer/edge RB allocation by relaxing emission requirements under the conditions where no co-existence issue is caused especially for the deployment of two adjacent frequency blocks, which make it feasible to further enhance the Tx power.

One of the method in Rel-19 discussion is to shift the OOB boundary based on the extended CBW in each or both side of UE CBW, which results in relaxed MPR in the outer/edge RB region. With the similar idea as Rel-19 that relaxing OOB boundary will cause the benefit of low MPR, when RAN4 define the OOB requirements in 6GR, the boundary between OOB emission and spurious emission domain could be discussed to further enhance UL coverage.

Observation 11: OOB boundary defined for channel bandwidth more than 5MHz in 3GPP is stricter than 250% of necessary channel bandwidth rule in ITU-R SM.329.

Proposal 14: It is proposed to discuss the boundary between out-of-band emissions and spurious emissions domain.

If we further look at SM.329 regulation, we can found that

“In case of very narrow or wide bandwidth, this method of determining the spurious domain might not be appropriate and Recommendation ITU-R SM.1539 provides further guidance.

Additional guidance concerning the necessary bandwidth required for determining the boundary between the out-of-band and spurious domains can be found in recommends 2.3 of Recommendation ITU-R SM.1541.”

And in SM.1539, it states that

“For some narrow-band emissions, it is appropriate to avoid specifying OoB and spurious domains in very narrow bandwidths near to the emission, which will usually be under the control of the same operator. On the other hand, it is necessary to restrict the linear growth of the OoB domain versus necessary bandwidth for wideband emissions, in order to restrict the incursion of the OoB domain into adjacent bands.”

“Table 2 provides guidance for determining general values of separation between the centre frequency of an emission and the beginning of its spurious domain. A minimum separation applies to the narrow-band case, while the normal and wideband cases have separate formulas for determining the boundary”

As mentioned in the table above, BN refers to the necessary bandwidth. It can be observed that OOB boundary is not uniformly defined as 250% of necessary bandwidth in SM.329, SM.1539 and SM.1541. Specifically, for wide band, OOB boundary is set to less than 250% of necessary bandwidth. Therefore, we propose to take SM.1539 and SM.1541 into consideration when discussing OOB boundary in 6GR.

Observation 12: OOB boundary is not entirely defined as 250% of necessary bandwidth in SM.329, SM.1539 and SM.1541, and OOB boundary is smaller than 250% of necessary bandwidth for wide band.

Proposal 15: It is proposed to take SM.1539 and SM.1541 into consideration when discussing OOB boundary in 6GR.

## 2.4	RF requirements for new channel bandwidth

According to SID, wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. It is no doubt that the all of the related Tx/Rx requirements for the new channel bandwidths should be discussed. As we discussed in section 2.2, we think it is important to use practical RF parameters/PA model assumption for MPR/A-MPR requirement considering more server PA memory effects would be foreseen for the large channel bandwidth.

Also, for range between FR1 and FR2-1 (including around ~7GHz), the UE RF architecture and requirements should be studied based on TR38.921/38.922, including wider channel bandwidth(at least 200MHz).

Moreover, if there is wider than 400MHz channel bandwidth is introduced for FR2-1, then all of the related Tx/Rx requirements should be studied either.

Proposal 16: All of the related Tx/Rx requirements for the new wider channel bandwidths should be studied based on practical RF parameters/PA model assumption.

# 3 Conclusion

In this contribution, we give some initial views on the 6GR system parameters, the conclusions are summarized below:

UL coverage:

Observation 1. There were many mechanisms introduced to increase UL coverage in 5G NR.

Proposal 1: To consider the following aspects for 6GR coverage enhancement in RAN4:

- Large array UL MIMO

- Coherent UL MIMO

- Low PAPR waveform (FDSS) for high order modulation and CP-OFDM

- Relax EVM/MPR requirements

UE RF capabilities:

Proposal 2: To study 1*29dBm PA to support PC1.5 1Tx for handheld UE in 6GR.

Observation 2. For single band, the current power class definition in 5G may constrain the UE maximum output power, especially for UL MIMO.

Observation 3. Introduce too many power classes would cause power class fragment and complexity for the requirements definition.

Proposal 3. For 6GR, to discuss the power class framework to reflect the PA capability for single band, especially for UL MIMO.

Proposal 4. To consider high power class like PC2 as a default power class for high TDD band such as around 7GHz in 6GR.

Observation 4. In NR, the IE higherPowerLimit-r17 can enable higher maximum output power for inter-band combination, but it is not applied to pure intra-band contiguous/non-contiguous CA.

Observation 5. For UL CA configuration in NR, the transmission power is restricted by the band combination power class.

Proposal 5: For CA power class framework in 6GR, the restriction of the band combination power class should be released in the power class framework discussion.

Proposal 6: To discuss the necessity of band combination power class in 6GR.

Coherent UL MIMO

- Observation 6: Besides for increase of #. Tx in a UE, UL coherent transmission and corresponding high-resolution UL codebook become critical for boosting up UL transmission, especially for cell-edge UE (50~100% UPT gain).

Proposal 7: Enable Coherent UL MIMO feature for 6GR.

Proposal 8: To meet the relative phase and power requirement for coherent UL MIMO, UL gap for coherence calibration could be further studied in 6GR if necessary.

UE RF requirements improvement and/or simplification

Observation 7: RAN1 agreed to take 5G NR CP-OFDM and DFT-s-OFDM as 6GR waveform basis and consider enhancements/modifications on CP-OFDM and DFT-s-OFDM to further reduce the PAPR of the uplink signal, which provides possibility to reduce MPR/A-MPR.

Observation 8: PA model/calibration point, reference waveform, IQ image and carrier leakage in 5G NR are reused from LTE, which may not be feasible for 6GR.

Proposal 9: New PA models as well as improved IQ image and carrier leakage requirements should be discussed for 6GR MPR /AMPR evaluation.

Proposal 10: To discuss how to redivide the overall RB allocation range for 6GR MPR on the top of the 5G NR MPR RB regions.

Proposal 11: The evaluation for relaxing Tx EVM should priority high order modulation.

Proposal 12: Non-linearity issue including PA model, IQ image/carrier leakage, CFR, DPD for UE transmitter side should be investigated for Tx EVM relaxing.

Observation 9: ∆TRxSRS requirement was not verified and the SRS insertion loss imbalance was not solved in 5G NR.

Observation 10: Duty cycle SAR solution was not implemented in 5G NR according to the previous discussion.

Proposal 13: To simplify the configured transmitted power requirement if possible, as well as to solve issues of ∆TRxSRS and duty cycle SAR solution.

Observation 11: OOB boundary defined for channel bandwidth more than 5MHz in 3GPP is stricter than 250% of necessary channel bandwidth rule in ITU-R SM.329.

Proposal 14: It is proposed to discuss the boundary between out-of-band emissions and spurious emissions domain.

Observation 12: OOB boundary is not entirely defined as 250% of necessary bandwidth in SM.329, SM.1539 and SM.1541, and OOB boundary is smaller than 250% of necessary bandwidth for wide band.

Proposal 15: It is proposed to take SM.1539 and SM.1541 into consideration when discussing OOB boundary in 6GR.

RF requirements for new channel bandwidth:

Proposal 16: All of the related Tx/Rx requirements for the new wider channel bandwidths should be studied based on practical RF parameters/PA model assumption.

# 4	Reference

[1]RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)