3GPP TSG-RAN WG4 Meeting #116bis		   R4-2513317
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on 6GR spectrum

Source:	OPPO

Agenda item:	8.6

Document for:	Approval

- Introduction

In RAN #108 meeting, the study item on 6GR was approved in [1], and the 6GR spectrum research is limited to below 52.6GHz, that is, the frequency ranges beyond 52.6 GHz are not in scope of the work.

In last RAN4 meeting，RAN4 6G Plan and meeting arrangement was given in [2]. The spectrum scope includes:

- Band definition and simplification

- Band combination definition and simplification

- The definition of frequency ranges

- 6G spectrum related other aspects, including the regulatory status survey

In this contribution, we mainly discuss the above 6GR spectrum scope and related issues.

- Discussion

  - Frequency ranges

2.1.1 NR Freq range

In 6GR SI, one objective of core and performance requirements is

- Definition of ‘frequency range(s)’[RAN4]

In 5G NR, the frequency range definition mainly depends on different UE implementation results in different RF requirements, the different SCS in different frequency range mainly depends on the maximum channel bandwidth and the delay requirements of the application scenario, as shown in Figure 1:

Figure 1 The relationship among FRs, RF requirements interface and SCS in 5G NR

In 5G NR, the reason why the RF requirements for FR2 are defined based on radiated characteristics rather than the same as FR1 which is based on conducted characteristics is mainly because of UE implementation rather than SCS:

- FR2 belongs to millimeter waves, the wavelength is relatively short, correspondingly, the size of RF proponents is relatively small.

- Moreover, the spatial transmission loss of millimeter waves is significant. The beamforming gain of the array antenna needs to be utilized to enhance the transmission distance in UE implementation.

- Further, each antenna element of the array antenna needs one Tx and Rx RF chain, the array antenna and the RF front-end are highly integrated, resulting in the RF front-end lacking a conductive test port. In additional, conductive test can’t reflect the array antenna’s beamforming gain.

Observation 1:	Millimeter waves categorized to FR2 is mainly due to adopt array antenna and lack a conductive test port.

In RAN1 #122 meeting, how to categorize the frequency range to use for 6GR numerologies, frame structure and MRSS was discussed.

In RAN #109 meeting, how to categorize the frequency range was discussed again in [3] based on the discussion in the RAN1 #122 meeting, the issues were excerpted here:

From contribution submitted to this meeting and the discussion that we had in RAN1#122, there are two controversial issues, for TN - Issue 1: How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group? - Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology. - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family. - Option 3: define separate FR for 7.125 – 24.25 GHz. - Issue 2: whether we need to highlight the specific 15GHz in the study or simply use a frequency range, e.g., Sub-24.25 GHz to cover 15GHz (Lower bound is dependent on the detailed definition of around 7)? - Potential way forward: for frequency range 24.25 – 52.6 GHz, we directly use frequency range rather a specific frequency point in the agreement, perhaps we can go with same logic, simply use frequency range, e.g., below 24.25 GHz rather than 15GHz. As to which frequency in this range is to be used for evaluation, it can be up to the output of RAN requirements (if confirmed in this meeting)

RAN1/RAN consider the numerology as the logic to categorize frequency range. The problems for them are what numerology should be defined for the existing bands and how to treat new 6G frequency spectrum of 7.125 – 24.25 GHz and related numerology.

Observation 2:	RAN1 is discussing frequency range based on numerology, which is different from RAN4 where the conducted/OTA requirement definition is the focus. RAN4 need to be deeply involved in the frequency range definition.

Proposal 1:	The frequency range definition shall consider the conducted/radiated requirement definition in RAN4 and also the UE implementation impacts.

2.1.2 Mixed numerology for same freq (sub)range in NR

One key issue for 6G frequency range definition is the alignment of  the frequency range definition for TN and NTN.

In NTN ku band, the original intention is to introduce lower SCS, e.g., 30kHz. Finally, due to the numerology is associated with frequency range which is reflected in the system parameters definition aspect as well as and UE capability signalling, as shown in Table 1 and Figure 2:

Table 1 Numerology for FR1 and FR2 in 5G NR

| SCS | Numerology |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| SCS for data | 15 kHz, 30 kHz, 60 kHz | 60 kHz, 120 kHz |
| SCS for SSB | 15 kHz, 30 kHz | 120 kHz, 240 kHz |

Figure 2 UE capability of CBW associated FR with SCS

To let Ku bands can use two sets of numerology, RAN4 introduced two sets of bands for Ku bands on the same spectrum as shown in Table 2:

Table 2 5G NR NTN ku bands

| FR | NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- | --- |
| FR1-NTN | n248 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz | FDD |
|  | n247 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |
| FR2-NTN | n509 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz | FDD |
|  | n508 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |

Correspondingly, extended the frequency range for both of FR1 and FR2 as show in Figure 3:

Figure 3 The relationship among FRs, RF requirements interface and SCS in 5G NR NTN

Only one set of radiated requirements was defined for ku bands since the VSET UE operating on NTN ku bands used the array antenna.

Observation 3:	In NR, Ku band has set an example of defining two bands within FR1 and FR2 for the same spectrum.

Observation 4:	RAN4 introduced two band for same Ku spectrum, one is FR1 band, the other is FR2 band. The two bands apply same radiated requirements.

In 6G, it should avoid to define multiple sets of bands just to adopt different numerologies for the same spectrum.

Proposal 2:	Multiple numerology should be avoided in the same frequency (sub)range in 6G.

In 6G SID, multiple numerology shall be avoided for the same Band / sub-range. Within each band/ sub-range, only one numerology should be defined. To make different bands can use different numerology, frequency range can be defined based on the interface of UE RF requirements.

Observation 5:	Multiple numerology shall be avoided for the same Band / sub-range.

Proposal 3:	Frequency range can be defined based on the interface of UE RF requirements. Within each frequency range, the applicable numerology can be defined per band or per sub-frequency range basis.

In 5G, the frequency range for TN and NTN became different from R19 due to introduce NTN ku bands, as shown in Figure 4:

Figure 4 Compared TN FR to NTN RR in 5G

Observation 6:	The frequency range for TN and NTN became different from R19.

A harmonized 6G Radio design for TN and NTN is targeted in the 6G SID “Aim at a harmonized 6G Radio design for TN and NTN, including their integration”. From UE implementation, the NTN and TN will be harmonized design in 6G. Freq range (applicable requirements) should also be aligned.

Proposal 4:	In 6G, align the frequency range definition for TN and NTN.

2.1.3 6G frequency range

Another key issue for 6G frequency range definition is how to treat new 6G spectrums.

About new frequency definition, one of important factors is researching the UE implementation feasibility. For different frequency range the key issue is the UE antenna architecture.  In 5G NR, the UE antenna has two kinds of implementation, one is FR1-like isotropic radiation pattern antenna and without beamforming. Another is FR2-like array antenna with analog beamforming.

Actually, in Rel-19, RAN4 has studied the identification of frequency band(s) for the terrestrial component of IMT, e.g., 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz from the RF requirements and BS/UE implementations. The study results have been captured into TR 38.922.

For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

Observation 7:	For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted isotropic radiation pattern antenna interface.

For 14.8-15.35GHz, one of the main challenges is the size of potential panel. As the researching in TR 38.922, the array antenna for different frequencies were compared as shown in Figure 5 from TR 38.922:

Figure 5 Patch array dimensions vs. frequency

We can observe that

- The size of single element for 15GHz is almost 1.5 times that of 24GHz, correspondingly, the array length of 15GHz band is almost 1.5 times that of 24GHz.

- The array length in the 15GHz band is almost twice as long as that of a single element of 7GHz.

The larger array antenna will increase the difficulty of UE implementation. In addition, the number of antennas and RF chains required for 6G UE will be larger than those used in 5G NR, e.g., new 6G bands of 8GHz and 15GHz need separate antennas from 3.5GHz bands. Therefore, FR1-like antenna is more feasible for 14.8-15.35 GHz, from UE design flexibility perspective.

Observation 8:	The array antenna of 15GHz is too large for physical constraints in case of handheld devices.

In TR 38.922, it also studied that FR1 like antenna is feasible for around 15GHz.

Proposal 5:	It is feasible to apply FR1 up to 15300MHz.

For NTN down to 10700MHz FR2 part where the VSAT is mainly targeted, this can be treated separately and is not applicable for TN.

Observation 9:	10700MHz-24250MHz is only used by NTN.

6G FR1 can be extended up to 15300 MHz and FR2 can be extended down to 10700 MHz, 10700MHz-24250MHz is only used by NTN as shown in Figure 6.

Figure 6: 6G frequency range

Proposal 6:	In 6G, define frequency ranges based on the interface of RF requirements, as show in Table 3.

Table 3: The uniform division of frequency range for 6G TN and NTN

| Frequency range | Spectrum range | Interface of RF requirements |
| --- | --- | --- |
| FR 1 | 410 MHz-15350 MHz | Conducted interface |
| FR 2 | 10700MHz-24250 MHz (NTN only) | Radiated interface |
|  | 24250 MHz-52600 MHz |  |

  - Band definition and simplification

2.2.1 New spectrum for 6G

WRC-23 had allocated 6425-7125 MHz and 10-10.5 GHz to international mobile communications (IMT) spectrums.

For 6425-7125 MHz is totally overlapping with NR band n104.

For 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

Observation 10:		6425-7125 MHz is totally overlapping with NR band n104.

Observation 11:	10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

The WRC-27 issue 1.7 aims to find new frequency allocations for future mobile communications (IMT) in the following new frequency bands: 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz.  Based on results of studies, the identification of frequency band(s), to consider for the terrestrial component of IMT:

- 4400-4800 MHz, or parts thereof, in Region 1 and Region 3;

- 7125-8400 MHz, or parts thereof, in Region 2 and Region 3;

- 7125-7250 MHz and 7 750-8 400 MHz, or parts thereof, in Region 1;

- 14.8-15.35 GHz;

Currently, ITU only identified potential new spectrum ranges of 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz.  Therefore, it’s unnecessary to research the spectrum ranges of 15-20GHz in 6G early phase.

Observation 12: Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

New frequency 4400-4800 MHz has been covered by NR band n79, as shown in below Figure 2, therefore, the general parameters can be extracted from requirements defined for NR operating band n79.

Because of n79 will also be considered for re-farming into 6G, RAN4 should first investigate whether the n79 can be re-employed for 6G to cover the new 4400-4800MHz spectrum range of 6G, rather than directly introducing a new 4400-4800MHz frequency band.

Figure 7 candidate new bands in 6GR

Observation 13: New spectrum range of 4400-4800 MHz has been covered by NR band n79, furthermore, band n79 will also be considered for re-farming into 6G.

Proposal 7:	RAN4 should first study whether the n79 can be re-employed for 6G to cover the new 6G spectrum 4400-4800MHz.

ITU may assign different parts of 7125-8400 MHz to different regions in 6G, to facilitate adopt a unified design especially UE RF front-end, RAN4 should first investigate whether 7125-8400 MHz can be introduced as a global band for 6G.

Observation 14: For the candidate spectrum 7125-8400 MHz, NS value could be used to solve specific requirements.

Proposal 8:	RAN4 should strive to define global bands for 7125-8400 MHz and 14.8-15.35 GHz for 6G.

2.2.2 Re-farming bands

Although 5G NR has defined 70+ FR1 bands and 6 FR2-1 bands, many frequency bands even for FR1 have not been actually deployed due to various reasons. The similar problems will still exist, and not all 5GR bands can be re-farmed into 6G quickly in 6G day one, operators need to coordinate the allocation of 6G and 5G frequencies for re-farming bands.

Observation 15: 6G will coexist with 5G NR for a long time, and operators also need time to re-farm the 5G spectrum.

Proposal 9:	FR1 re-farming bands should be introduced step by step based on Operators’ request.

Proposal 10:	For FR1, reuse 5G requirements for the same band with same PC, e.g., reuse same hardware for both 5G and 6G.

In the beginning of 6G, a variety of aspects will be brought up and required to be enhanced here, however, with limited TU, it is important for RAN4 to identify the most important aspects that need to be focus on.

In additional, so far there is only a few countries deployed 5G in FR2 across the globe due to the limitation of the application scenarios.

Observation 16: With 6G limited TU, it is important for RAN4 to identify the most important aspects to focus on.

Proposal 11:	The re-farming of FR2 bands should depend on the level of interest from industry.

Proposal 12:	Reuse 5G FR2 UE type/power class and Requirement directly.

2.2.3 Re-farming band simplification

As description in the 6G study item, from the technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). Therefore, the 6G bands mainly come from Re-farming bands from 5G NR and 6G new bands.

In current 5G NR spec, it has defined 70+ FR1 bands and 6 FR2-1 bands, some bands are partially overlapping as shown in below Figure 8:

Figure 8 Partially overlapping bands in 5G NR

These overlapping bands defined through separate band number is due to they are deployed in different countries or regions. For example, for overlapping bands of band n85 and band n12, n85 was defined for 700 MHz A Block spectrum of America, and n12 is only be deployed in Japan; for band n105 and n71, n105 is almost deployed in APT and n71 is deployed mainly in north America; for band n77 and n78, n77 is deployed in those countries like North America and Japan that require ultra-wideband, but n78 is one 5G frequency band globally, with extremely strong universality in Europe, China, and the Asia-Pacific region, and so on.

Although these overlapping frequency bands were defined as separate band in 5G, with the evolution of technology, the UE implementations have begun to adopt a unified design especially RF front-end to support dual overlapping frequency bands. For example, n2 and n25, n77 and n88, n38 and n41, these overlapping bands have been unified design using one set of RF front-end in UE implementation.

Observation 17: In 5G NR, many overlapping bands have adopted same RF front-end in real UE implementation which makes defining separate bands is not meaningful.

In additional, 5G NR has defined tens of thousands of band combs, more band numbers will cause more band combination numbers, this trend is unsustainable in the future. RAN4 should avoid to define separate band numbers for overlapping frequency bands. For example, RAN4 should consider to introduce band n25 into 6G when operators’ request re-farm band n2 into 6G, since the spectrum range of n25 is totally overlapping with band n2 and its spectrum range is larger than n2.

Proposal 13:	Consider to merge overlapping bands as much as possible when re-farming FR1 bands.

  - Band combination definition and simplification

2.4.1 Basket procedure simplification

In LTE and 5G NR, CA was introduced to aggregate multiple contiguous or non-contiguous carriers in single band or in multiple bands. The original intention of CA is to enable multiple UL carriers to work simultaneously for transmission and multiple DL carriers to work simultaneously for reception, in order to achieve a higher data rate in UL and DL. With the application of CA, on one hand, the number of aggregated bands and aggregated carriers is constantly increasing; On the other hand, to meet some deployment requests, more different working modes have been introduced, especially in 5G NR, such as Tx switching and SDL switching.

In 5G NR tens of thousands of band combination has been defined. The band combination definition and related requirements for CA take up a significant portion in Spec. If no any limitation, this trend is unsustainable in the future.

In 6G, although some new technologies to aggregate multiple carriers are raised in RAN1, CA is expected to be supported. From RAN4 perspective, one of important task is how to simplify band combination.

In 5G NR, to introduce new band combinations, operators need initiate the request and then RAN4 define the requirements following the workflow in TR 38. 862 as shown in Figure 9:

Figure 9 The workflow on the introduction of band combinations for block approval

From the workflow, we can see at least two RAN4 meetings and two RAN meetings are needed from initiating request to introducing the band combination into the Spec. That is, this will take at least six months.

Observation 18: It takes at least two RAN4 meetings and two RAN meetings to complete one band combination from request to introduce into the spec.

In order to introduce band combinations more efficiently, the current request procedure can be further optimised. E.g., one special band combination request procedure can be defined

- When operator want to introduce a band combination, it can request it in RAN4 meanwhile provide TP/draft CR at the same RAN4 meeting;

- RAN4 can endorse this TP/draft CR in that meeting;

- In the following RAN plenary, this operator can bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision;

To obey RAN plenary’s procedure, some limitation is also needed. Like

- This special procedure is limited to at most N times/ M band combination number per operator

- The urgency of this band combination needs to be well justified at the very beginning of RAN4 discussion

- This justification needs to be well accepted in RAN4

- The whole workload is within the TU budget, i.e., it should not be used to overload RAN4

Proposal 14:	Consider introduce “high speed band combination request procedure”:

- Operator request one band combination and provide TR/draft CR at the same RAN4 meeting;

- RAN4 endorse this TP/draft CR in that meeting;

- In the following RAN plenary, operator bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision.

Proposal 15:	To avoid overload RAN4:

- This special procedure is limited to at most N times with M band combinations requested per operator each time;

- The urgency should be well justified at the very beginning of RAN4 discussion;

- The newly requested band combinations should not exceed the TU limits for basket WI.

2.4.2 BCS simplification

In additional, the definition of BCS for each band combination increased a lot of workload and also took up a lot of space in Spec. In NR, BCS4 and BCS5 were introduced to reduce the workload. BCS4 is to indicate UE to support up to all of the channel bandwidths for the band in the band combination. To reduce the UE capability, 6G can directly define the band in band combination can support all of the channel bandwidths for this band.

Observation 19:	BCS4 is introduced in NR in Release 17. It can be supported in 6G day one.

Proposal 16:	Consider remove BCS definition for band combination.

2.4.3 Intra-band contiguous CA simplification

In 5G, the maximum aggregated bandwidth of intra-band contiguous CA is smaller than the max bandwidth of single CC for this band. For example, the max channel bandwidth of band n1 is 50MHz, but the max aggregated bandwidth of intra-band CA_n1B is 40MHz, as shown in Table 4:

Table 4 The max aggregated bandwidth of intra-band CA_n1B

Actually, intra-band CA_n1B with 40MHz can be replaced by one single CC with 40MHz. the smaller channel bandwidth aggregation in 5G is to take into account backward compatibility due to 5G was introduced in day one is based on none stand-alone architecture.

6G will be designed based on a stand-alone architecture from day one. To solve the partial spectrum re-farming issue, 6G can work together with 5G in the re-farming band through MRSS operating.

Observation 20:	In NR, there are many intra-bands congruous CA with smaller aggregated bandwidth, which will not be a typical application.

Therefore, RAN4 should optimise the max aggregated bandwidth for intra-band contiguous CA to reduce the amount of band combination. At least, the max aggregated bandwidth of 2 carriers or intra-band contiguous CA should be large than the maximum channel bandwidth of single carrier in this band

Proposal 17:	In 6G, strive to avoid using multiple small contiguous CCs aggregation to derive the large CBW, and use one CC with large CBW as much as possible.

  - SCMC

2.4.1 NR CA implementation

Intra-band contiguous CA, intra-band non-contiguous CA, inter-band CA were introduced in LTE. In LTE, intra-band contiguous CA was assumed all CCs were received by one RF chain, and intra-band non-contiguous CA was assumed each CC was received by separate RF chain. Inter-band CA was assumed each band used separate RF chain but need share antenna.

NR did some enhancement to resolve some deployment scenarios, e.g.,

- From R16, Tx switching was introduced to make CA can support MIMO through TDD mode.

- NR R19 researched whether intra-band non-contiguous CA of two CCs can share one RF chain to improve the utilization of RF Chain.

- In 5G NR Rel-19, the SDL switching was introduced to enable low-low band CA, that the implementation of the RF front-end presented challenges, these could be overcome through the switching method.

Regardless of the RF implementation using one RF chain or multiple RF chain, under CA framework, one cell only treated one CC by configured one active BWP, as shown in Figure 10:

Figure 10 The RF implementation and active BWP configurations for CA

Observation 21: To resolve the deployment scenarios, Tx switching, SDL switching and intra-band NC CA sharing one RF chain were enhanced under NR CA framework, however, they were all configured all with one cell and one CC.

2.4.2 6G SCMC scheme and implementation

- SCMC concept

To solve the CA configuration complexity and redundant control configurations, there were proposals on Multiple carriers in single cell (SCMC) in RAN1 to enable more efficient Cell/UE management, comparing with CA, e.g., associated all CCs as one cell as show in Figure 11. It will simultaneously transmit for multiple UL CCs and/or simultaneously reception for multiple DL CCs.

Figure 11 SCMC comparing with CA

Observation 22:	SCMC concept is discussing in RAN1 mainly for the purpose of solving CA configuration complexity and redundant control configurations.

- Difference of CA and SCMC in implementation

It may extend from intra-band SCMC to inter-band SCMC to utilize the benefits it brings in 6G, as shown in Figure 12:

Figure 12 The RF implementation and possible active BWP configurations for SCMC

The difference from CA is one cell can treat multiple CCs. In addition, multiple CCs in one cell sharing one RF chain can be treated as one virtual CC by configured one BWP, if not, maybe multiple active BWP for multiple CCs are needed, it may depend on synchronization conditions between multiple RF chains. Anyway, how to handle the BWP depends RAN1’s design. From RAN4 RF perspective, the cell or the BWP cannot be seen, only the number of CC and RF chain can be identified. Compared the parts within the dashed frame of Figure 10 and Figure 12, it is clear that the RF implementations are similar between SCMC and CA, as shown in Figure 13 for inter-band as an example, it can support inter-band SCMC and inter- band CA.

Figure 13 RF implementation for inter-band

That is, the RF implementation supporting intra-band C CA, intra-band NC CA and normal inter-band CA can support intra-band C SCMC, intra-band NC SCMC and inter-band SCMC.

Observation 23: From UE perspective, SCMC and CA can share one set of RF implementation, there is no significant difference between them.

Proposal 18:	Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation.

- Mixing combination of SCMC and CA

For band combination Band A + Band B, from UE side, when a UE indicates it support inter-band SCMC Band A + Band B, it means the UE expect the Network schedule these two bands as one Cell. That is, the UE don’t expect the Network schedule these two bands as separate Cells. From Network side, when the Network schedule Band A + Band B as one cell as SCMC, it can’t schedule them as two separate cells at the same time, as shown in Figure 14:

Figure 14 Band combination Band A + Band B

Considering SCMC and CA both are aggregating carriers, there might be one situation that a band combination includes multi-carriers with SCMC and CA simultaneously as shown in figure 15. The question is whether this kind of band combination will be supported.

Figure 15 Mixing Band combination of SCMC and CA

From RF implementation perspective, the UE need support all bands at simultaneously, which caused the UE RF front-end is complex to support mixed SCMC and CA. and the simultaneously scheduling CA and SCMC is also complex.

Therefore, to simplify the band combination definition, it is proposed to not support such mixed SCMC and CA.

Observation 24: The complexity of supporting SCMC + CA simultaneously is high.

Proposal 19:	In 6G day one, mixed combination of SCMC + CA is not supported.

  - UL/DL decouple

2.5.1 Background of UL/DL decouple

In NR R19, the UL and DL is fixed for both FDD and TDD, the impact of UL to DL is guaranteed though the Tx-Rx isolation of duplexer.

For UL/DL decouple, the potential problem is that the harmonic of Band A UL may fall on Band B DL, this will cause a larger REFSENs degradation. To solve these issues, operators have proposed the concept of resource pool among different bands/CCs, NW configure one UL CC with the most suitable DL CC among these CCs, e.g., Band A UL +Band B DL.

In addition, the purpose of UL/DL decouple is to enhance coverage in the cell edge by flexible association of DL carrier and UL carrier, e.g., Low Band UL+ mid-High Band DL, or mid-High Band UL+ High Band DL.

Observation 25:	UL/DL decouple is discussed in RAN1 for the purpose of cell-edge performance and UL coverage.

2.5.2 Implementation of UL/DL decouple

RAN4 need discuss how to define the recouple pair of Band A UL and Band B DL and how to implement it.

If band A and Band B adopt separate antenna in UE RF implementation, UL/DL decouple have not any impact on UE RF implementation.

Observation 26: UL/DL decouple have not any impact on UE RF implementation when band A and Band B adopt separate antenna in UE RF implementation.

Here only analysis the scenario that band A and Band B sharing one antenna. From our side, there are some options for UL/DL decouple sharing one antenna.

- Option 1: Defining a new band number for the recouple pair of Band A UL and Band B DL, then it can work as a normal FDD band, the UE only needs to support this new band, don’t need to support band A and band B, as shown in Figure 16.

Figure 16 Defined a new band for recouple pair of Band A UL and Band B DL

From spectrum perspective, this new recouple band comes from Band A and Band B, if the operator has this new recouple band, it must have Band A and Band B, why the operator only deploys this new recouple band. From UE perspective, UE will support one operator’s all bands, so if the UE support this new recouple band, it must support Band A and Band B. it seems defining one new band number for this new recouple band is unnecessary and increase the complexity of UE implementation due to add new RF front-end component. In addition, introduce more new bands will cause more band combination and bring more workload.

Observation 27: New RF front-end component will increase the complexity of UE implementation and new band number cause more band combinations and bring more workload.

Proposal 20:	Avoid to define a new band for UL/DL decouple pair.

- Option 2: Enable it work as one of configuration for inter-band CA Band A+ Band B, the UE needs to support CA between Band A and Band B as shown in Figure 17:

Figure 17 Recouple pair of Band A UL and Band B DL as one of CA configuration

Observation 28: A UE supports inter-band CA (Band A + Band B), will support the recouple pair of Band A UL and Band B DL.

Proposal 21:	To enable more efficient Cell/UE management, study UL/DL decouple based on non-CA architecture.

- Option 3: Association of Band A UL and Band B DL as a cell

From UE side, the recouple pair of Band A UL and Band B DL is also one combination, in principle, the UE will need support normal band A and Band B separately, then can support the recouple pair of Band A UL and Band B DL as one band combination.

If the cell only includes Band A UL and Band B DL without band A DL and band B UL and the UE don’t need support CA between A and Band B, then Band A UL and Band B DL can share one set of RF chain band BB with Band A and band B, although new RF front-end component need introduce. The possible implementation as shown in Figure 18:

Figure 18 Association of Band A UL and Band B DL as a cell

Observation 29: New RF front-end component will increase the complexity of UE implementation although can save one set of RF chain band BB.

Observation 30: if both bands are within same band group (e.g., both are low bands), then for shared antenna UEs, this will need a new quadplexes which will increase much complexity in implementation.

Proposal 22:	The decouple/recouple band combination shall not be within the same band group (e.g., both are low bands). And the UL/DL frequency separation, band A Tx to band B Rx interference also need to be considered in RAN4.

If the cell not only includes Band A UL and Band B DL but also include band A DL and band B UL, the network only configures one UL CC and one DL CC for the UE at one time, if dynamic scheduling is allowed will further increase the complexity of UE implementation, as shown in Figure 19:

Figure 19 Switching among Band A UL + Band B DL, Band A and Band B

Observation 31: Ddynamic scheduling between Band A UL+ Band B DL and Band A/Band B will increase the flexibility of scheduling.

Proposal 23:	The decouple/recouple band combination shall be based semi-static approach if introduced due to complexities.

Proposal 24:	The decouple/recouple band combination shall be UE capability based, it cannot be for any band combination.

  - Carrier-switching

Low band FDD-SDL carrier switching has been introduced in Rel-19, and FDD-FDD switching was also proposed by some companies, due to the implementation of the RF front-end presented challenges.

This kind of switching has some benefits from NW side in terms of scheduling flexibility and load balance. From single UE side, no obvious throughput gain but maybe useful when the cell is too crowded and UE can move to another cell.

Therefore, carrier switching can be further simplified from RF band baseband, because of switching bands don’t need to work simultaneously. Such as with single carrier processing capability (both RF and baseband) and function shared by switched carriers and the switching bands also can be associated as one cell to utilize the benefits it brings in 6G, as shown in Figure 22.

Figure 20 Carrier switching within inter-band

Observation 32:	Carrier switching is under discussion in RAN1 which is targeted to simplify from RF and baseband especially for the band combinations which cannot supported simultaneously like low band FDD + FDD.

From RAN4 perspective, carrier switching is similar as SDL switching that also can enable low-low band combination, that the implementation of the RF front-end presented challenges. The difference is the bands within carrier switching are considered as one cell but the bands within SDL switching are considered as multiple cells.

Proposal 25:	Study the RF impacts for carrier/band switching in terms of considering the switched carriers as one cell or two cells.

  - The relationship among the aggregated technologies

Based on above analysis from RF implementation and utilization of the benefits of 6G, SCMC, UL/DL decouple and carrier switching can be considered as one cell, Figure 21 shows the similarities and differences:

Figure 21 The similarities and differences among different aggregated technologies

The purpose of all these new candidates aggregated technologies have some similar with enhanced CA in NR, consider enabling more efficient Cell/UE management, RAN4 and RAN1 need further down select from CA and these new candidates aggregated technologies.

Observation 33: All these new candidates aggregated technologies have similar purpose with enhanced CA in NR, except the different of Cell/UE management.

Proposal 26:	For one band combination, which aggregated technologies will be adopt need further down selection, and UE implementation complexity need to be well considered.

- Conclusion

This paper discussed the related topics of spectrum and proposed:

Frequency ranges

Observation 1:	Millimeter waves categorized to FR2 is mainly due to adopt array antenna and lack a conductive test port.

Observation 2:	RAN1 is discussing frequency range based on numerology, which is different from RAN4 where the conducted/OTA requirement definition is the focus. RAN4 need to be deeply involved in the frequency range definition.

Proposal 1:	The frequency range definition shall consider the conducted/radiated requirement definition in RAN4 and also the UE implementation impacts.

Observation 3:	In NR, Ku band has set an example of defining two bands within FR1 and FR2 for the same spectrum.

Observation 4:	RAN4 introduced two band for same Ku spectrum, one is FR1 band, the other is FR2 band. The two bands apply same radiated requirements.

Proposal 2:	Multiple numerology should be avoided in the same frequency (sub)range in 6G.

Observation 5:	Multiple numerology shall be avoided for the same Band / sub-range.

Proposal 3:	Frequency range can be defined based on the interface of UE RF requirements. Within each frequency range, the applicable numerology can be defined per band or per sub-frequency range basis.

Observation 6:	The frequency range for TN and NTN became different from R19.

Proposal 4:	In 6G, align the frequency range definition for TN and NTN.

Observation 7:	For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted isotropic radiation pattern antenna interface.

Observation 8:	The array antenna of 15GHz is too large for physical constraints in case of handheld devices.

Proposal 5:	It is feasible to apply FR1 up to 15300MHz.

Observation 9:	10700MHz-24250MHz is only used by NTN.

Proposal 6:	In 6G, define frequency ranges based on the interface of RF requirements, as show in Table 3.

Table 3: The uniform division of frequency range for 6G TN and NTN

| Frequency range | Spectrum range | Interface of RF requirements |
| --- | --- | --- |
| FR 1 | 410 MHz-15350 MHz | Conducted interface |
| FR 2 | 10700MHz-24250 MHz (NTN only) | Radiated interface |
|  | 24250 MHz-52600 MHz |  |

New spectrum for 6G

Observation 10: 6425-7125 MHz is totally overlapping with NR band n104.

Observation 11: 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

Observation 12: Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

Observation 13: New spectrum range of 4400-4800 MHz has been covered by NR band n79, furthermore, band n79 will also be considered for re-farming into 6G.

Proposal 7:	RAN4 should first study whether the n79 can be re-employed for 6G to cover the new 6G spectrum 4400-4800MHz.

Observation 14: For the candidate spectrum 7125-8400 MHz, NS value could be used to solve specific requirements.

Proposal 8:	RAN4 should strive to define global bands for 7125-8400 MHz and 14.8-15.35 GHz for 6G.

Re-farming bands

Observation 15: 6G will coexist with 5G NR for a long time, and operators also need time to re-farm the 5G spectrum.

Proposal 9:	FR1 re-farming bands should be introduced step by step based on Operators’ request.

Proposal 10:	For FR1, reuse 5G requirements for the same band with same PC, e.g., reuse same hardware for both 5G and 6G.

Observation 16: With 6G limited TU, it is important for RAN4 to identify the most important aspects to focus on.

Proposal 11:	The re-farming of FR2 bands should depend on the level of interest from industry.

Proposal 12:	Reuse 5G FR2 UE type/power class and Requirement directly.

Re-farming band simplification

Observation 17: In 5G NR, many overlapping bands have adopted same RF front-end in real UE implementation which makes defining separate bands is not meaningful.

Proposal 13:	Consider to merge overlapping bands as much as possible when re-farming FR1 bands.

Basket procedure simplification

Observation 18: It takes at least two RAN4 meetings and two RAN meetings to complete one band combination from request to introduce into the spec.

Proposal 14:		Consider introduce “high speed band combination request procedure”:

- Operator request one band combination and provide TR/draft CR at the same RAN4 meeting;

- RAN4 endorse this TP/draft CR in that meeting;

- In the following RAN plenary, operator bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision.

Proposal 15:	To avoid overload RAN4:

- This special procedure is limited to at most N times with M band combinations requested per operator each time;

- The urgency should be well justified at the very beginning of RAN4 discussion;

- The newly requested band combinations should not exceed the TU limits for basket WI.

BCS simplification

Observation 19:	BCS4 is introduced in NR in Release 17. It can be supported in 6G day one.

Proposal 16:	Consider remove BCS definition for band combination.

Intra-band contiguous CA simplification

Observation 20:	In NR, there are many intra-bands congruous CA with smaller aggregated bandwidth, which will not be a typical application.

Proposal 17:	In 6G, strive to avoid using multiple small contiguous CCs aggregation to derive the large CBW, and use one CC with large CBW as much as possible.

SCMC

Observation 21: To resolve the deployment scenarios, Tx switching, SDL switching and intra-band NC CA sharing one RF chain were enhanced under NR CA framework, however, they were all configured all with one cell and one CC.

Observation 22:	SCMC concept is discussing in RAN1 mainly for the purpose of solving CA configuration complexity and redundant control configurations.

Observation 23: From UE perspective, SCMC and CA can share one set of RF implementation, there is no significant difference between them.

Proposal 18:	Study whether it is possible to define only one set of requirements for CA and SCMC when introduced in RAN1 considering CA and SCMC can share the same RF implementation.

Observation 24: The complexity of supporting SCMC + CA simultaneously is high.

Proposal 19:	In 6G day one, mixed combination of SCMC + CA is not supported.

UL/DL decouple

Observation 25:	UL/DL decouple is discussed in RAN1 for the purpose of cell-edge performance and UL coverage.

Observation 26: UL/DL decouple have not any impact on UE RF implementation when band A and Band B adopt separate antenna in UE RF implementation.

Observation 27: New RF front-end component will increase the complexity of UE implementation and new band number cause more band combinations and bring more workload.

Proposal 20:	Avoid to define a new band for UL/DL decouple pair.

Observation 28: A UE supports inter-band CA (Band A + Band B), will support the recouple pair of Band A UL and Band B DL.

Proposal 21:	To enable more efficient Cell/UE management, study UL/DL decouple based on non-CA architecture.

Observation 29: New RF front-end component will increase the complexity of UE implementation although can save one set of RF chain band BB.

Observation 30: if both bands are within same band group (e.g., both are low bands), then for shared antenna UEs, this will need a new quadplexes which will increase much complexity in implementation.

Proposal 22:	The decouple/recouple band combination shall not be within the same band group (e.g., both are low bands). And the UL/DL frequency separation, band A Tx to band B Rx interference also need to be considered in RAN4.

Observation 31: Ddynamic scheduling between Band A UL+ Band B DL and Band A/Band B will increase the flexibility of scheduling.

Proposal 23:	 The decouple/recouple band combination shall be based semi-static approach if introduced due to complexities.

Proposal 24:	 The decouple/recouple band combination shall be UE capability based, it cannot be for any band combination.

Carrier-switching

Observation 32:	Carrier switching is under discussion in RAN1 which is targeted to simplify from RF and baseband especially for the band combinations which cannot supported simultaneously like low band FDD + FDD.

Proposal 25:	Study the RF impacts for carrier/band switching in terms of considering the switched carriers as one cell or two cells.

The relationship among the aggregated technologies

Observation 33: All these new candidates aggregated technologies have similar purpose with enhanced CA in NR, except the different of Cell/UE management.

Proposal 26:	For one band combination, which aggregated technologies will be adopt need further down selection, and UE implementation complexity need to be well considered.

- Reference

[1] RP-251881 New SID: Study on 6G Radio, NTT DOCOMO	RAN#108

[2] R4-2511652 RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4 #116

[3] RP-252963 Summary for “6G - system design aspects (numerology, spectrum)”, Moderator (RAN1 Chair, CMCC), RAN #109