3GPP TSG RAN WG4 Meeting#116bis                                R4-2513329

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	8.11

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6G ISAC study

Document for:	Approval

Introduction

In RAN#108 meeting, a new SID on Study on 6G Radio [1] was approved. In this contribution, we would like to share some initial understandings on 6GR ISAC study.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

Discussion

## 2.1. Timeline for 6G ISAC study

In the following figure 2.2-1, we provide the timeline for ISAC study from 3GPP SA to RAN1/RAN4 in both 5G-A and 6G.

Figure 2.2-1. The timeline for 5G/6G ISAC study in 3GPP

Option 1: align with RAN1 6G ISAC study timeline, there will be less than 5 Quarter left for RAN4 discussions;

Option 2: start the ISAC from Q4,2025,there will be 7 quarters left for RAN4 discussions;

Figure 2.2-2. Two alternatives of study timeline for 6G RAN4 ISAC

Proposal 1: Propose to discuss the timeline between option 1 and option 2 for 6G ISAC RAN4 study considering the overall workload in SI phase.

## 2.2. Motivation and use case of ISAC

### 2.2.1. General

In this section, we provide some ISAC use cases which are very promising to commercialize. Most of them have been well tested in the field trial.

Specifically, the first four ISAC use cases have been included in SA1 5G-A requirements [2]. Also, the Rel-19 ISAC channel model has supported these four use cases. The last three use cases are also promising, and very useful especially for public safety and these new use cases are supported by a number of companies including many operators as shown in our companion contribution [18]. Some commercialized or field trial information are also listed in the corresponding subsections.

Proposal 2: For 6G ISAC, the applicable use cases should at least include

- Detection and tracking of UAV, Pedestrian, Vehicle, AGV, and Ship

- Infrastructure collapse monitoring

- Micro-deformation detection

### 2.2.2. UAV detection and tracking

Low altitude economy is a comprehensive economic form based on manned and unmanned aerial vehicles. It has the characteristics of a long industrial chain, wide radiation, strong growth and driving force, etc. The development of the low-altitude economy helps to expand the market; it contributes to promoting innovation and enriching supply; it has the capacity to break down barriers and promote integration.

UAV is one of the main drivers of low altitude economy. The integration of UAVs with the low-altitude economy is ushering in a new chapter for future airspace, driving transformations across multiple sectors including transportation, logistics, agriculture, and emergency rescue.

There are numerous applications for UAVs in low altitude economy, including but not limited to power line inspections, UAV delivery, intrusion detection, agricultural crop protection, and more. Some use cases are as shown in Figure 2-1.

- Power line inspection         (b) UAV delivery               (c) UAV intrusion detection

Figure 2-1 Use cases for UAV sensing

The flight safety of UAVs and their regulation thus becomes a pressing issue that needs to be addressed. Solving flight safety relying on the UAV itself has issues such as limited observation range and blind spots. Therefore, networks providing sensing is becoming a trend. In short, sensing will play a crucial role in these applications, and sensing can be used to monitor illegal intrusions by low, slow, and small UAVs. Sensing can also be used for trajectory tracking and for collision avoidance.

In [3], some field test results for UAV intrusion detection and tracking were provided. It shows good sensing accuracy with implementation based on base station sensing.

### 2.2.3. Pedestrian detection and tracking

According to data from the World Health Organization, road traffic accidents cause nearly 1.3 million deaths globally each year, with pedestrians accounting for approximately one quarter of all global road traffic fatalities annually. Ensuring pedestrian safety becomes an urgent issue that needs to be addressed.

With the trend of digitization and intelligence in the transportation field, 6G system is expected to provide real-time data communication for massive traffic terminals, and at the same time efficiently sense the real-time status of roads, vehicles, and people to enable the transportation system to have the capabilities of sensing, interconnection, analysis, prediction and control in a larger space-time range.

Hence, pedestrian detection and tracking becomes one of the critical scenarios in smart transportation. Use cases for pedestrian detection include but are not limited to railway pedestrian monitoring, highway pedestrian monitoring, and pedestrian monitoring in urban district roads.

- Highway intrusion detection         (b) Railway intrusion detection     (c) Pedestrian monitoring in urban

Figure 2-2 Use cases for pedestrian detection

### 2.2.3. Vehicle detection and tracking

The global automotive industry size is calculated at USD 4,359.98 billion in 2024 and is expected to reach around USD 6,678.28 billion by 2032[3]. And the global intelligent driving solutions market was worth around USD 29 billion in 2023 and is predicted to grow to around USD 78.44 billion by 2033[4].

Currently, technologies such as in-vehicle radars and lidars are thriving, but these technologies do have limitations.

- Blind Spots in Coverage: The coverage area of single-vehicle sensing is fan-shaped, leading to blind spot regions.

- Large Momentary Errors: Due to high speeds, single vehicles only have access to instantaneous sensing information, resulting in a high probability of misidentification or missed detection of targets.

- Significant Impact of Obstructions: When obstructed by preceding vehicles, it's unable to detect obscured areas.

- Limited Sensing Distance: The effective range of sensing is restricted.

- Inability to Sense All-Weather: Perception range is drastically reduced under adverse weather conditions, and sensing efficacy is also limited without proper lighting.

Therefore, the sensing capabilities provided by cellular systems will be a beneficial complement. The vehicle monitoring and tracking provided by cellular networks not only aids in traffic management but also supports autonomous driving illustrated in the figures below.

- Vehicle information statistics                      (b) Automotive maneuvering and navigation

Figure 2-3 Use cases for vehicle detection and tracking

### 2.2.4. AGV detection and tracking

Based on market forecasts, the global industrial automation and control systems market size was estimated at USD 206.33 billion in 2024. The market is growing rapidly, driven by the rising demand for efficiency, precision, and safety across manufacturing industries[5].

Automated Guided Vehicles (AGVs) are key components of the new smart factories, used for a variety of tasks such as heavy or hazardous materials transportation and distribution. Based on market forecasts, the global automated guided vehicle market size was valued at USD 2.51 billion in 2024. The market is projected to grow from USD 2.75 billion in 2025 to USD 5.44 billion by 2032[6].

Simultaneous presence of AGVs and human workers at the industrial side creates safety challenges and calls for stringent safety requirements [7]. For example, the driverless, automated guided industrial vehicles ANSI/ITSDF B56.5 [8] safety standard requires that “the AGV shall detect and avoid both static and dynamic obstacles appearing in the path of travel direction”. Reliable detection of AGV/human presence or proximity is therefore an important safety criterion.

6G system can be deployed in a factory which uses RAN entities and/or UEs to measure sensing signal and achieve sensing data, that are made available to sensing management entities in order to derive sensing results such as the detection of the presence or proximity of AGVs and humans and the surrounding environment, as shown in Figure below.

- AGV detection and tracking in factories           (b) AMR collision avoidance in smart factories

Figure 2-4 Use cases for AGV detection and tracking

### 2.2.5.Ship detection and tracking

Maritime and inland waterway transportation are the primary means of cargo transport worldwide. Over two-thirds of global international trade volume and approximately 90% of import and export freight are transported by sea and river routes. Ship detection plays an important role in port management, ship traffic, maritime rescue, cargo transportation and so on. Usually, ship detection in nearshore waters and rivers is a crucial scenario. Since base stations are primarily land-based, large sensing coverage is required to ensure effective ship detecting at nearshore areas.

Figure 2-5 Ship detection in ocean

By allowing 6G network to detect and track vessels in real time, ISAC can dramatically improve maritime safety, efficiency, and environmental monitoring. For example, ISAC-enabled networks can support collision avoidance, search and rescue, or the detection of unauthorized vessels even in challenging conditions such as fog or rough seas, the corresponding capabilities are highlighted in the 3GPP TR 22.837 [10].

From a commercial perspective, the prospects are highly promising. The maritime industry is undergoing rapid digital transformation, driven by the need for better security, smarter port operations, and stricter environmental regulations. Market research predicts that the global maritime surveillance sector will grow to over $35 billion by 2029, fueled in part by demand for real-time, network-based sensing and tracking [11] . ISAC offers a cost-effective solution by combining communication and sensing infrastructure, enabling operators and authorities to extend surveillance coverage without the high costs of traditional radar or satellite systems.

Moreover, as noted in [12], ISAC’s ability to leverage existing and future mobile networks makes it particularly attractive for ports and coastal regions. This positions ISAC as a key enabler for smarter, safer, and more sustainable maritime operations, with clear and growing business opportunities for network operators, equipment vendors, and maritime stakeholders alike.

Table 2-1 Estimation of market of ISAC enabled ship detection and maritime surveillance

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Maritime Surveillance | USD 35.7 billion (market size) | By 2029 [11] |
| Smart Ports & Shipping | USD 9.8 billion (market size) | By 2027 [12] |

Some field tests for ship intrusion detection and tracking have been done with operators [17]. The good sensing accuracy with implementation based on base station sensing was observed.

### 2.2.6.Infrastructure collapse monitoring

ISAC has the potential to revolutionize how we respond to sudden infrastructure collapse such as subsidence and landslides, especially in critical environments like highways, farmlands, and so on. In these scenarios, the rapid detection and precise localization of a disaster event are crucial for saving lives and minimizing property loss. Traditional monitoring systems, such as cameras or standalone sensors, often suffer from coverage gaps and delayed reporting. In contrast, ISAC-enabled base stations can simultaneously monitor ground movement while providing real-time communication services.

Figure 2-6 An illustration for infrastructure collapse

For example, if a landslide or sudden road collapse occurs on a highway, nearby ISAC-capable base stations can immediately sense the structural anomaly using their radio signals - such as by detecting abnormal changes in signal reflection or propagation. This information can then be instantly communicated to vehicles and personnel in the vicinity, triggering automated warnings and rerouting instructions. Such rapid response is critical, as drivers may not have enough time to react if they only notice a hazard visually, especially at high speeds. In rural or agricultural zones, ISAC can also help protect equipment and workers from unexpected ground failures.

Figure 2-7 Infrastructure collapse monitoring

The commercial outlook for this use case is highly positive. With growing awareness of climate change and the increasing frequency of extreme weather events, there is a rising demand for intelligent, network-based disaster monitoring systems. Industry analyses predict robust growth in the smart disaster management market, with ISAC technologies expected to play a key role in enhancing early warning capabilities and reducing the costs associated with manual monitoring and dedicated sensor infrastructure. According to a MarketsandMarkets report, the global disaster management market is projected to reach USD 222.2 billion by 2027 [13], with a significant portion driven by rapid sensing and alerting solutions.

Table 2-2 Estimation of market of infrastructure collapse monitoring

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Disaster Management | USD 222.2 billion (market size) | By 2027 [13] |
| Smart Sensing for Geological Hazards | Multi-billion USD (market size) | By early 2030s [12] |

### 2.2.7.Micro-deformation detection

As urbanization accelerates and infrastructure ages, monitoring the subtle movements of critical structures, such as bridges, flyovers, and skyscrapers has become increasingly important for public safety and asset management. These structures are constantly exposed to environmental factors like temperature changes, wind, rain, and even minor seismic activity, all of which can cause micro-deformations or displacements at the millimeter or centimeter scale. Although such movements are small, they can indicate early signs of fatigue, stress, or potential failure, making their detection crucial for preventive maintenance and extending infrastructure lifespan.

ISAC deployed over wide-area networks like 6G, offers a transformative solution. By leveraging the high-resolution sensing capabilities of 6G base stations, it becomes possible to detect and track these micro-deformations in real time across large and complex urban environments. ISAC-enabled base stations can monitor changes in the reflection and propagation of their radio signals caused by even slight shifts in structural position.

A typical implementation might see 6G base stations installed around or within major bridges and high-rise buildings, continuously scanning for abnormal movements. When micro-deformations are detected, maintenance teams can be promptly alerted, enabling targeted inspections and repairs before problems escalate. This approach is particularly valuable for megacities with extensive infrastructure networks, where manual monitoring is costly and inefficient. According to a [14], the integration of ISAC for structural health monitoring is regarded as a game-changer for smart city management, offering both operational and financial benefits.

Figure 2-8  Micro-deformation monitoring on a bridge or a construction

The commercial outlook for ISAC-driven micro-deformation detection is robust. The global market for structural health monitoring (SHM) systems is expected to reach over USD 4.1 billion by 2029, driven by increasing demand for real-time, automated monitoring solutions in both developed and emerging economies. The unique advantage of ISAC lies in its ability to combine communication and sensing on the same infrastructure, reducing deployment costs while expanding coverage. Industry studies and 3GPP discussions identify this use case as a key vertical for next-generation networks, particularly as cities and private operators move toward smarter, more proactive infrastructure management.

Table 2-3 Estimation of market of micro-deformation monitoring

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Structural Health Monitoring (SHM) | Was USD 2.5 billion | In 2024 [15] |
|  | USD 4.1 billion | By 2029 [15] |

## 2.3. Regulatory requirement for ISAC study

Similar to SBFD operation, for the new technology introduced into 3GPP, some survey on the regulatory requirements for such kind of service could be collected in RAN4 to provide guidance on the system design. If ISAC operation will also introduce BS2BS CLI or UE2UE CLI problem according to the proposed operation mechanism. Then the relevant SBFD regulatory information in TR38.858 could be used as starting input for 6G ISAC input.

Proposal 3: Collect the regulatory requirements for sensing if available from different regions.

## 2.4.Basic operation mechanism for ISAC deployment

### 2.4.1. Basic algorithm: distance estimation

Here we use UAV sensing detection as an example in Figure 2.4.1-1 to clarify the distance estimation theory from our understanding. According to the channel estimation based on reference signals of ISAC operation designed in RAN1, the timing delay profile for distance estimation can be obtained by transforming the estimation results in frequency domain to timing domain by IFFT operation to get channel impulse response CIR (similar to channel truncation in channel estimation function.) .

Figure 2.4.1-1. Illustration of basic principle of distance estimation for ISAC operation

If the transmission bandwidth is assumed as, the finest granularityin time domain can be expressed as follows:

As shown in the figure 2.4.1-1, the distance estimation for targeted UAV can be calculated as follows:

If we want to make sure that two targeted devices could be distinguished in the distance domain, then the the timing difference from the receiver side should be at least equal to or larger than the finest timing granularity as follows.

According to the above derivation, then the ideal finest distance estimation granularity can be calculated as follows:

For example, the ideal finest distance estimation granularity according to different sensing BW is calculated as shown in the following Table 2.4.1-1 :

Table 2.4.1-1: the ideal finest distance estimation granularity according to different sensing BW

| BW [MHz] | Ideal finest distance estimation granularity [m] | Remark |
| --- | --- | --- |
| 50MHz | 3.0 | Might be worse than Horizontal localization accuracy: 2m for RAN SI for IMT-2030 |
| 100MHz | 1.5 | FR1 high bands |
| 200MHz | 0.75 | FR1 high bands |
| 400MHz | 0.375 | FR1 high bands and FR2-1 |
| 800MHz | 0.1875 | FR1 high bands and FR2-1 |

Observation 1: In order to meet the RAN Sensing KPI for IMT-2030, the measurement bandwidth for sensing signal should be at least larger than 100MHz;

Observation 2: In order to meet more than 100MHz sensing signal bandwidth, FR1 high TDD bands and FR2-1 bands are more suitable for ISAC deployment;

Proposal 4: For 6G ISAC study, consider FR1 TDD with available spectrum wider than 100MHz at least, FR2-1 bands and FFS for FR3 bands;

In signal processing theory, then the achievable timing estimation performance according to Cramér-Rao Lower Bound, CRLB theory can be calculated as follows:

Then, the corresponding achievable distance estimation performance can be calculated as follows:

### 2.4.2. Basic algorithm: Angle estimation

Here we use UAV sensing detection as an example in Figure 2.4.1-2 to clarify the azimuth/zenith angle estimation theory from our understanding. The angle arrival direction could be estimated by the phase difference among different sub-arrays or antenna elements of each digital chain from frequency domain similar to other legacy timing estimation method.

Figure 2.4.1-2.Illustration of basic principle of azimuth and Zenith angle estimation for ISAC operation

According to the antenna theory, the finest angle estimation granularity could be calculated by basic 3dB beamdwidth as follows:

In signal processing theory, then the achievable angle estimation performance according to Cramér-Rao Lower Bound can be calculated as follows:

### 2.4.3. Basic algorithm: velocity estimation

Here we use UAV sensing detection as an example in Figure 2.4.1-3 to clarify the velocity estimation theory from our understanding. According to the channel estimation results and its conjugate of reference signal of ISAC operation designed in RAN1, then we could get the following conjugate estimation results:

Figure 2.4.3-1. Illustration of basic principle of velocity estimation for ISAC operation

In addition, considering estimated doppler shift reflected in the phase domain and also the accurate phase detection threshold without phase estimation ambiguity, then we could get the velocity estimation results as follows:

In signal processing theory, then the achievable velocity estimation performance according to Cramér-Rao Lower Bound, CRLB theory can be calculated as follows:

### 2.4.4. Basic mode for ISAC operation

For the UAV use case as shown in the following figures, there are two potential modes including the mono-static and bi-static mode.

As shown in Figure 2.4.4-1, for mono-static deployment, ISAC reference signal is transmitted and received by the same BS which is also the basic assumption for 5G-A assumption (e.g. without special optimization for network signalling for bi-static use case).

Figure 2.4.4-1. Mono-static mode for ISAC operation

As shown in Figure 2.4.4-2, for bi-static deployment, ISAC reference signal is transmitted and received by different BSs. For this use case, the timing requirement across different BSs will have the direct impacts on the measurement performance.

Figure 2.4.4-2. Bi-static mode for ISAC operation [existing timing accuracy requirement as 3us]

Proposal 5: For the UAV use case of 6G ISAC study, consider the mono-static deployment as 1st priority and bi-static deployment as 2nd priority;

Proposal 6: For the UAV use case of 6G ISAC study, discuss the achievable timing accuracy performance among different BSs for bi-static deployment.

### 2.4.5. Basic mechanism for ISAC operation

For the mono-static deployment, as shown in Figure 2.4.5-1/2, there are two potential basic mechanism for ISAC operation from our understanding.

Option 1: TDM&Full array&High Tx power.

Option 2: Full duplex&half array&Low Tx power

For the option 1, due to non-simultaneous transmission and reception in TDM, the corresponding coverage is expected to be larger. The drawback for the option 1 is the blind detection area due to the transmission period and switching period.

Figure 2.4.5-1. Illustration of Pulse signal: TDM&Full array&High Tx power.

For the option 2, due to simultaneous transmission and reception, the areas close to the ISAC BS can be also detected without any blind detection regions compared with option 1. However self interference and BS2BS co-channel interference will also result in lower power transmission and smaller coverage compared with option 1.

Figure 2.4.5-2. Illustration of Pulse signal:Full duplex&half array&Low Tx power [similar to Rel-19 SBFD digital IC calibration process]

Figure 2.4.5-3. Comparison between pulse signal and continuous-wave signal [3]

Proposal 7: For the 6G ISAC study, consider TDM based sensing operation and full duplex based sensing operation.

### 2.4.6. Basic physical layer processing for UAC use case of ISAC operation

As shown in Figure 2.4.6-1, the sensing algorithm used in the evaluation includes the following steps:

- Frequency-domain Channel Estimation

Use the received sensing signal to estimate frequency-domain channel.

- Range-doppler (RD) Map Construction

After obtaining multiple channel estimations in different time, apply an IFFT to the channel coefficients of the sub-carriers and an FFT to different channel estimations, and the range-Doppler (RD) map is obtained.

- CFAR Detection

CFAR is used to detect the target in the RD map, and the range index and Doppler index of each target can be obtained. Note that a coarse clustering is used to combine the peaks whose the range index and Doppler index are both close.

- Angle Estimation

For each detected target, the Azimuth-of-Arrival (AoA) and Zenith-of-Arrival (ZoA) are further estimated.

- Position and velocity determination

Determine the coordinate (x, y, z) and the radical velocity according to CFAR detection results and angle estimations.

- Multi-node Collaboration

For each target, combine the points (x, y, z) provided by different TX-RX pairs into a single point, and combine the radical velocities into a velocity vector only when the number of TX-RX pairs is not less than three.

Figure 2.4.6-1. Algorithm procedure to process the received sensing signal

- Measurement report

In the whole pipeline to generate sensing results with different levels, we summarize all procedures as four main steps, which is related to four kinds of measurement data, from specific to abstract. The whole pipeline is shown as follows:

Figure 2.4.6-2. Pipeline to derive measurement data with different levels

Once receiving the echo signal from the Rx side, the processor at Rx could record the whole echo signal from multiple channels. The raw echo signal would be too dense to be transferred by any network entity. The general echo signal recorded at Rx could be visualized as follows:

Figure 2.4.6-3 Echo signal from multiple channels

Step 1: Channel estimation (Channel Impulse Response (CIR) I/Q sample)

Then the Rx would conduct the channel estimation to derive the channel impulse response (CIR) I/Q sample, based on the recorded echo signal and the known reference signal. The peak in CIR I/Q sample profile would be used to indicate the delay/distance information of each multi-path, which is potentially related to sensing results.

Figure 2.4.6-4 CIR I/Q profiles from multiple channels

The raw data size of CIR I/Q samples would also be very large. As an example, we have following configurations for sensing RS:

- Number of symbols used for sensing:

- Sub-carrier space:

- Bandwidth:

- Number of TRPs:

- Number of channels:

- Number of trajectory points:

If the single precision floating point number is used to describe the CIR I/Q samples, the data size would be about 480 Gbits, based on the following calculation:

If the double precision floating point number is used to describe the CIR I/Q samples, the data size would be about 960 Gbits, based on the following calculation:

It could be seen that the data size of CIR I/Q samples leads to the situation that the delivery of such amount of data would cost too much resources for communication and calculation for sensing operation.

Step 2: Accumulating and FFT transforming (RD profile)

Given the CIR I/Q samples with multiple channels, the processor could accumulate the CIR profile on Doppler dimension and implement FFT operation on Doppler dimension to derive the range-Doppler (RD) profile. The accumulation and FFT operation could also be implemented on both Doppler dimension and spatial dimension, to derive the range-Doppler-angle (RDA) profile. The step1 only digs out the sparsity on time domain, and the step2 could derive the sparsity in Doppler domain and spatial domain. Then any peak in such RD profile (or RDA profile) would indicate the delay and velocity (and angle) of a potential sensing target.

The general RD profile could be visualized as following figure:

Figure 2.4.6-4. RD profile of one channel

The RD profiles would have comprehensive information about sensing target, including the position, velocity, RSRP, micro-Doppler features, etc., which is the most useful and important tool to analyse the status of sensing target. However, if the data size of RD profile would be same as the CIR I/Q profile, with the same configuration as step1, the data size of RD profile stored as single precision floating point number would be 480 Gbits, and the data size of RD profile stored as double precision floating point number would be 960 Gbits.

Step 3: Target detection (Point cloud)

It could be seen that most of points in the RD profile carries with very limited power, and only small number of points in the point cloud would indicate the sensing target. Thus the detection algorithm could be implemented on the RD profile, such as spectrum peak searching, or constant false alarm rate searching (CFAR), to obtain the point cloud which could consist of the TOA, AOA, ZOA, Doppler and other information of potential sensing target. One set of detected points cloud could be visualized in the following figure:

Figure 2.4.6-5. A cluster of points cloud

The data size of points cloud could be simplified a lot. For example, if each point in points cloud would consist of TOA, AOA, ZOA, and Doppler frequency, which have data dimension of , then the data size would be decided by the total number of detected points in points cloud, as shown in following calculation.

When 1000 points are detected:

- If single precision floating point number is used to described the point cloud, the data size of point cloud would be 14.6 Mbits

- If double precision floating point number is used to described the point cloud, the data size of point cloud would be 29.3 Mbits

When 10000 points are detected:

- If single precision floating point number is used to described the point cloud, the data size of point cloud would be 146.5 Mbits

- If double precision floating point number is used to described the point cloud, the data size of point cloud would be 293.0 Mbits

Step 4: Tracking and Deduplication (X-Y-Z coordinates)

Each point in points cloud could be seen as single measurement at one time, which could have limited performance and duplicating information. For example, multiple points could be used to describe one same sensing target because of the high resolution of sensing system, and then one set of points cloud could be clustered as one single trajectory. What’s more, sensing results from different Rxs could be merged together. All the deduplication operations could decrease the data size of measurement report a lot.

Figure 2.4.6-6 A list of trajectory point

We suppose that each trajectory point would have four dimensions of statement: X-Y-Z coordinates, and velocity. If the single precision floating point number is used, the data size of the trajectory point would be . If the double precision floating point number is used, the data size of the trajectory point would be .

Actually both the sensing results from step3 and step4 could be acceptable considering the data size. The sensing results from step3 or step4 could also be merged together with the following format:

- 3D coordinates (X-Y-Z location)

- 3D velocity (X-Y-Z speed)

- Orientation angles

- RSRP/RSRPP

- TimeStamp

- Uncertainty

- ......

Both points derived from step 3 and step 4 could include part or all of the above information elements. The difference between the measurement report from step 3 and step 4 would only be the number of points to be reported and also ID linking multiple points to form the trajectory of an object.

## 2.5. RF feasibility for ISAC study

For the RF hardware assumption of option 1 as shown in Figure 2.5-1, although it is expected to work as TDD BS, however it will go to receive the backscattering signal from the sensing target or potential co-channel interference from other sensing BS transmitting signals, therefore in RAN4, we also need to further consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

Figure 2.5-1. Illustration of RF architecture for pulse wave signal [similar to the legacy TDD BS]

Proposal 8: For the 6G ISAC BS with option 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

For the generic baseband signal generation of option 1 as shown in Figure 2.5-2, it is somehow similar to LP-WUS signal generation to create the short pulse signal to minimize the blind detection regions.

Figure 2.5-2 Generation of pulse signal by traditional DFT-s-OFDM waveform [3]

For the RF hardware assumption of option 2 as shown in Figure 2.5-2, this is similar to SBFD RF architecture as specified in Rel-19. To enable this sensing feature, the Tx/Rx panels are separated to ensure the large isolation to minimize the receiver impacts. In addition, the sensing operation is full carrier transmission instead of DL sub-band transmission which is one significant difference compared with Rel-19 SBFD operation.

Figure 2.5-3. Illustration of RF architecture for contiguous wave signal [similar to Rel-19 SBFD digital IC calibration process]

Proposal 9: For the 6G ISAC BS with option 2 FDM operation, consider the RF feasibility including fully overlapping transmission/reception between DL and UL.

## 2.6. Coexistence study for ISAC study

### 2.6.1.Coexistence case for ISAC study

Since the ISAC system is designed as new RAT with different measurement metric compared with the legacy wireless communication system, in general, the appropriate coexistence study is needed to guarantee the coexistence with the legacy system and new ISAC system.

As shown in the following Figure 2.6-1/2 for ISAC BS with option 1/2 implementation, the coexistence studies between ISAC system and legacy system should be considered in 6G SI phase.

In addition, ISAC system coexisting with ISAC system in the adjacent channel should be also considered similar to Rel-20 SBFD coexisting with SBFD.

Figure 2.6-1. co-existence case for ISAC BS with option 1 assumption [similar to SBFD co-existence study]

Figure 2.6-2. co-existence case for ISAC BS with option 2 assumption [similar to SBFD co-existence study]

Proposal 10: for the 6G ISAC BS, the coexistence studies between ISAC system(s) and the legacy system need to be conducted to figure out the appropriate RF requirements.

In addition, for the different use case of ISAC as mentioned in section 2.2, the deployment is expected to be different. In the following table 2.6-1, we want to highlight the potential ISAC deployment to make the evaluation case more realistic and closer to the deployment.

Table 2.6. 1-1. the potential deployment for different ISAC use case

| Use case | Deployment | Remarks |
| --- | --- | --- |
| UAV detection and tracking | WA deployment |  |
| Pedestrian detection and tracking | WA deployment |  |
| Vehicle detection and tracking | WA deployment |  |
| AGV detection and tracking | Indoor factory scenario |  |
| Ship detection and tracking | WA deployment |  |
| Infrastructure collapse monitoring | WA deployment |  |
| Micro-deformation detection | WA deployment |  |

Proposal 11: for the different use case of ISAC deployment, propose to consider differentiating the deployment to make the evaluation more realistic and closer to the deployment.

### 2.6. 2.Coexistence case for ISAC study

In the following table 2.6.2-1, we provide some initial understanding on the potential coexistence cases for ISAC study in RAN4.

Table 2.6. 2-1. the potential deployment for different ISAC use case

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 4GHz | Urban Macro | Urban Macro |
| 2 | FR1 4GHz | Indoor factory | Indoor factory |
| 3 | FR2-1 28GHz | Urban Macro | Urban Macro |
| 4 | FR2-1 28GHz | Indoor factory | Indoor factory |
| 5 | Around 7GHz | Urban Macro | Urban Macro |
| 6 | Around 7GHz | Indoor factory | Indoor factory |
| Note 1: More scenarios could be added based on the consensus in RAN1/4. |  |  |  |

In the following table 2.6.2-2/3, we provide some initial analysis on adjacent channel interference for ISAC study from both mono-static and bi-static implementation.

Table 2.6. 2-2: Adjacent channel co-existence cases for mono-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 1 | ISAC | TDD DL | TDD DL | Option 1: Option 2: |
| 2 | TDD DL | No aggressor network | ISAC |  |
| 3 | ISAC | No aggressor network | ISAC |  |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

Table 2.6. 2-3: Adjacent channel co-existence cases for bi-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 4 | ISAC | TDD DL | TDD DL | Option 1: Option 2: |
| 5 | TDD DL | No aggressor network | ISAC |  |
| 6 | ISAC | No aggressor network | ISAC |  |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

### 2.6.3.Simulation assumption for ISAC coexistence study

Regarding the antenna configurations, ISAC BS is expected to configure with the same as 6GR BS since it is integrated BS to supporting both communication and sensing functionality. From RAN4 coexistence evaluation perspective, we propose to use the same configuration as captured in Annex E of TR 38.858 as starting point and make further updates based on discussion if necessary.

Proposal 12: for the 6G ISAC coexistence study in RAN4, propose to use the same configuration as captured in Annex E of TR 38.858 as starting point and make further updates based on discussion if necessary.

### 2.6. 4. Metrics for ISAC coexistence study

Similar to other legacy coexistence study in the past, in order to quantify the performance impacts, we need to discuss the acceptable performance degradation for both ISAC and legacy system or 6GR communication system due to the adjacent channel leakage.

Proposal 13: For the UAV use case of 6G ISAC study at least, propose to consider the distance, angle estimation and velocity estimation as the basic metric for the study in RAN4.

Proposal 14: For the UAV use case of 6G ISAC study at least, propose to consider the distance accuracy impacts, angle estimation accuracy impacts and velocity estimation accuracy impacts to quantify the performance impacts from both co-channel and adjacent channel.

Proposal 15: for the 6G ISAC coexistence study in RAN4, for 6GR performance metric, propose to use the legacy throughput loss as basic metric.

## 2.7. System parameters and RF requirement impacts for ISAC study

In the following table 2.7-1, we provide some initial view on the 6G ISAC BS requirement comparing to 6GR BS RF requirements.

Table 2.7-1: general view on the relationship between 6GR BS and 6G IASC requirement

|  | 6GR BS | 6G ISAC |
| --- | --- | --- |
| carrier bandwidth | Yes | The same as 6GR BS |
| Numerology | Yes | The same as 6GR BS |
| Channel raster | Yes | ## The same as 6GR BS |
| Sync raster | Yes | ## N/A |
| Base station output power | Yes | ## The same as 6GR BS |
| Output power dynamics | Yes | N/A |
| Transmit ON/OFF power | Yes | ## The same as 6GR BS |
| Transmitted signal quality | Yes | ## N/A |
| OBW | Yes | ## The same as 6GR BS |
| Tx intermodulation | Yes | ## The same as 6GR BS |
| ACLR | Yes | ## Depends on coexistence study |
| Operating band unwanted emissions | Yes | Depends on coexistence study |
| Transmitter spurious emissions | Yes | The same as 6GR BS |
| Rx part |  |  |
| REFSENS | Yes | E.g.distance, angle estimation and velocity estimation for UAV |
| dynamic range | Yes | FFS |
| ACS | Yes | Depends on coexistence study |
| blocking | Yes | Depends on coexistence study |
| NBB | Yes | FFS |
| RX IMD | Yes | Depends on coexistence study |
| RX NB IMD | Yes | FFS |
| RX spurious emission | Yes | The same as 6GR BS |
| RX ICS | Yes | FFS |

## 2.7. RRM requirement impacts for ISAC study

Similar to Rel-17 positioning WID, for 6G ISAC, it might be also necessary to define the certain RRM requirement at BS side to verify the sensing accuracy requirement, however these details is highly dependent on RAN1’s physical layer and framework design. We propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

Figure 2.7-1. Illustration of Rel-17 positioning framework.

Proposal 16: for the 6G ISAC RRM requirement, propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

## 2.8. Testing impacts for ISAC study

For ISAC BS, since its conformance testing is expected to be different from the legacy 5G conformance testing due to the moving testing target, therefore we propose to discuss the applicable conformance testing for 6G ISAC in the OTA chamber. In the following figure 2.8.1, we provide some initial understanding how this test setup should be built upon.

Figure 2.8-1.Illustration of OTA conformance testing for ISAC BS

Proposal 17: for the 6G ISAC conformance testing, propose to discuss the OTA test setup for conformance testing of moving sensing target.

Conclusions

In this contribution, we shared some initial views on the 6GR ISAC study and proposals are made as follows:

Proposal 1: Propose to discuss the timeline between option 1 and option 2 for 6G ISAC RAN4 study considering the overall workload in SI phase.

Figure 2.2-2. two alternatives of study timeline for 6G RAN4 ISAC

Proposal 2: For 6G ISAC, the applicable use cases should at least include

- Detection and tracking of UAV, Pedestrian, Vehicle, AGV, and Ship

- Infrastructure collapse monitoring

- Micro-deformation detection

Proposal 3: Collect the regulatory requirements for sensing if available from different regions.

Observation 1: In order to meet the RAN Sensing KPI for IMT-2030, the measurement bandwidth for sensing signal should be at least larger than 100MHz;

Observation 2: In order to meet more than 100MHz sensing signal bandwidth, FR1 high TDD bands and FR2-1 bands are more suitable for ISAC deployment;

Proposal 4: For 6G ISAC study, consider FR1 TDD with available spectrum wider than 100MHz at least, FR2-1 bands and FFS for FR3 bands;

Proposal 5: For the UAV use case of 6G ISAC study, consider the mono-static deployment as 1st priority and bi-static deployment as 2nd priority;

Proposal 6: For the UAV use case of 6G ISAC study, discuss the achievable timing accuracy performance among different BSs for bi-static deployment.

Proposal 7: For the 6G ISAC study, consider TDM based sensing operation and full duplex based sensing operation.

Proposal 8: For the 6G ISAC BS with option 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

Proposal 9: For the 6G ISAC BS with option 2 FDM operation, consider the RF feasibility including fully overlapping transmission/reception between DL and UL.

Proposal 10: For the 6G ISAC BS, the coexistence studies between ISAC system(s) and the legacy system need to be conducted to figure out the appropriate RF requirements.

Proposal 11: For the different use case of ISAC deployment, propose to consider differentiating the deployment to make the evaluation more realistic and closer to the deployment.

Proposal 12: For the 6G ISAC coexistence study in RAN4, propose to use the same configuration as captured in Annex E of TR 38.858 as starting point and make further updates based on discussion if necessary.

Proposal 13: For the UAV use case of 6G ISAC study at least, propose to consider the distance, angle estimation and velocity estimation as the basic metric for the study in RAN4.

Proposal 14: For the UAV use case of 6G ISAC study at least, propose to consider the distance accuracy impacts, angle estimation accuracy impacts and velocity estimation accuracy impacts to quantify the performance impacts from both co-channel and adjacent channel.

Proposal 15: For the 6G ISAC coexistence study in RAN4, for 6GR performance metric, propose to use the legacy throughput loss as basic metric.

Proposal 16: For the 6G ISAC RRM requirement, propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

Proposal 17: For the 6G ISAC conformance testing, propose to discuss the OTA test setup for conformance testing of moving sensing target.

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- RP-251742, Study on 6G scenarios and requirements - ISAC, ZTE,

- R1-2506131,Discussion on 5G-A ISAC evaluation,ZTE, Noted.

- RP-252024 RAN KPIs for IMT-2030