3GPP TSG-RAN WG4 Meeting #116bis	R4- 2513341

Prague, CZ, October 13rd – October 17th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM requirements in MRSS

Agenda item:	8.10

Document for:	Discussion

Introduction

At RAN#109 meeting, the following was agreed on the 6GR requirements for architecture and migration [1]. In particular, Multi-Radio Spectrum Sharing shall be supported to enable sharing of spectrum between 6GR and NR systems. In this paper, we are discussing the potential impacts on the RRM requirements due to MRSS introduction.

Requirements for architecture and migration - The 6G RAN architecture shall support standalone RAN architecture. - The RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public network.

Discussion

6GR radio on a new spectrum is an engine for capacity growth and enables wider channel bandwidths for elevated radio performance and improved spectral efficiency. A standalone 6GR architecture shall be the basic framework for 6GR deployment.  Considering the coexistence with 5G, we believe multi-radio spectrum sharing (MRSS) provides the best possible 5G to 6GR migration solution by dynamically sharing a carrier between 5G and 6G. As shown in Fig.1, the 5G and 6G systems are shared on radio level to minimize the impact on RAN architecture.

6G

Core

6G UE

RU

5G

Multi-RAT Radio

5G UE

User plane

Control plane

Fig.1 MRSS between 6GR and 5G

The details of MRSS design are under discussion in RAN meetings. In our views, 6GR design is expected to be stand-alone, and hence agnostic to MRSS deployment. Meanwhile, 6GR design will inherit the successful aspects of 5G design wherever relevant, and hence we expected that some signals and reference signals design can be inherited from 5G NR. In this direction, RAN1 is discussing reuse or share of 5G signal when feasible to enable:

- Reducing NW overhead by avoiding duplicated transmission of some reference signals.

- Enhancing 6G UE performance by leveraging 5G reference signals received by the UE.

Observation 1: 6GR radio is expected to be standalone, and 5G reference signals may be reused or shared when feasible.

From RAN4 point of view, many of the UE procedures and corresponding RRM requirements depend significantly on the MRSS design and 6GR reference signaling design done by RAN1 e.g. RRM measurements, mobility, SCell activation etc. If 5G and 6G radios are to share the same spectrum on the same carrier in MRSS, the RRM requirements could also be defined considering the potential to leverage 5G reference signaling to improve the performance. The feasibility and applicable scenarios for such RRM requirements shall also be carefully discussed based on the details of MRSS solution.

Proposal 1: Based on RAN1 progress on MRSS, RAN4 shall discuss the 6G RRM requirements considering the potential to leverage 5G reference signaling to improve performance. The feasibility and applicable scenarios shall be carefully discussed based on the details of MRSS solution.

Conclusion

In this paper we briefly discuss MRSS and possible RAN4 work on high level. However, as the RAN4 work has to wait a bit on the RAN1 progress we make the following Proposal:

Proposal 1: Based on RAN1 progress on MRSS, RAN4 shall discuss the 6G RRM requirements considering the potential to leverage 5G reference signaling to improve performance. The feasibility and applicable scenarios shall be carefully discussed based on the details of MRSS solution.

- References

- RP-252880, Summary of Discussion on 6G Requirements for architecture and migration, NTT DOCOMO, INC, RAN#109 meeting