3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513348

Prague, Czech Republic, 13 – 17th October 2025

Source:	BT

Title:	On demodulation and performance requirements in 6G Radio SI

Agenda item:	8.8

Document for:	Discussion

Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1] with the guidance for RAN4 demodulation and performance aspects. This contribution addresses aspects of demodulation and performance requirement framework for 6G, building on lessons learned from 5G NR. Our proposals aim to ensure that 6G specifications translate into credible, deployable performance improvements for networks and devices.

Discussion

In 5G, many requirements were defined at the component level (e.g., demodulation BLER, CSI reporting, or channel estimation in isolation). While this enabled easier testing, such approach does not reflect true system performance in the field, where these processes operate jointly. For example, demodulation accuracy depends strongly on channel estimation quality, interference handling, and link adaptation. In 6G, where physical layer features will be even more interdependent, requirements that target only isolated aspects may be insufficient. By focusing on integrated performance of combined functionalities, RAN4 can ensure that specifications validate end-to-end behaviour as experienced in live networks. This alignment with system-level performance makes the specifications more predictive of real-world outcomes and facilitates adoption by operators and vendors.

Performance requirement setting to move beyond individual component verification to assessment of the integrated performance of key functionalities and processes in 6G.

Legacy TDL models, widely used for performance requirement setting purposes in LTE and NR, lack spatial richness and do not capture realistic multi-antenna behaviour. This gap has already been observed in 5G, where requirements based on TDL sometimes failed to represent actual field performance of MIMO features. The Rel-19 study in TR 38.753 [2] introduced the rCDL model, which provides explainable angular spreads, spatial correlation, and Doppler evolution aligned with measured channels and with assumptions in RAN1 feature design. Using CDL/rCDL as the baseline for 6G ensures that MIMO features and CSI feedback are validated under representative propagation conditions. Starting with CDL avoids repeating past issues, ensures continuity with ongoing 5GA work, and provides a robust foundation for 6G requirement setting.

CDL-based channel modelling methodology should be used as a starting point for setting performance requirements for 6G.

In NR, baseline receivers were often defined conservatively, with simple linear detectors assumed as minimum. This could be discouraging stronger receiver implementations from being reflected in the standard. By contrast, modern chipsets and commercial deployments today could be benchmarked against advanced receivers such as MMSE-IRC and R-ML, as well as advanced precoding and interference suppression. Defining these as the baseline for 6G requirement setting ensures that specifications push performance forward, rather than anchoring it to outdated technology. It also ensures the standard remains relevant over the long lifecycle of 6G, which must remain competitive for more than a decade. Basing benchmarks on state-of-the-art receiver and transmitter technologies drives innovation, ensures that devices tested against requirements reflect realistic market capabilities, and provides operators with confidence that mandatory performance levels match user expectations.

: Performance benchmarks should be set using modern, competitive receiver and transmitter technologies to ensure that the specifications drive innovation and reflect the capabilities of state-of-the-art devices.

Conclusion

This contribution makes the following proposals:

- Performance requirement setting to move beyond individual component verification to assessment of the integrated performance of key functionalities and processes in 6G.

- CDL-based channel modelling methodology should be used as a starting point for setting performance requirements for 6G.

- : Performance benchmarks should be set using modern, competitive receiver and transmitter technologies to ensure that the specifications drive innovation and reflect the capabilities of state-of-the-art devices.

References

- RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), RAN #108, June 2025

- TR38.753, Study on spatial channel model for demodulation performance requirements for NR, Sep 2025