# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513362

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	4.15.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for NR RRM Phase 5

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM core requirements for NR RRM Phase 5 were discussed and the WF [1] was approved.

In this paper, we present our views on RRM core requirements maintenance for NR RRM Phase 5 captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 1-1-2: Re-evaluation mechanism Agreement: FFS following options in the maintenance stage: - Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be: - Alt 1: Use the values for T1 as a starting point. - Alt 2: 5 s, which is predefined in RAN4 spec. - Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS. - Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS. Agreement in Friday online: - No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

For re-evaluation mechanism, in our views, a predefined time duration T2 can be introduced, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. In order to avoid ping-pong, the timer for re-evaluation mechanism need to be considered coordinately with T1 for quitting mechanism. It was agreed in last meeting that the value for T1 is a fixed value of 90 seconds, in our views, the value for T1 can be considered as a starting point for the value of T2.

Proposal 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. And the value for T1 can be considered as a starting point for the value of T2.

- Conclusion

The contribution presents our views on RRM core requirements maintenance for NR RRM Phase 5 with the following proposals:

Proposal 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. And the value for T1 can be considered as a starting point for the value of T2.

- References

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509055, Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116, 25 – 29 August, 2025.