# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513372

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.4.3.2

Source:	China Telecom

Title:	Discussion on RRM requirements for (e)RedCap UE enhancement

Document for:	Discussion

- Introduction

In RAN #109 meeting, the new WID on NR Radio Resource Management (RRM) Phase 6 was approved [1]. There are two objectives in the work item, one is requirements for (e)RedCap UEs, the other is requirements on fast SCell activation for intra-band co-located contiguous CA.

In this paper, we present our views on RRM requirements for (e)RedCap UE enhancement described in the new WID [1].

- Discussion

The objectives in the new WID [1]:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

For relaxed RLM/BFD RRM requirements, the requirements are specified for legacy UEs in Rel-17 which are beneficial for UE power saving. Meanwhile, UE power saving is also important for RedCap UEs. In our views, R17 RLM/BFD relaxation for legacy UE can be used as a baseline, for 2Rx RedCap UE, the requirements for non-RedCap UE can be reused, and for 1Rx RedCap UE, the relaxation criteria and the scaling factor for relaxation requirement need to be further discussed.

Proposal 1: For relaxed RLM/BFD RRM requirements for (e)RedCap UEs, R17 RLM/BFD relaxation for legacy UE can be used as a baseline

- For 2Rx RedCap UE, the requirements for non-RedCap UE can be reused.

- For 1Rx RedCap UE, the relaxation criteria and the scaling factor for relaxation requirement need to be further discussed.

- Conclusion

The contribution presents our views on RRM requirements for (e)RedCap UE enhancement, with the following proposals:

Proposal 1: For relaxed RLM/BFD RRM requirements for (e)RedCap UEs, R17 RLM/BFD relaxation for legacy UE can be used as a baseline

- For 2Rx RedCap UE, the requirements for non-RedCap UE can be reused.

- For 1Rx RedCap UE, the relaxation criteria and the scaling factor for relaxation requirement need to be further discussed.

- References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple, RAN Meeting #109, September 2025.