3GPP TSG-RAN WG4 Meeting #116bis          	R4-2513399

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.14.4

Source:	Samsung

Title:	View on demodulation requirement for SBFD

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope for UE and BS demodualtion requirements with supporting SBFD feature was discussed,  the related agreement was captured into WF in [1] as following

- UE demodulation requirement - General Scope - Not to introduce UE demodulation requirements with SBFD carrier (with DL sub-band and UL sub-band) - BS demodulation requirement - Types of interference modeling for demodulation requirement with UL muting - Evaluate PUSCH performance with UL muting under inter-site gNB interference modelling - FFS on self-interference and intra-site inter sector interference - how to model interference modeling for performance evaluation - Considering TDL+INR type of interference model for evaluation - Note: the RF assumption can be considered for interference modelling - The following candidate value for INR can be considered for evaluation as starting point - Option 1: based on link budget analysis Conditions; FR1. Interf1 > Interf2; INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB. Interf1 = Interf2; INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Other options are not precluded - Interesting companies can provide the system level simulation to derive the INR value - Evaluation Metrics to choose interference model and make decision on requirement introduction - RAN4 can use the following metrics to choose interference model based on evaluation result as starting point - The peak throughput can be achieved - The obvious performance gain can be shown by comparing using muting Res for estimation to not using muted REs for estimation Types of interference modeling for demodulation requirement with UL muting - The target SNR (e.g., SNR at70% Tput) should be feasible for both network deployment and tests - Based on the evaluation, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting - Baseline Receiver Assumption - MMSE-IRC with same assumption in Rel-19 performance enhanced WI - Test setup for demodulation requirement if introduced - Frequency Range - For performance evaluation, prioritized FR1 - Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting - Waveform for wanted signal - For performance evaluation, prioritized CP-OFDM waveform - Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting - sub-band pattern - For performance evaluation, considering sub-band configuration as {DU} - Note: the UL sub-band is located in the lower band - Channel Bandwidth for wanted signal - For performance evaluation - For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS - For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS - TDD patterns - For performance evaluation - 30 kHz SCS: XXXXXXXXUU - 120 kHz SCS: XXXXU - UL sub-band transmission could start from the first F slots - Skip U slots to simply simulation setup - Sub-band configuration in SBFD slot - For performance evaluation - Take SBFD UL sub-band spans all the symbols with a SBFD slot - Number of symbol muting - For performance evaluation - Consider both 1 and 2 symbols, 1 symbol is prioritized - Companies are also encouraged to evaluate the number of muting symbols as 2 - Muting symbol configuration - For performance evaluation - PUSCH mapping type for wanted signal - For evaluation - mapping type A - Number of DMRS symbols for wanted signal - For evaluation - Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1 - Channel model - For evaluation - TDLC300-100 for wanted signal in FR1 - TDLC300-0 for interference signal if considering INR based interference modelling in FR1 - MCS - For evaluation - MCS16 for wanted signal in FR1 - [16QAM for interference signal in FR1], - Rank 1 for wanted signal - Rank 1 for interference signal as starting point to simply the test setup - Antenna Configuration - For evaluation - Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1 - 1Tx for gNB DL transmission as starting point to simply simulation setup - Other Assumption - For performance evaluation - Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively. - Specification Impact - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

In this contribution, the view on test scope and test setup for UE and BS demodulation requirement with supporting SBFD operation is provided. Meanwhile, the initial simulation results are provided for feasibility evaluation with UL muting

# 2	Overall RAN4 demodulation impact

2.1	BS demodulation requirement

Interference modeling

In the last meeting, RAN4 agreed to introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling.

Regarding the interference modelling for SBFD gNB, from gNB perspective, the interference scenario can be identified as 3 categories that

- gNB Self-Interference (SI)

- co-site inter-sector (or intra-site) co-channel inter-subband CLI

- gNB-to-gNB inter-site interference

From Link level simulation perspective, RAN1 considers them in the TDD system with considering the following modeling method in TR

- Option 1 - For SBFD slot, additive white Gaussian noise with variance of NSBFD is generated, where NSBFD=ISI+Ico-site+IgNB-gNB+IUE-gNB+N0 - ISI, Ico-site, IgNB-gNB, IUE-gNB are self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference (in linear scale), respectively - Based on the modelling method, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 2: For SBFD system with frame structure XXXXU, assume the SNR in UL only slot is SNRSBFD-UL=SNTDD and the SNR in SBFD slot is SNRSBFD-X=SNSBFD. Perform LLS to get the required SNR (SNRSBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Option 2 - The UE-gNB interference and inter-site gNB-gNB co-channel inter-subband CLI in LLS coverage evaluation are explicitly modelled based on a given topology of aggressor UEs and gNBs. The UE-gNB and gNB-gNB fast fading channels are explicitly modelled in LLS. The signal model is as follows - Y=HkSk +n≠kHnSn +HgSg+HCLI,mWmSm+Iself+N - Y is the received signal vector at the victim gNB Hn, Sn, n≠k, are the channel matrix and transmitted signal of the UE in the same cell as the target user - Hg and Sg are the channel matrix and transmitted signal of the UEs in the adjacent cell - HCLI,m, Wm and Sm are the channel matrix, the precoding matrix, and leakage CLI signal from aggressor gNB m to the victim gNB. - The power of the signal and interference is included in the channel matrix respectively - Iself and N are the self-interference vector of the co-site sectors and the thermal noise signal vector on the receiving antennas - Based on the above modelling, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 2: For SBFD system with frame structure XXXXU, perform LLS to get the required SNR (SNRSBFD-UL=SN0) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.)

According the TR 38.858, for above two options, self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1 dB densene similar as SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-XdB based on the assumption of co-site isolation αco-site.

Regarding the gNB-gNB interference, for option1, the additive white Gaussian noise is modeling, the value is derived based on a certain assumption of the topology of gNBs and UEs, and the interference is pre-receiver interference.

For option 2, the inter-site gNB-gNB interference is explicitly modelled with including the fading channel based on a given topology of aggressor gNBs.

In general, for option 2, the interference profile requires the system level simulation with proper test setup. It requires to define a commonly agreed leakage model.

Regarding the inter-site gNB-gNB interference, for evaluation purpose, the TDL+INR type of interference model are considered with some candidate values derived by link budget. Since our main targeting is to introduce the PUSCH requirement with UL symbol muting, where the main motivation to configure UL muting is to estimate the inter-gNB interference. Given that test purpose, we think it is enough to consider inter-site gNB-gNB interference with INR based modeling methodology for specifying the PUSCH requirement with UL symbol muting

For intra-sector gNB-gNB interference and gNB-gNB self-interference, based on RAN4 co-existence simulation assumptions for RF requirement study, we see that they can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1dB UL receiver sensitivity degradation. To some extent, with UL symbol muting, the noise estimation performance can also be improved. Therefore, if including the intra-sector gNB-gNB interference and gNB-gNB self-interference for specifying PUSCH requirement with UL muting, the additive white gaussian noise with fixed INR =-6dB for modeling can be considered.

Observation 1: Self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense similar to SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-X dB based on the assumption of co-site isolation  αco-site.

Observation 2: Either option 1 and option 2 for gNB-gNB interference modeling, the interference level is derived based on a certain assumption of the topology of gNBs and UEs.

Observation 3: The overall of inter-sector gNB-gNB interference including the gNB-gNB self-interference can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1 dB UL receiver sensitivity degradation

Proposal 3: RAN4 consider INR with fading based modeling methodology for inter-site gNB-gNB interference when specifying the PUSCH requirement with UL symbol muting. If including the including the intra-sector gNB-gNB interference and gNB-gNB self-interference for specifying PUSCH requirement with UL muting, the additive white gaussian noise with fixed INR =-6dB for modeling can be considered.

# 3	Simulation Results

In this section, the initial simulation results for UL muting were provided based on agreed simulation assumption. The simulation assumption can be refereed table 1

| Parameter |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interfer 1 | Interfer 2 |
| Transform precoding |  |  | Disabled |  |  |
| Example TDD UL-DL pattern (Note 1) |  |  | 30 kHz SCS: XXXXXXXXUU  UL sub-band transmission could start from the first F slots  Skip U slots to simply simulation setup |  |  |
| INR (Note3) |  | Set 1(Interf1 > Interf2) | N/A | Case 1: 5 Case 2: 9 Case 3: 13 Case 4: 17 | Case 1: -5 Case 2: -1 Case 3: 3 Case 4: 7 |
|  |  | Set 2 (Interf1 = Interf2) |  | Case 1: -1 Case 2: 3 Case 3: 7 Case 4: 11 | Case 1: -1 Case 2: 3 Case 3: 7 Case 4: 11 |
| Cell ID |  |  | 0 | 1 | 2 |
| Interference model |  |  | N/A |  |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
|  | RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DM-RS | DM-RS configuration type |  | 1 |  |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |  |
|  | Additional DM-RS position |  | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |  |
|  | DM-RS port(s) |  | {0} |  |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | A |  |  |
|  | Start symbol |  | 0 |  |  |
|  | Allocation length |  | 14 |  |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |  |
|  | Frequency hopping |  | Disabled |  |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |  |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. NOTE 2: One explicit interferer, i.e., interferer 1, is modelled for tests with 2/4/8 RX antennas. Two explicit interferers are modelled for tests with 2/4/8 RX antennas. NOTE 3: INR is defined in Annex B.X.1. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. NOTE 4 All cells are time-synchronous. |  |  |  |  |  |

For UL symbol muting pattern, since the interference modeling is considered from neighboring gNB without considering the mobility, there is no performance difference excepted with the UL symbol muting pattern in different symbol location. In current performance evaluation, we consider the following UL symbol muting pattern to check the performance difference with and without using muting REs for interference covariance estimation.

UL symbol muting pattern

Figure 1: Muting symbol configuration with 2 symbols muting

Figure 2: Muting symbol configuration with 1 symbol muting

The simulation results were summarized in table 2

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.444 | 13.158 | 0.286 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 14.653 | 14.376 | 0.277 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 3 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 17.266 | 16.854 | 0.412 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 4 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 20.662 | 19.77 | 0.892 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.69 | 12.369 | 0.321 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.59 | 13.297 | 0.293 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 3 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 15.043 | 14.74 | 0.303 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 4 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 17.835 | 17.31 | 0.525 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.908 | 12.682 | 0.226 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 14.213 | 14.058 | 0.155 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 3 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 16.947 | 16.586 | 0.361 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 4 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 20.209 | 19.412 | 0.797 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.152 | 11.918 | 0.234 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.055 | 12.849 | 0.206 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 3 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 14.615 | 14.419 | 0.196 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 4 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 17.435 | 17.069 | 0.366 |

Based our evaluation, the following observations can be drawn as

For all the cases evaluated based on the candidate INR value proposed by company, there is at least around 0.15dB gain with using muting REs for interference covariance estimation.

Under high INR condition, where two interferes are considered with INR1 is larger than INR2, the performance gain achieved can be up to 0.9dB for 2 UL symbols muting scenario, and up to 0.8dB for 1 UL symbol scenario. Based on our evaluation, we think the feasibility of UL symbol muting has already been verified. Therefore, we propose to introduce PUSCH requirement with UL symbol muting.

Observation 4: For all the cases evaluated based on the candidate INR value proposed by company, there is at least around 0.15dB gain with using muting REs for interference covariance estimation

Observation 5: Under high INR condition, where two interferes are considered with INR1 larger than INR2, the performance gain achieved can be up to 0.9dB for 2 UL symbols muting scenario, and up to 0.8dB for 1 UL symbol scenario.

Proposal 2: RAN4 introduce the PUSCH requirement with UL symbols muting.

# 4	Test setup

Based on evaluation, it is feasible to define PUSCH requirement with UL symbol muting. Regarding the test setup for requirement, the following simulation assumption can be considered.

Frequency Rang

Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

considering sub-band configuration as {DU}, wherethe UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

30 kHz SCS:  XXXXXXXXUU

120 kHz SCS: XXXXU

UL sub-band transmission could start from the first F slots

Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

Consider 2 symbols muting configuration

Muting symbol configuration

PUSCH mapping type for wanted signal and interferers

Both mapping A and mapping B in FR1

Mapping B in FR

Number of DMRS symbols for wanted signal and interfers

Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

Channel model

TDLC300-100 for wanted signal in FR1

TDLC300-0 for interference signal if considering INR based interference modelling in FR1

MCS and Rank

MCS16 for wanted signal in FR1 and FR2

16QAM for interference signal in FR1],

Rank 1 for wanted signal and interference signal

Antenna Configuration

Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Other Assumption

Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

Proposal 3: RAN4 introduce the PUSCH requirement with UL symbols muting with the following assumption

Frequency Rang

Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

considering sub-band configuration as {DU}, wherethe UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

30 kHz SCS:  XXXXXXXXUU

120 kHz SCS: XXXXU

UL sub-band transmission could start from the first F slots

Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

Consider 2 symbols muting configuration

Muting symbol configuration

PUSCH mapping type for wanted signal and interferers

Both mapping A and mapping B in FR1

Mapping B in FR

Number of DMRS symbols for wanted signal and interfers

Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

Channel model

TDLC300-100 for wanted signal in FR1

TDLC300-0 for interference signal if considering INR based interference modelling in FR1

MCS and Rank

MCS16 for wanted signal in FR1 and FR2

16QAM for interference signal in FR1],

Rank 1 for wanted signal and interference signal

Antenna Configuration

Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Other Assumption

Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

# 5	Conclusion

In this contribution, the view on the demodulation scope for SBFD is provided.

Observation 1: Self-interference can be modeled as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense similar to SLS, and co-site inter-sector interference can be modelled as additive white gaussian noise with fixed INR=-X dB based on the assumption of co-site isolation  αco-site.

Observation 2: Either option 1 and option 2 for gNB-gNB interference modeling, the interference level is derived based on a certain assumption of the topology of gNBs and UEs.

Observation 3: The overall of inter-sector gNB-gNB interference including the gNB-gNB self-interference can be modelled as additive white gaussian noise with fixed INR =-6dB targeting 1 dB UL receiver sensitivity degradation

Proposal 3: RAN4 consider INR with fading based modeling methodology for inter-site gNB-gNB interference when specifying the PUSCH requirement with UL symbol muting. If including the including the intra-sector gNB-gNB interference and gNB-gNB self-interference for specifying PUSCH requirement with UL muting, the additive white gaussian noise with fixed INR =-6dB for modeling can be considered.

Observation 4: For all the cases evaluated based on the candidate INR value proposed by company, there is at least around 0.15dB gain with using muting REs for interference covariance estimation

Observation 5: Under high INR condition, where two interferes are considered with INR1 larger than INR2, the performance gain achieved can be up to 0.9dB for 2 UL symbols muting scenario, and up to 0.8dB for 1 UL symbol scenario.

Proposal 2: RAN4 introduce the PUSCH requirement with UL symbols muting.

Proposal 3: RAN4 introduce the PUSCH requirement with UL symbols muting with the following assumption

Frequency Rang

Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

considering sub-band configuration as {DU}, wherethe UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

30 kHz SCS:  XXXXXXXXUU

120 kHz SCS: XXXXU

UL sub-band transmission could start from the first F slots

Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

Consider 2 symbols muting configuration

Muting symbol configuration

PUSCH mapping type for wanted signal and interferers

Both mapping A and mapping B in FR1

Mapping B in FR

Number of DMRS symbols for wanted signal and interfers

Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

Channel model

TDLC300-100 for wanted signal in FR1

TDLC300-0 for interference signal if considering INR based interference modelling in FR1

MCS and Rank

MCS16 for wanted signal in FR1 and FR2

16QAM for interference signal in FR1],

Rank 1 for wanted signal and interference signal

Antenna Configuration

Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Other Assumption

Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

# References

RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung