3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513401

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.8.4

Source:	Samsung

Title:	Simulation results of PUSCH demodulation requirement for ATG

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup for performance part of ATG for Rel-19 was under discussion. The related agreement was captured into WF in [1] as following

- Test scope - Antenna configuration for incremental test - 2T2R with X-pol low correlation. - MCS for incremental test - 64QAM table 1 MCS 20 - Test applicability rule for 2Tx - Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1).

In this contribution, based on the agreed simulation assumption, the ideal simulation results are provided for alignment

# 2	Simulation Results

In this section, the ideal simulation results are provided based on the agreed simulation assumption as following

Table 1: Simulation assumption for PUSCH requirement under ATG scenario

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| CHBW |  | FDD: 5MHz TDD:10MHz |
| MCS |  | 64QAM Table 1 MCS 20 |
| Antenna configuration |  | 2T2R with X-pol low correlation |
| Propagation conditions and Frequency offset |  | FDD 15kHz SCS: H= 1j1-j+200Hz TDD 30kHz SCS: H= 1j1-j+500Hz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

Table 2: Initial simulation results for PUSCH with 2Tx/2Rx

| Case | Tx/Rx | SCS | BW | Channel | Frequency offset | Mapping Type | DMRS | MCS | SNR@70% TP (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2Tx/2Rx | 15KHz | 5MHz | Static channel | 200Hz | A | (2,11) | MCS 20 table 1 | 13.4 |
| 2 | 2Tx/2Rx | 30KHz | 10MHz | Static channel | 500Hz | A | (2,11) | MCS 20 table 1 | 13.6 |

# 3	Conclusion

In this contribution, the ideal simulation results with UL MIMO with 2Tx for ATG scenario were provided for alignment purpose.

# References

R4-2512602, Way Forward for [116][326] NR_ATG_enh_demod, CMCC