3GPP TSG-RAN WG4 Meeting #116bis				                              R4-2513447

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.14.3

Title: 	Discussion RRM requirements test cases for Ambient IoT

Source: 	Xiaomi

Document for:		Discussion

- Introduction

The work item of Ambient IoT was approved in [1, RP-243326]. And in the las meeting, the corresponding RRM core requirements were agreed [2, R4-2504905]. In this paper we start the discussion on whether and how to define the test cases for AIoT RRM requirements.

- Discussion

In the last RAN4 meeting, RAN4 agreed to define the following RRM test cases for AIoT device in Rel19 [3].

Issue 2-1-1: Test scenario Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

For the timing issue, the expected performance impacts can be observed in the demod part.

Observation 1: The performance impact due to D2R transmit timing error (e.g. SFO) can be reflected by the demod test.

Thus, we can propose that:

Proposal 1: RAN4 can define a single D2R transmit timing test cases for both demod and RRM .

On the other hand, for the random access as illustrated in Figure 1 below, since the access failure or not will depend on whether D2R upper layer transfer message (MSG3) reception by the reader [5]. That is such test failure could be upon the device side as the demod test.

Observation 2: Whether the testing for AIoT random access is failure or not is upon the reader reception.

Figure 1. AIoT Random access procedure

Thus, we can propose that:

Proposal 2: RAN4 can define the dedicated RRM test cases for AIoT random access.

According to the physical layer design [5] (e.g. SCS =15kHz only), the test cases with 15KHz SCS is needed only.

And for BW, since there are amount of TBW size for R2D, RAN4 needs to consider part of them to be tested in order to reduce the testing efforts.

Therefore, we can also propose the common testing configuration as:

Proposal 3: The common testing configuration for Rel19 AIoT RRM test cases can be defined as:

- SCS = 15KHz

- BW =[180kHz]

- Device 1 only

- Summary

This paper provided our consideration on RRM test cases for AIoT.

Observation 1: The performance impact due to D2R transmit timing error (e.g. SFO) can be reflected by the demod test.

Proposal 1: RAN4 can define a single D2R transmit timing test cases for both demod and RRM .

Observation 2: Whether the testing for AIoT random access is failure or not is upon the reader reception.

Proposal 2: RAN4 can define the dedicated RRM test cases for AIoT random access.

Proposal 3: The common testing configuration for Rel19 AIoT RRM test cases can be defined as:

- SCS = 15KHz

- BW =[180kHz]

- Device 1 only

- References

- RP-243326

- R4-2504905

- R4-2512134

- 3GPP TS 38.133 v18.7.0

- 3GPP TS38.191 v19.0.0