3GPP TSG-RAN WG4 Meeting #116bis	R4-2513451

Prague, Czech, Oct. 13-17, 2025

Source: 	Xiaomi

Title: 	Draft reply LS on BS Tx power assumption for coverage evaluation in A-IoT

Agenda item:	7.11.2

Document for:	Discussion

- Introduction

RAN1 send a LS [1] to RAN4 to inform the following information,

For the Rel-20 study, RAN1 discussed BS Tx power assumption for coverage evaluation in A-IoT and made the following agreements:

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Agreement For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

This contribution discusses how to assume the in-band mode power in RAN4 co-existence simulation. A draft LS is also provided in the annex.

- Discussion

In the SID [2], there’s no clear assumption that if AIoT reader for in-band operation shares hardware with NR BS. If they share hardware, the same PSD should apply. However, if the same PSD assumption applies, the AIoT reader power is limited. For example, for 10MHz CBW, 1RB power is 17 dB lower than 52 RB total power. If NR BS Tx power is 46 dBm, AIoT reader power is only 29 dBm which is smaller than either of the choices in RAN1 LS.

Observation 1: Shared hardware assumption limits the AIoT reader power for small R2D CBW.

For RAN4 of co-existence simulation, it may be safe to simulate higher power for AIoT reader first. If high interference is found for the higher power, it can be considered to choose the lower power level. But of course, both of the two choices can be simulated by companies.

RAN4 discussed AIoT BS Tx power assumption for RAN4 co-existence simulation purpose and have the following agreement,

RAN4 will use the following AIoT reader Tx power assumption in RAN4 co-existence simulation for D4T1 scenario,

- For standalone: 43 dBm

- For in-band:

  - Both 38 and 33 dBm can be assumed.

    - 38 dBm is in high priority as the starting point.

RAN4 will update the assumption if severe interference is found in RAN4 co-existence evaluation due to the high AIoT reader Tx power.

Based on the above analysis, we propose to reply RAN1 LS as followings,

A draft LS is provided in Annex for reference.

Reference

[1] R1-2506575, “LS on BS Tx power assumption for coverage evaluation in A-IoT”, RAN1#122

[2] RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108

Annex: Draft reply LS

3GPP TSG-RAN WG4 Meeting #116bis	R4-250xxxx

Prague, Czech, Oct. 13-17, 2025

Title:	Reply LS on BS Tx power assumption for coverage evaluation in A-IoT

Response to:	LS R1-2506575 on BS Tx power assumption for coverage evaluation in A-IoT from RAN1

Release:	Release 20

Work Item:	FS_Ambient_IoT_Outdoor

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:	Huiping Shan

E-mail Address:	shanhuiping@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN4 would like to thank RAN1 for the LS on BS Tx power assumption for coverage evaluation in A-IoT in R1-2506575 (R4-2513009).

RAN4 discussed AIoT BS Tx power assumption for RAN4 co-existence simulation purpose and have the following agreement,

RAN4 will use the following AIoT reader Tx power assumption in RAN4 co-existence simulation for D4T1 scenario,

- For standalone: 43 dBm

- For in-band:

  - Both 38 and 33 dBm can be assumed.

    - 38 dBm is in high priority as the starting point.

RAN4 will update the assumption if severe interference is found in RAN4 co-existence evaluation due to the high AIoT reader Tx power.

2	Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to take above agreement in the future work.

3	Dates of next TSG RAN WG4 meetings

TSG RAN4 Meeting #117		17 - 21 Nov 2025					Dallas, U.S.

TSG RAN4 Meeting #117		9 - 13 Feb 2026					Stor-Göteborg, SE