3GPP TSG-RAN WG4 Meeting #116bis                   	R4-2513465

Agenda item:			7.6.4.1

Source:	Xiaomi

Title:	Discussion on R20 HD-FDD NTN RF requirement

Document for:	Discussion

Background

In t he RAN#109 meeting, New WID on Enhanced requirements for HD-FDD for NTN Core Part by the meeting aims to determine the RF requirement for HD-FDD VASTs for both Ka and Ku bands with FR1 and FR2 numerologies.

HD-FDD for NTN Core Part

- Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 2:  Ku band and Ka band with FR2 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

- Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

In this document, we would like to discuss the Phase 1 of the work.

Disucussion

As the information for this NTN HD-FDD VSAT UE types shown as below:

Table 1 NTN VSAT class and type

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: UE may need power reduction for meeting OFF-axis EIRP requirement defined in clause 9.2.2. Value is implementation dependent. NOTE3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247, LEO support is only defined for n509, n508 and n248, n247 NOTE3: VSAT type 6 is only defined for n509, n508 and n248,n247 |  |  |

The first request for the phase 1 of this HD-FDD for NTN Core Part is “Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna”.

RAN4 should discuss both HD-FDD fixed and mobile VSAT which can support GSO and LEO, LEO only electronic steering antenna.

Propose to clarity/discuss the difference between fixed VSAT and mobile VSAT in HD-FDD VAST.

Here are some key advantages of a 20 cm-small-antenna HD-FDD Mobile VSAT: it eliminates the duplexer, cuts power consumption by more than half, drops BOM cost, and still closes the link thanks to its improved receiver sensitivity and high-efficiency PA, all while fitting into a laptop-thin terminal that can ride on a backpack, vehicle or unmanned boat and hop transparently between LEO and GSO beams.

In the Table-1 which was discussed in the Rel-19, the mobile VAST have type 5 and 6. However, to specific a new type of 20 cm-small-antenna HD-FDD Mobile VSAT means that the type 5 and 6 cannot cover this type of Mobile VSAT. This new type should cover the feature “small size” that should be separated with the above types.

Meanwhile, we should not limit the “small size” antenna to one size or one configuration, e.g. 20cm * 20cm, it is better to have a range for the size or antenna element or antenna configuration, in order to distinguish it from standard-size VSAT antennas. Otherwise, it would be better to keep the original VSAT type.

RAN4 should discuss how to distinguish standard-size VSAT antennas and small size VSAT antennas.

Consider the following new mobile VSAT types for the small size antenna.

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
|  | 7 | Mobile VSAT communicating with GSO and LEO with small size electronic steering antenna. |
|  | 8 | Mobile VSAT communicating with LEO only with small size electronic steering antenna. |

Small size antenna, means small maximum EIRP due to the size of antenna will limit the number of antenna element. The link budget should be considered by the meeting in order to make sure the connection.

Encourage VSAT providers to provide parameters of small size antenna mobile VSAT to the RAN4 meetings.

Conclusion

- RAN4 should discuss both HD-FDD fixed and mobile VSAT which can support GSO and LEO, LEO only electronic steering antenna.

- Propose to clarity/discuss the difference between fixed VSAT and mobile VSAT in HD-FDD VAST.

RAN4 should discuss how to distinguish standard-size VSAT antennas and small size VSAT antennas.

Consider the following new mobile VSAT types for the small size antenna.

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
|  | 7 | Mobile VSAT communicating with GSO and LEO with small size electronic steering antenna. |
|  | 8 | Mobile VSAT communicating with LEO only with small size electronic steering antenna. |

Encourage VSAT providers to provide parameters of small size antenna mobile VSAT to the RAN4 meetings.