3GPP TSG-RAN WG4 Meeting # 116bis		                                         R4-2513467

Prague, Czech Republic, 13-17 October 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	4.15.1

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during the last meeting. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

The enhancement on fast SCell activation for UE supporting EMR was completed in previous meetings. In last RAN4 meeting, some maintenance issues and UE capability were discussed with agreements captured in [1]. In this contribution, we further provide our views on the UE capabilities.

Discussion

Optimising Rx BSF

In the last meeting, there is one remaining issue about the BSF MO selection when multiple bands are configured for BFS and reporting. One option captured in the WF [2] is captured in the following:

Way forward: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

The scenario that the MOs are configured in multiple bands, and the UE report the RSRP/RSRQ value for BSF. If UE report the RSRP/RSRQ value of all these MOs meet the threshold of BSF, which MO should be set as the BSF MO is the issue. However, the scenario of the BSF is one FR2-2 serving MO and one target MO. When the serving cell RSRP decreases, the UE will start the target fast measurement on the MO that is configured by the network. Therefore, the multiple band MOs need UE to do the BSF is not in the Rel-19 BSF scope.

Proposal 1: The scenario that multiple bands are involved and UE reports different BSF values for different bands is out of the Rel-19 BSF scope. Since the scenario of Rel-19 BSF is one serving cell and one target MO.

Optimising CSSF outside gap

In the last meeting, the companies agreed to introduce Solution 1 and Solution 3. For these two solutions, there are still some parts that need to be discussed. There are two points we would like to provide our views which are whether solution 1 is possible to extend to the FR1 and FFS of solution 3.

Solution 3

In the last meeting, the 3-searcher solution CR has been endorsed; some remaining issues need to be discussed.

Issue of inter-frequency measurement in SA mode

The agreement in the last meeting that captured in the WF[1].

Issue 1-2-1: applicability limitation for solution 3 Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Following the last RAN4 meeting, it was agreed that inter-RAT measurements without measurement gap would be conducted using the legacy two-searcher approach. This issue is related to the GAP enhancement. In our view, the MO without measurement gap are the same type of MO and under the same restriction; UE can only handle one CC that is different from the serving CC at one moment. Here we would like to provide one example.

Figure 1. Example of UE handling two inter-frequency without measurement gap

This example demonstrates a UE with three-searcher capability where two CCs, distinct from the serving CC, require measurement without a measurement gap. While the three-searcher solution theoretically enables the UE to measure three CCs simultaneously without a measurement gap, measurement gap limitations must be considered.

During prior discussions on measurement gap enhancement, it was established that UE can only perform measurements on one inter-frequency CC within a specific band at any given time, in addition to maintaining the serving cell connection and performing one intra-frequency measurement. The relevant UE capability parameters are as follows:

#### – NeedForGapsInfoNR The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured. NeedForGapsInfoNR information element - -- ASN1START - -- TAG-NeedForGapsInfoNR-START - NeedForGapsInfoNR-r16 ::= SEQUENCE { - intraFreq-needForGap-r16 NeedForGapsIntraFreqList-r16, - interFreq-needForGap-r16 NeedForGapsBandListNR-r16 - } - NeedForGapsIntraFreqList-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 - NeedForGapsBandListNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 - NeedForGapsIntraFreq-r16 ::= SEQUENCE { - servCellId-r16 ServCellIndex, - gapIndicationIntra-r16 ENUMERATED {gap, no-gap} - } - NeedForGapsNR-r16 ::= SEQUENCE { - bandNR-r16 FreqBandIndicatorNR, - gapIndication-r16 ENUMERATED {gap, no-gap} - } - -- TAG-NeedForGapsInfoNR-STOP - -- ASN1STOP

This capability indicates that UE can handle the serving cell connection, one intra-frequency CC measurement, and one inter-frequency CC measurement. No defined requirement or UE capability would allow the UE to manage the serving cell connection alongside one intra-frequency and two inter-frequency CC measurements.

Observation 1: There is no requirement or UE capability that would allow the UE to manage the serving cell connection alongside one intra-frequency and two inter-frequency CC measurements.

Given these constraints, we note that restricting inter-RAT MOs without measurement to the legacy two-searcher solution would not fully resolve the issue. Therefore, we recommend that the 3-searcher solution should not be applied to inter-frequency MOs without a measurement gap, and instead allow them to continue using the legacy 2-searcher solution.

Proposal 2: To avoid contradicting the gap enhancement requirement, the 3-searcher solution should not be applied to inter-frequency MOs without a measurement gap directly. Instead, a relaxation for inter-frequency MOs without a measurement gap is needed in the 3-searcher solution.

UE supporting Rel-18 EMR

In last RAN4 meeting, one controversial issue is whether to introduce new UE capabilities for Rel-19 fast SCell activation based on Rel-18 EMR. The status is summarized as follows:

## Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR ### Issue 2-1-1: UE capability for EMR-based fast SCell activation Agreement in Friday online: - Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

From our understanding, as long as UE supports IDLE mode EMR, UE shall be able to support the Rel-19 fast SCell activation. Based on the discussion in previous meetings, the final SCell activation delay is rather conservative compared with the original proposals (i.e. exactly the same as the legacy known case). Only one/two samples are reduced compared with purely unknown cases. However, RAN4 have an agreement that introduces a new UE capability for R19 SCell activation based on EMR. We propose the new UE capability as shown in the Annex.

This new Rel-19 capability is needed to link to the Rel-18 39-8 or 39-9 UE feature in the Rel-18 UE feature list, which is shown as follows in Table 5.2 in Annex.

Observation 2: There is no need to add the R16 feature in the Prerequisite feature group part.

Proposal 3: The details of UE capability for Rel-19 fast SCell activation based on EMR are shown in Annex Table 5.1.

- 3. Conclusions

Proposal 1: The scenario that multiple bands are involved and UE reports different BSF values for different bands is out of the Rel-19 BSF scope. Since the scenario of Rel-19 BSF is one serving cell and one target MO.

Observation 1: There is no requirement or UE capability that would allow the UE to manage the serving cell connection alongside one intra-frequency and two inter-frequency CC measurements.

Proposal 2: To avoid contradicting the gap enhancement requirement, the 3-searcher solution should not be applied to inter-frequency MOs without a measurement gap directly. Instead, a relaxation for inter-frequency MOs without a measurement gap is needed in the 3-searcher solution.

Observation 2: There is no need to add the R16 feature in the Prerequisite feature group part.

Proposal 3: The details of UE capability for Rel-19 fast SCell activation based on EMR are shown in Annex Table 5.1.

- 4. References

- R4-2512124 WF on RRM requirements for NR_RRM_Ph5_Part1

[2] R4-2512125 WF on RRM requirements for NR_RRM_Ph5_Part2

- Annex

Table 5.1 UE capability for Rel-19 fast SCell activation based on EMR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on EMR measurement | Supports fast SCell activation based on EMR measurement as specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 (RAN4 R18 feature list) | Yes | N/A | UE does not support fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per-UE | No | No | N/A |  | Optional without capability signaling |

Table 5.2 RAN4 Feature list of Release 18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-8 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup |  | Yes | N/A | UE does not support measurement validation during connection setup/resume | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-9 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation and report based on non-EMR measurement during connection setup/resume for fast CA/DC setup |  | Yes | N/A | UE does not support measurement validation during connection setup/resume | Per-UE | No | FFS | N/A |  | Optional with capability signaling |