3GPP TSG-RAN WG4 Meeting # 116bis	 R4-2513472

Prague, Czech Republic, 13-17 October 2025

Agenda item:	6.12.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of other RRM requirements

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we provide our views regarding the RAN4 impacts on the performance part of CJTC and asymmetric DL sTRP/UL mTRP.

- Discussion

UE reporting enhancement for CJT calibration

Test Scope

During the last meeting, RAN4 had a WF about the CJTC test scope. The content has been captured as follows:

Issue 2-2-3: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Nokia; Ericsson; MTK. Whether to define TCs to verify CJTC-Dd, measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-f, measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-P, (To be verify?); N; N; N. Whether to define TCs to verify scheduling restriction; Y (if core req.). Whether to define TCs to verify CJTC-Dd, CJTC-f, accuracy; Y; Y.

We would like to update our view based on this table. The following aspect is necessary to define the test case:

- Measurement period CJTC-Dd and CJTC-f.

- Do not define the test case for CJTC-P, as no requirement was introduced in the core part.

- Scheduling restriction.

- Verifying the accuracy of CJTC-Dd and CJTC-f.

Proposal 1: Defined test cases for CJTC in the following aspects:

- Measurement period CJTC-Dd and CJTC-f.

- Do not define the test case for CJTC-P, as no requirement was introduced in the core part.

- Scheduling restriction.

- Verifying the accuracy of CJTC-Dd and CJTC-f.

Note: We update our views about the CJTC test scope in the table, which you can find in the Annex.

CJTC accuracy

During the last meeting, the RAN4 agreed to define the core requirement of CJTC-Dd and CJTC-f based on 1 sample. The content of the agreements is captured from the WF[5] and shown as follows:

Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration? Agreement: Define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part. Companies are encouraged to discuss how to derive the accuracy performance based on the simulation results.

Companies provided the approach that the UE report accuracy is needed to meet the error budget on the network side to ensure the CJTC work well. In our view, the network error budget is realised through a particular implementation. Different companies may have different implementations of the CJTC, such as the algorithm, and it is hard to come up with a typical error budget value from the network side.

Observation 1: It is hard to come up with a typical value for the error budget, due to this is related to the network implementation.

To define the accuracy of CJTC-Dd and CJTC-f, the current simulation results can be the reference. Here, we provided our view on the accuracy value for FO and DO.

Frequency offset accuracy:

SCS; SNR; Frequency offset (FO) b/w two TRPs; FO estimation error in AFO = 0.2 ppm and MFO = 256. MTK; Ericsson; Huawei; Nokia. Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels. 256; 256; 256; 256. #samples. #1; #1; #1; #1. 15KHz; -3; 350 Hz; 65; 14; 72; 24. 15KHz; 0; 350 Hz; 39; -; 39; 16. 15KHz; 3; 350 Hz; 26; 7; 33; 11. 30KHz; -3; 350 Hz; 133; 26; 148; 42. 30KHz; 0; 350 Hz; 83; -; 104; 27. 30KHz; 3; 350 Hz; 52; 12; 68; 17.

Based on the simulation results from companies, we propose that the SNR of CJTC-f needs to be set to 0 dB, and the accuracy value we propose is as follows.

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 39 | 0 | 15 | 1 |
| 33 | +3 |  |  |
| 104 | 0 | 30 |  |
| 68 | +3 |  |  |

With some tolerances:

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 |  |  |
| 108 | 0 | 30 |  |
| 70 | +3 |  |  |

Delay offset error budget:

SCS; SNR; Frequency offset (FO) b/w two TRPs; TO estimation error in AD = CP and MD = 256. MTK; Ericsson; Huawei; Nokia. Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels; Max(CDF5%, CDF95%) #levels. 256; 128; 128; 256. #samples. #1; #1; #1; #1. 15KHz; -3; 0.5CP; 29; 4; 28; 8. 15KHz; 0; 0.5CP; 17; -; 16; 8. 15KHz; 3; 0.5CP; 10; 3; 12; 5. 30KHz; -3; 0.5CP; 30; 5; 16; 8. 30KHz; 0; 0.5CP; 16; -; 10; 8. 30KHz; 3; 0.5CP; 14; 5; 8; 8.

Based on the simulation results from companies, we propose that the SNR of CJTC-Dd needs to be set to 0 dB, and the accuracy value we propose is as follows.

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 17 | 0 | 15 | 1 |
| 12 | +3 |  |  |
| 16 | 0 | 30 |  |
| 14 | +3 |  |  |

With some tolerances:

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 |  |  |
| 18 | 0 | 30 |  |
| 16 | +3 |  |  |

Proposal 2: RAN4 to consider the following accuracy value and SNR of the CJTC-Dd and CJTC-f:

Option 1: Worst value in the simulation results.

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 39 | 0 | 15 | 1 |
| 33 | +3 |  |  |
| 104 | 0 | 30 |  |
| 68 | +3 |  |  |
| Delay offset |  |  |  |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 17 | 0 | 15 | 1 |
| 12 | +3 |  |  |
| 16 | 0 | 30 |  |
| 14 | +3 |  |  |

Option 2: Worst value in the simulation results with additional tolerances.

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 |  |  |
| 108 | 0 | 30 |  |
| 70 | +3 |  |  |
| Delay offset |  |  |  |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 |  |  |
| 18 | 0 | 30 |  |
| 16 | +3 |  |  |

Enhancement for asymmetric DL sTRP/UL mTRP

During the last RAN4 meeting, the core part discussion for DL sTRP/UL mTRP was finished. In this paper, we will provide our views on the performance part of asymmetric DL sTRP/UL mTRP based on the core part agreements. According to the summary in the WF[5], the test scope of asymmetric DL sTRP/UL mTRP, which is captured from WF [5], is as below.

Issue 2-3-1: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Huawei; Nokia; MTK. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; Both for FR1. Scenario#2 for FR2; Both; Both; Scenarios#1; both; Scenarios#1.

In this paper, we aim to present our analysis of the performance requirements of enhancements for asymmetric DL sTRP/UL mTRP. Given the discussions and agreements on the core requirements, it is necessary to determine whether to design test cases to verify these enhancements.

- Timing requirements: The legacy timing accuracy test case can be reused and does not need to be defined as a new test case.

Proposal 3: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates.

- TCI state switching requirements: Since there is no core requirement defined for the TCI state switching, the test cases for TCI state switching requirements are not needed.

Proposal 4: RAN4 does not define test case(s) for TCI state switching requirements.

- From our perspective, scenario #2, where the UE has not had its PL offset configured, represents a traditional case and does not require the definition of new test cases. The performance considerations are limited to scenario #1 and apply to both FR1 and FR2.

Proposal 5: Performance considerations should be limited to scenario #1 and apply to both FR1 and FR2.

Note: We update our views about the asymmetric DL sTRP/UL mTRP test scope in the table, which you can find in the Annex.

- Conclusions

Proposal 1: Defined test cases for CJTC in the following aspects:

- Measurement period CJTC-Dd and CJTC-f.

- Do not define the test case for CJTC-P, as no requirement was introduced in the core part.

- Scheduling restriction.

- Verifying the accuracy of CJTC-Dd and CJTC-f.

Note: We update our views about the CJTC test scope in Table 5 – 1, which you can find in the Annex.

Observation 1: It is hard to come up with a typical value for the error budget, due to this is related to the network implementation.

Proposal 2: RAN4 to consider the following accuracy value and SNR of the CJTC-Dd and CJTC-f:

Option 1: Worst value in the simulation results.

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 39 | 0 | 15 | 1 |
| 33 | +3 |  |  |
| 104 | 0 | 30 |  |
| 68 | +3 |  |  |
| Delay offset |  |  |  |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 17 | 0 | 15 | 1 |
| 12 | +3 |  |  |
| 16 | 0 | 30 |  |
| 14 | +3 |  |  |

Option 2: Worst value in the simulation results with additional tolerances.

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 |  |  |
| 108 | 0 | 30 |  |
| 70 | +3 |  |  |
| Delay offset |  |  |  |
| Accuracy | Conditions |  |  |
| Max Estimation error | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 |  |  |
| 18 | 0 | 30 |  |
| 16 | +3 |  |  |

Proposal 3: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates.

Proposal 4: RAN4 does not define test case(s) for TCI state switching requirements.

Proposal 5: Performance considerations should be limited to scenario #1 and apply to both FR1 and FR2.

Note: We update our views about the CJTC test scope in Table 5 – 2, which you can find in the Annex.

4. References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2

[4] R4-2512130 WF on RRM requirements for NR_MIMO_Ph5_Part1

[5] R4-2512131 WF on RRM requirements for NR_MIMO_Ph5_Part2

5. Annex

Table 5-1 Views on CJTC test scope

| High level: | Huawei |
| --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y |
| Whether to define TCs to verify CJTC-f, measurement period | Y |
| Whether to define TCs to verify CJTC-P, (To be verify?) | N |
| Whether to define TCs to verify scheduling restriction | Y |
| Whether to define TCs to verify CJTC-Dd, CJTC-f, accuracy | Y |

Table 5-2 Views on asymmetric DL sTRP/UL mTRP test scope

| High level: | Huawei |
| --- | --- |
| Whether to define TCs to verify timing requirements? | Y |
| Whether to define TCs to verify TCI state switching requirements? | N |
| FR1 and/or FR2? | FR1 and FR2 for Scenario #1 |
| Whether to cover Scenario#1 and Scenario#2? | Scenarios#1 |