3GPP TSG-RAN WG4 Meeting # 116bis	 R4-2513476

Prague, Czech Republic, 13-17 October 2025

Agenda item:	7.10.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on MIMO phase 6 for Rel-20

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. RAN4 discussed the RRM impacts with objective captured in WID[1]. In this contribution, we further provide our views on the remaining issues on enhancing UL capacity and coverage and enhancing DL CSI acquisition.

Based on the latest WID [1], the objectives are shown as follows:

RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - On phase continuity, RAN4 to study and conclude by RAN#111 whether the requirements from the legacy RAN4 spec for DMRS bundling can be retained. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

- Discussion

CSI-RS overhead reduction

The intention of this objective is to improve the frequency detection granularity of SRS and achieve cross-slot multiplexing without destroying the existing SRS mechanism. At the same time, a special study is conducted on phase continuity to ensure that the new function can provide more detailed measurement capabilities in actual deployment without causing compatibility or phase error issues. In our view, no RRM impact on this UL capacity and coverage enhancement.

Proposal 1: No RRM impact on CSI-RS overhead reduction.

Early SRS/CSI/CSI-RS triggering

When a UE moves from IDLE/INACTIVE to CONNECTED mode, or when an SCell is transitioned from a deactivated or dormant state to an active state, the network must schedule DL data immediately in order to complete the connection setup. However, the UE may not have yet completed the measurement cycle required to acquire and report CSI‑RS measurements for the target SCell. Until the UE finishes the configured measCycleSCell and sends a valid CSI‑Report, the network has no prior CSI on which to base closed‑loop precoding for MIMO arrays.

Because closed‑loop precoding relies on an accurate CSI report, transmitting data with an unavailable precoding matrix can lead to severe beam mispointing and an unacceptably high block error rate. To guarantee reliable delivery during this interim period, the legacy use of a fallback transmission scheme.

Early triggering via RACH

The design of the procedure for early CSI/CSI-RS/SRS triggering via RACH will be based on the RAN1 agreements. The RAN1 agreements are shown as follows:

Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately Agreement: For early triggering of SRS-AS/CSI-RS/CSI when UE transition from IDLE to CONNECTED mode, study the following at least three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability.

In the RAN1 agreements, the procedure of the early reporting trigger is based on the 4-step RACH. The trigger action happens during the RACH procedure, and the UE will finish the report at the end of the RACH.

Observation 1: The early trigger action happens during the procedure of RACH, and the UE will finish the report at the end of the RACH.

In terms of the RAN1 agreements, this report is a periodic report and tied to the procedure of the RACH. Although the timeline of the SRS-AS transmission, aperiodic CSI-RS reception, and aperiodic CSI reporting is FFS, the report moment is fixed at the final step of RACH. Therefore, RAN4 do need to define a new RRM delay requirement for early trigger report via RACH.

Proposal 2: No need to define a new RRM delay requirement for early CSI/CSI-RS/SRS trigger report via RACH.

Early triggering via SCell activation

Similarly, the design of the early trigger procedure needs to reference the RAN1 agreements. The RAN1 agreements of the last meeting are shown as follows:

Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is associated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? FFS: Whether/what new information is provided in SCell activation command?

In the RAN1 agreements, the early report of SRS-AS and CSI/CSI-RS will be triggered based on the legacy SCell activation command, the legacy SCell procedure with three stages shown as follows:

- Stage 1: Activation indication, which includes SCell activation MAC-CE transmission and processing. The time for this stage is THARQ.

- Stage 2: SCell activation, which mainly includes DL synchronisation based on SSB and/or TRS. The time for this stage is Tactivation_time.

- Stage 3: CSI acquisition, which includes CSI measurement and reporting. The time for this stage is TCSI_Reporting.

Here is one principle that the CSI measurement and report are supposed to be conducted after the UE finishes DL synchronisation. The reason is that only after the UE achieves precise DL synchronisation, the UE can correctly demodulate the CSI‑RS and avoid channel estimation errors caused by clock deviation. Similarly, for early triggering of SRS, it is necessary to ensure that the triggered SRS transmission is conducted after the UE finishes DL synchronisation, since the UL synchronisation is based on DL synchronisation.

Observation 2: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation.

To figure out the requirement of the new early report/measurement, RAN4 needs to discuss which activation of SCell is applicable. Here is the analysis of the Rel-15 SCell activation and Rel-17 fast SCell activation.

The Rel‑15 SCell activation MAC‑CE only provides the basic SCell activation functionality. The UE’s DL synchronisation relies on measuring the SSB. In this situation, the network is unaware of the moment the UE completes DL synchronisation, because the network does not know which SSB the UE has used. Moreover, the UE may need to observe several SSB periods for AGC adjustment and for DL synchronisation, and the network has no information about the SSB periodicity at which the UE finishes this process.

In contrast, the Rel‑17 fast SCell activation assumes that the UE has already performed coarse synchronisation of the SCell and only needs to carry out precise synchronisation. To enable this, the enhanced MAC‑CE specifies a set of TRS resources for each activated SCell. Because the transmission times of the TRS are known to the network, the network can determine exactly when the UE will complete DL synchronisation.

Based on this analysis, we recommend adopting the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

Observation 3: It is possible to adopt the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

The discussion of applicable scenario(s) is unclear in RAN1. RAN4 should wait for the RAN1 progress on this.

Proposal 3: Wait for RAN1 further agreements on the early CSI/CSI-RS/SRS triggering via SCell activation.

Early triggering when switching out of SCell dormancy

We recommend waiting for the RAN1 design for the early reporting indication or CSI/SRS request. RAN4 wait for further agreement in RAN1 for early triggering when switching out of SCell dormancy.

Proposal 4: Due to no explicit product has been defined in RAN1 for early triggering when switching out of SCell dormancy, RNA4 need to wait for further agreement in RAN1.

CSI-RS ports aggregation

This objective is in the WID [1] proposes to extend CSI‑RS support from the legacy 48‑port limit to 64 and 128 ports by aggregating multiple CSI‑RS resources per carrier and by employing higher RE‑per‑RB‑per‑port densities (1/3, 1/4, 1/6, 1/8). This is achieved while fully re‑using the existing legacy CSI‑RS RE mapping and the established rate‑matching procedure. Consequently, the change introduces no impact on any RRM functionality or operation.

Proposal 5: No RRM impact on the CSI-RS ports aggregation.

- Conclusions

Proposal 1: No RRM impact on CSI-RS overhead reduction.

Observation 1: The early trigger action happens during the procedure of RACH, and the UE will finish the report at the end of the RACH.

Proposal 2: No need to define a new RRM delay requirement for early CSI/CSI-RS/SRS trigger report via RACH.

Observation 2: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation.

Observation 3: It is possible to adopt the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

Proposal 3: Wait for RAN1 further agreements on the early CSI/CSI-RS/SRS triggering via SCell activation.

Proposal 4: Due to no explicit product has been defined in RAN1 for early triggering when switching out of SCell dormancy, RNA4 need to wait for further agreement in RAN1.

Proposal 5: No RRM impact on the CSI-RS ports aggregation.

4. References

[1] RP-252510 Revised WID - NR_MIMO_Ph6