3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513492

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	4.8.2

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides analysis on the maintenance issues in core part for low band carrier aggregation via switching from RRM perspective.

Discussion

Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA - [Agreement]: - FFS this issue in maintenance stage.

Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA - Proposal 1 (Apple, Samsung/MTK(not consider colliding case)): - If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA. - Proposal 2 (LGE): - If the CQI or L1-RSRP reporting is colliding with SCell measurement during SDL SCell activation, the SCell measurement should be prioritized. - Proposal 3 (vivo): - UE is to drop invalid CQI reporting and prioritize valid CQI reporting when SSB reception is collided with CQI reporting, or UE doesn’t report invalid CQI during SCell activation. - Proposal 4 (HW): - During the SDL SCell activation - Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until Tactivation, SMTC based measurement on SDL is prioritized over transmission/reception on PCell. - During TCSI_reporting , CSI-RS based CSI measurement on SDL is prioritized over transmission/reception on PCell. - Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, SSB/CSI-RS L1-RSRP measurement on SDL is prioritized over transmission/reception on PCell. - There are no requirements for the SCell activation which requires L1 RSRP reporting, when all L1-RSRP reporting occasions are colliding with L1-RSRP measurement occasions. - Proposal 5(ZTE): - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined. - Recommended WF - [Moderator]: Check if P1 is agreeable, or discuss other proposals based on the online comments from companies.

In previous meeting, one issue regarding OOR CSI reporting and lowest valid L1 RSRP reporting during SCell activation procedure was raised. As per legacy SCell activation requirement, UE would report OOR to report CQI for the to-be-activated SCell during the time period from after activation MAC CE decoding and to the valid CQI reporting.

TS 38.133 clause 8.3.2 Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

In LB CA via switching the issue is that if the CQI OOR reporting occasion (@PCell) is colliding with SMTC occasion (@SDL carrier), what is UE behaviour.

As per the updated agreement in RAN4#116 meeting, the RRC switching pattern is to be applied during SCell activation procedure. Then either DL synchronization/AGC or CSI measurement based on CSI-RS resource shall be performed in SDL active duration, and OOR CQI and valid CQI shall be reported in PCell active duration. So the issue of CQI reporting colliding with SSB/CSI-RS of being-activated SDL SCell is invalid. Likewise, the same situation is present for lowest valid L1 SS-RSRP range reporting during SCell activation procedure as well.

Proposal 1: The case is invalid: CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation.

Issue 1-3-5: Activation of Switching Pattern [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

In RAN4#116 meeting, it is agreed that switching pattern is applied to SCell activation procedure. The motivation of the agreement is that some companies think if seme-statically configured switching pattern is activated by the DCI based aperiodic CQI reporting which is the ending point of SCell activation procedure, certain time margin will be needed for applying RRC switching pattern. The bottleneck is due to the lack of interaction between physical layer and higher layer. Based on the implementation restriction, RAN4 reverted the previous conclusion and agreed to use SCell activation command MAC CE to trigger RRC configured switching pattern in RAN4#116 meeting. In our understanding, with the updated agreement no further time margin is needed for applying the switching pattern (i.e., TLBCA =0ms), as the switch pattern is triggered by higher layer now. This is alike legacy DRX activation triggered by MAC CE, and no additional processing time for applying DRX. Furthermore as we discussed in R19 NES, there is also no additional processing time for OD-SSB activation in SCell activation procedure.

As the switching pattern is applied in SCell activation procedure, it is straight forward that UE switches to SCell or PCell according to switch pattern after n+(THARQ+3ms)/slot-length for MAC CE based SCell activation.

Proposal 2: In SCell activation procedure, no additional time is needed for applying switch pattern (i.e., TLBCA =0ms), as the RRC switching pattern has been triggered by high layer (either MAC CE based SCell activation or RRC based direct SCell activation).

Proposal 3:

For MAC CE based SCell activation UE switch to SCell or PCell according to switch pattern after n+(THARQ+3ms)/slot-length.

For RRC based direct SCell activation UE switch to SCell or PCell according to switch pattern after n+(TRRC_proc+3ms)/slot-length.

As we know, an interruption window is defined for the legacy SCell activation procedure (duplicated as below). The candidate interruption location on PCell due to RF retuning to receive SSB on SCell is between after HARQ and before the first SSB.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is: - 0, if Tactivation_time is 3 ms; - TFirstSSB, for any scenario where Tactivation_time includes TFirstSSB; - TFirstSSB_MAX, for any scenario where Tactivation_time includes TFirstSSB_MAX; - Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time includes only TFineTiming and no TFirstSSB_MAX; - Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time includes Tfirst_TRS; - Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time includes Tfirst_ATRS.

In LB-CA scenario, for the case as presented in Figure 1 (SCell activation MAC CE is received during FDD PCell duration) the instance RF retuning to switch to SDL fixedly happens on “switch from” carrier. As per RAN4 agreement, RAN4 not to define interruption/scheduling restriction due to the SDL/FDD switching.

For the case as presented in Figure 2 (SCell activation MAC CE is received during SDL duration), UE can retune RF chain to receive SSB on SDL SCell after slot Tharq+3ms and before the first SSB on SCell. It shall be noted that there is no interruption on PCell, as there is no data on PCell at this moment.

Based on the above analysis, the current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario. There is no interruption during SCell activation in LB-CA.

Figure 1. SCell activation MAC CE is received during FDD PCell duration

Figure 2. SCell activation MAC CE is received during SDL duration

Proposal 4: Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

The switching pattern is NOT applied after UE completed the SCell deactivation. It is alike the procedure UE quits the DRX activation. We don’t think there is additional processing time for deactivating the switching pattern, i.e., TLBCA =0ms.

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms)/slot-length), TLBCA =0ms.

After UE completed the SCell deactivation, UE shall switch to PCell immediately, otherwise additional interruption would be caused. We think it is common understanding, and no spec impact is observed.

Proposal 6: After UE completed the SCell deactivation, UE shall switch to PCell immediately otherwise additional interruption would be caused. No need to capture it in spec.

Conclusions

This contribution provides further analysis on low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

Proposal 1: The case is invalid: CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation.

Proposal 2: In SCell activation procedure, no additional time is needed for applying switch pattern (i.e., TLBCA =0ms), as the RRC switching pattern has been triggered by high layer (either MAC CE based SCell activation or RRC based direct SCell activation).

Proposal 3:

For MAC CE based SCell activation UE switch to SCell or PCell according to switch pattern after n+(THARQ+3ms)/slot-length.

For RRC based direct SCell activation UE switch to SCell or PCell according to switch pattern after n+(TRRC_proc+3ms)/slot-length.

Proposal 4: Not to define interruption requirements during SCell activation in LB-CA. Make a clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario.

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms)/slot-length), TLBCA =0ms.

Proposal 6: After UE completed the SCell deactivation, UE shall switch to PCell immediately otherwise additional interruption would be caused. No need to capture it in spec.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple