3GPP TSG-RAN WG4 #116bis	R4-2513551

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.12.2.1		Enhancement for UE-initiated/event-driven beam management

Source:	Qualcomm Incorporated

Title:	On performance requirements for UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #115 the work on RRM performance requirements for MIMO Phase 5 has been started. In this paper we provide our view on RRM performance requirements for UE-initiated/event-driven beam management identified in [1] in RAN4 #116.

# Discussion

#### Issue 2-1-1: Test scope

<Way forward >

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

Regarding the test scope, the following table has been collected in RAN4 #116.

Our updated view is as follows:

Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed.

Regarding inter-cell beam management based on CSI-RS, it is still our understanding that it is not very clear whether there are core requirements for this case. Hence, we prefer not to define performance requirements.

Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management.

In RAN4 #116, the following agreement was reached, i.e., no new TCI switching requirements will be defined in Rel-19.

#### Issue 1-2: Whether to specify new unified TCI switching delay requirements?

Agreement:

- This issue is closed.

Hence, we do not think that it is needed to define performance tests for TCI state switching.

Proposal 3: No performance requirements for TCI state switching should be defined.

Regarding the window-based approach, it was agreed in RAN4 #116 that core requirements will be defined in TS 38.133. However, we do not see a need to define performance requirements.

Proposal 4: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed.

Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management.

Proposal 3: No performance requirements for TCI state switching should be defined.

Proposal 4: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined.

# References

- R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #116, August 2025