3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513557

Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on RRM enhancement phase 6

Agenda item:	7.4.3.2

Document for:	Discussion

Introduction

In agreed WID for Rel-20 RRM enhancement phase 6 [1], following objective was captured:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

In this contribution, we provide our initial views on RRM core requirements for Rel-20 RRM enhancement phase 6

Discussion

As stated in WID[2], the relaxed requirements for RLM/BFD in Release 17 and the less-than-5MHz channel bandwidth requirement for non-RedCap UEs in Release 18 can be a baseline. The motivation for this working item is to extend these existing features to RedCap UEs, thereby addressing the current gap in RedCap UE requirements regarding supporting for relaxed RLM/BFD and operation with channel bandwidths less than 5MHz. Based on our understanding, the following table provides an assessment of the specification impact.

Table 1 Assesment of the specfication impact

| Requirements for Non-RedCap UE |  | RedCap UE | Spec impact |
| --- | --- | --- | --- |
| 8.1 Radio link monitoring | 8.1.1 Introduction | Reuse | No |
|  | 8.1.1.1 Introduction of relaxed measurement Criteria | Need to specify relaxed measurement criteria | Yes |
|  | 8.1.2 Requirements for SSB based radio link monitoring 8.1.2.1 Introduction | Need to specify PDCCH transmission parameters for a RedCap UE on lessthan 5MHz | Yes |
|  | 8.1.2.2 Minimum requirement for SSB | Reuse | No |
|  | 8.1.2.3 Measusrement restriction for SSB | Reuse | No |
|  | 8.1.2.4 Minimum requirement for relaxed measurement for SSB | Need to specify relaxed measurement requirement | Yes |
|  | 8.1.3 Requirements for CSI-RS based radio link monitoring 8.1.3.1 Introduction | Reuse | No |
|  | 8.1.3.2 Minimum requirement for CSI-RS | Reuse | No |
|  | 8.1.3.3 Measusrement restriction for CSI-RS | Reuse | No |
|  | 8.1.3.4 Minimum requirement for relaxed measurement for CSI-RS | Need to specify relaxed measurement requirement | Yes |
| 8.5 Link recovery procedure | 8.5.1 Introduction | Reuse | No |
|  | 8.5.1.1 Introduction of relaxed measurement Criteria | Need to specify relaxed measurement criteria | Yes |
|  | 8.5.2 Requirements for SSB based BFD 8.5.2.1 Introduction | Need to specify PDCCH transmission parameters for a RedCap UE on lessthan 5MHz | Yes |
|  | 8.5.2.2 Minimum requirement for SSB based BFD | Reuse | No |
|  | 8.5.2.3 Measusrement restriction for SSB based BFD | Reuse | No |
|  | 8.5.2.4 Minimum requirement for relaxed measurement for SSB based BFD | Need to specify relaxed measurement requirement | Yes |
|  | 8.5.3 Requirements for CSI-RS based BFD 8.5.3.1 Introduction | Reuse | No |
|  | 8.5.3.2 Minimum requirement for CSI-RS based BFD | Reuse | No |
|  | 8.5.3.3 Measusrement restriction for CSI-RS based BFD | Reuse | No |
|  | 8.5.3.4 Minimum requirement for relaxed measurement for CSI-RS based BFD | Need to specify relaxed measurement requirement | Yes |

Proposal 1: RAN4 to specify following requirement of non-RedCap UEs for (e)RedCap UEs for relaxed RLM/BFD and lessthan 5MHz channel bandwidth considering both 1Rx and 2Rx, and HD-FDD

- 8.1 Radio link monitoring

  - 8.1.1.1 Introduction of relaxed measurement criteria

  - 8.1.2.1 Introduction for SSB based radio link monitoring

  - 8.1.2.4 Minimum requirement of SSB based RLM for relaxed measurement criteria

  - 8.1.3.4 Minimum requirement of CSI-RS based RLM for relaxed measurement criteria

- 8.5 Link recovery procedure

  - 8.5.1.1 Introduction of relaxed measurement criteria

  - 8.5.2.1 Introduction for SSB based BFD

  - 8.5.2.4 Minimum requirement of SSB based BFD for relaxed measurement criteria

  - 8.5.3.4 Minimum requirement of CSI-RS based BFD for relaxed measurement criteria

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to specify following requirement for (e)RedCap UEs for relaxed RLM/BFD and lessthan 5MHz channel bandwidth

- 8.1 Radio link monitoring

  - 8.1.1.1 Introduction of relaxed measurement criteria

  - 8.1.2.1 Introduction for SSB based radio link monitoring

  - 8.1.2.4 Minimum requirement of SSB based RLM for relaxed measurement criteria

  - 8.1.3.4 Minimum requirement of CSI-RS based RLM for relaxed measurement criteria

- 8.5 Link recovery procedure

  - 8.5.1.1 Introduction of relaxed measurement criteria

  - 8.5.2.1 Introduction for SSB based BFD

  - 8.5.2.4 Minimum requirement of SSB based BFD for relaxed measurement criteria

  - 8.5.3.4 Minimum requirement of CSI-RS based BFD for relaxed measurement criteria

Reference

[1] RP-252955, “WID: NR Radio Resource Management (RRM) Phase 6,” Apple