3GPP TSG-RAN4 Meeting #116bis	R4-2513565

# Prague, CZ, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements for PC1.5 NB-Iot based Iot-NTN HPUE are still absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements for PC1.5 NB-Iot based Iot-NTN were introduced. The applicable bands are band 255 and 256. MPR/A-MPR requirements were added. The configurated transmitted power are unchanged. To introduce the NS_02N additional spectrum emissions requirements for PC1.5 for category NB1/NB2 The Rx requirements are unchanged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for PC1.5 NB-Iot based Iot-NTN HPUE are still absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B, 6.5B.3.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.521-4 CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.2B	Transmit power for category NB1 and NB2

6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 29 | [+2/-3] | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 29 | [+2/-3] | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  |  |  |  |  | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| E-UTRA Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
|  | NS_11N | 0.2 | 1610 - 1618.25 | 15dBm/4kHz (peak EIRP limit) |
|  | NS_12N | 0.2 | 1618.25 - 1626.5 |  |

6.2B.2	UE maximum output power reduction for category NB1 and NB2

For UE category NB1 and NB2, the allowed Maximum Power Reduction (MPR) for the maximum output power are specified in following tables.

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0 | 1-10 | 11 | 0-3 | 4-43 | 44-47 |
| MPR | ≤ 0.5 dB | 0 dBNote 2 | ≤ 0.5 dB | ≤ 0.5 dB | 0 dBNote 2 | ≤ 0.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For tone positions 3 to 8 for 15kHz SCS and 8 to 39 for 3.75 kHz, 1dB MPR applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Table 6.2B.2-3a: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | [0-1] | [2-9] | [10-11] | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | 0 dB | ≤ [0] dB | ≤ [1.5] dB | 0 dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-3b: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 2 dB | 0 dB | ≤ 2 dB | ≤ 3.5 dB | 0 dB | ≤ 3.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 2 dB | ≤ 1 dB |  | ≤ 2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 2.5dB |  | ≤ 2.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 4 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

For the UE maximum output power modified by MPR, the power limits specified in sub-clause 6.2B.4 apply.

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE PC3/PC5 and PC1/PC2, the specific requirements and identified subclauses are specified in Table 6.2B.3-1 and 6.2B.3-2 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC3 and PC5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 |  |  |
| NS_05N | 6.5B.4.4.6 | 254 |  |  |
| NS_06N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_11N | 6.5B.4.4.5 | 254 |  |  |
| NS_12N | 6.5B.4.4.6 | 254 |  |  |

Table 6.2B.3-2: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1, PC1.5 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC1 | PC1.5 | PC2 |
|  |  |  | ≤ 10 | ≤ [8.5] | ≤ 5.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |  |

< Unchanged sections omitted >

6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

6.5B.3.3.1	Requirements for network signalling value "NS_02N”

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.1-1.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | 6 for PC1 4 for PC1.5 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -4 for PC1 -6 for PC1.5 -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC1, PC1.5, PC2, PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

< End of changes >