# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513646

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.17.2.2

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for CSI-RS based L1 measurement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there is discussion for RRM performance requirements for NR mobility enhancements, and a WF was approved [3]. This contribution provides discussion on RRM performance requirements for CSI-RS based L1 measurement.

- Discussion

For CSI-RS based L1-RSRP measurement on neighbor cell, it was agreed to define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs. As for side condition for CSI-RS based L1 measurement for neighbor cell, it was agreed to use legacy side condition of SINR=-3dB. It can be seen that the BW, density, and side condition are same are same as that for legacy CSI-RS based L1-RSRP measurement are for serving cell. And in last meeting, it was agreed not to define RAN4 requirements for RTD > CP. Taking above into account, in our view, the legacy accuracy requirements can be used as baseline to define accuracy requirements for CSI-RS based LTM candidate cell measurements. In last meeting, it was agreed that CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell under certain conditions.

- Issue 4-2-1: CSI-RS based L1 measurement accuracy Agreement: - CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

As for the test cases for CSI-RS based L1-RSRP measurement on neighbor cell, there is no detailed discussion in last meeting, the candidate options are duplicated as below.

- Issue 4-2-2: test case list for CSI-RS based L1 measurement Candidate options - Option 1: Nokia - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1 - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1 - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping - Option 2: CATT - RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2. - Option 3: Apple, OPPO, [vivo?] - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 4: MTK - CSI-RS based L1 RSRP measurement for neighbour cell in FR1 - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 5: CMCC - For CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. - Option 6: HW - As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement. - In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. - FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified. - Option 7: E/// Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells,. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated.

In general, both FR1 and FR2 are considered for core requirements specification. Considering different operator may have different deployment, to guarantee the performance, both FR1 and FR2 need to be considered.

Proposal 1: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for both FR1 and FR2.

According to previous agreements, both FR1 and FR2-1 are in scope of RRM requirements for CSI-RS based L1-RSRP measurement on LTM candidate cell(s). CSI-RS based L1 measurement without gap is supported. From RAN4 perspective, not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell.

- The agreements on measurement procedure for CSI-RS based LTM candidate cell measurements are summarized as following:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Depending on whether step 2 is skipped, the requirements in step 3 may be different.

It can be seen that whether step 2 is removed or not has impact on CSI-RS based L1-RSRP measurement period. Without step 2, UE Rx beam sweeping factor is included in the CSI-RS based L1-RSRP measurement period. With step 2, UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period. Since UE behaviour is different, it is necessary to define test cases to cover both cases.

Proposal 2: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Conclusion

This contribution provides discussion on RRM performance requirements for CSI-RS based L1 measurement. The proposals are:

Proposal 1: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for both FR1 and FR2.

Proposal 2: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple