# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513656

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.4

Source: 	CMCC

Title: 			Discussion on requirements for two-sided AI/ML models

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements was approved [1]. The objectives for RAN4 are duplicated as following.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

This contribution provides discussion on requirements for two-sided AI/ML models.

- Discussion

For CSI compression using two-sided model, the temporal-domain sub-use cases in following Table1 are adopted in Release-19 study. It is noted that for the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them. “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well. According to the WID, case 0 (CSI spatial/frequency compression without temporal aspects) is the focus.

Table 1: sub-use cases for CSI feedback using two-sided model

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the NW uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

One of the objectives is to define necessary RRM requirement for specified LCM procedures. The discussion and agreements on LCM in Rel-19 can be used as reference. Below is summary of Rel-19 agreements on LCM.

RAN4 Agreements on LCM in Rel-19 Agreements in RAN4#111 meeting (2401570) Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded Agreements in RAN4#114bis meeting (R4-2505107) Delay requirements for activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting Agreements in RAN4#115 meeting (R4-2508051) Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring Agreements in RAN4#116 meeting (R4-2511890 & R4-2511849) General LCM principles: Agreement: - No need to create deactivation requirements for AI functionality LCM for beam management: Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. No agreements on performance monitoring accuracy. RRM core requirement and testing framework for beam management Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

LCM procedures include activation, deactivation, switch, fallback, etc. In Rel-19, it was agreed to deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement. AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference. Functionality deactivation refers to the process of network deactivating an active functionality. For CSI compression, it is proposed to follow Rel-19 agreements and firstly focus on activation/deactivation delay requirement.

Proposal 1: for CSI feedback enhancement (two-sided model) use case, the discussion on LCM can firstly focus on activation/deactivation delay requirement.

For activation part, it was agreed for beam prediction that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. Inference day will be included in the inference reporting delay. We believe these agreements also apply to CSI prediction, since RAN2 agreed that applicability reporting procedures for CSI prediction are the same as for beam management. However, whether the agreements can be reused for CSI compression with two-sided model need further discuss, since RAN2 design on LCM is not clear

Similar situation for deactivation procedure. It was agreed in Rel-19 that no need to create deactivation requirements for AI functionality. However, for CSI feedback enhancement (two-sided model) use case, whether  Rel-19 agreements can be reused need further discussion based on RAN2 design.

Proposal 2: for CSI feedback enhancement (two-sided model) use case, the activation/deactivation delay requirements can be further discussed based on RAN2 design on LCM.

For CSI prediction, the agreement on CSI reporting delay in last meeting is to wait for RAN1 decision. In RAN1 #121 meeting, it was agreed that for CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS. Based on RAN1 agreements, the inference reporting delay shall be at least legacy Z/Z’ plus t, t is reported by UE.

For Beam prediction, for aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay, and the reporting delay includes the inference delay. For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion.

For positioning case 1, it is agreed to define reporting delay requirements, and the reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

For CSI compression case 0, CSI reporting delay requirements can be considered. But the details are pending on RAN1 design.

Proposal 3: For CSI feedback enhancement (two-sided model) use case, the prediction reporting delay can be further discussed.

For case 0, the output is reconstructed precoding matrix in the spatial-frequency. In Rel-19, it was agreed that PMI requirements based on AI/ML CSI prediction are defined for CSI prediction  (i.e. Time domain CSI prediction using UE-side model). The metric/KPI for CSI prediction can be used as baseline to discuss the metric/KPI for CSI compression case 0. 	Following is the summary of agreements for CSI prediction in Rel-19.

RAN4 Agreements for CSI prediction Agreements in RAN4#110 meeting (R4-2403712) Issue 4-1: CSI Prediction Accuracy metrics - Proposals - Option 1: Prediction accuracy can be used as KPI/metric - Option 2: Prediction accuracy cannot be used because the “correct” value is not available - Option 3: Throughput should be the default metric, others should be discussed only if throughput is not feasible - Option 4: Others Agreement: - Agree option 3 for inference only. TBD whether we use relative or absolute throughput. - Monitoring will be discussed separately. Agreements in RAN4#112 meeting (R4-2414323) Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to Agreements in RAN4#113 meeting (R4-2420513) Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition Agreements in RAN4#114 meeting (R4-2502856) Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: - Postpone this discussion until the requirement and test definition is clear Agreements in RAN4#114bis meeting(R4-2505105) Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4 Agreements in RAN4#115 meeting(R4-2508080) Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS Agreements in RAN4#116 meeting(R4-2511890) Issue 1-5: Scheduling delay Agreement: - The delay from when the UE prediction is send until the time the TE applies is n+4 (slots) for FDD Agreements in ad-hoc session (R4-2511891) - Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this Issue 1-4: Reporting delay requirement Agreement: wait for RAN1 decision

For legacy PMI reporting, the requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively.

For CSI prediction, it was agreed that throughput should be the default metric for inference. And relative throughput is agreed as requirement metric for CSI prediction, which is the ratio of throughput obtained by using the precoders configured according to the predicted PMI reported by UE, and the throughput obtained by using the random precoding. The reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook. Rel-18 eType II Doppler codebook is used for AI/ML based CSI predictions.

For CSI compression case 0, it is propose to use relative throughput as KPI/metric for inference, similar as that for CSI prediction.

Proposal 4: For CSI feedback enhancement (two-sided model) use case, it is proposed to use relative throughput as KPI/metric for inference, similar as that for CSI prediction.

- Conclusion

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models. The proposals are:

Proposal 1: for CSI feedback enhancement (two-sided model) use case, the discussion on LCM can firstly focus on activation/deactivation delay requirement.

Proposal 2: for CSI feedback enhancement (two-sided model) use case, the activation/deactivation delay requirements can be further discussed based on RAN2 design on LCM.

Proposal 3: For CSI feedback enhancement (two-sided model) use case, the prediction reporting delay can be further discussed.

Proposal 4: For CSI feedback enhancement (two-sided model) use case, it is proposed to use relative throughput as KPI/metric for inference, similar as that for CSI prediction.

- Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R4-2503953, WF on AI/ML air interface study

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface