3GPP TSG-RAN WG4 Meeting #116bis		R4-2513684

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.7.3

Source: 	CMCC

Title: 			 Discussion on test cases for R19 ATG

Document for:	Discussion

- Introduction

In last meeting, the test scope was discussed and partially agreed. The WF has been approved in [1].

Issue 2-1-3: Test scope Agreement: - Introduced test cases: - Interruptions during measurements on deactivated NR SCC in FR1 - test together with Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms - If it is feasible to test both neighbour cell measurement skipping and not skipping in this test case, - not introduce the test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”. - If not feasible, - introduce one more test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle” - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification) - Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification) - Direct SCell activation at handover with known SCell in FR1 - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX - Only consider periodic TRS for this test case - This test case is only applicable to case 4 UE - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

In this contribution, we further provide our views on test setups for ATG.

- Discussion

  - ATG CA test cases

Event triggered reporting on SCC with deactivated SCell test

In last meeting, following way forward has been agreed for Event triggered reporting on SCC with deactivated SCell test

- Interruptions during measurements on deactivated NR SCC in FR1 - test together with Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms - If it is feasible to test both neighbour cell measurement skipping and not skipping in this test case, - not introduce the test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”. - If not feasible, - introduce one more test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”

Firstly, we discuss the feasibility of test both neighbour cell measurement skipping and not skipping in one test case, i.e. Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms.

We find several test configurations which are different between test of neighbour cell measurement skipping and not skipping:

- Test scenario

- The neighbour cell measurement skipping scheme will only be used in inter-band CA scenario

- UE type

- The neighbour cell measurement skipping scheme will only be applied for UE capable of antenna array on SCC

- Reporting event

When network doesn’t indicate the neighbour cell measurement skipping, the test procedure in Clause A.6.6.1.8 TS38.133 can be taken as the reference, that Event A6 (Neighbour becomes offset better than SCell) is used.

When network indicate the neighbour cell measurement skipping, since UE can only acquire the SCell measurement result, the Event A1 (Serving becomes better than threshold) shall be used.

- Considering these differentiates, we think it is not reasonable to merge both neighbour cell measurement skipping and not skipping scheme in one test case, therefore, we propose to define two separate test cases as below:

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (together with interruptions during measurements on deactivated NR SCC in FR1 )

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (neighbour cell measurement skipping is indicated)

Proposal 1: For Event triggered reporting on SCC with deactivated SCell test, define two separate test cases as below:

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (together with interruptions during measurements on deactivated NR SCC in FR1 )

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (neighbour cell measurement skipping is indicated)

Secondly, we discuss the specific test configuration, test procedure and test requirement for these two test cases.

For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except for highSpeedMeasCA-Scell-r17 indication, DRX configuration, Initial/Active UL BWP configuration, propagation condition and test requirements. No highSpeedMeasCA-Scell-r17 indication configured, the DRX configuration shall be OFF, the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A, the propagation condition shall follow R18 ATG test cases. PCell shall be continuously scheduled in DL. The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2, and include the interruption requirement.

Proposal 2: For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the propagation condition shall follow R18 ATG test cases. PCell shall be continuously scheduled in DL

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2, and include the interruption requirement

For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except for highSpeedMeasCA-Scell-r17 indication, DRX configuration, Initial/Active UL BWP configuration, propagation condition, measCycleSCell configuration, neighbour cell measurement skipping indication and test requirements. No highSpeedMeasCA-Scell-r17 indication configured, the DRX configuration shall be OFF, the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A, the measCycleSCell shall be configured as 160ms, the neighbour cell measurement skipping indication shall be configured, the propagation condition shall follow R18 ATG test cases. The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2.

Proposal 3: For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the measCycleSCell shall be configured as 160ms

- the neighbour cell measurement skipping indication shall be configured

- the propagation condition shall follow R18 ATG test cases

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2.

Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

For the test of Interruption requirement at SRS antenna port switching, the existing test case defined in Clause A.6.5.2.3 and A.6.5.2.4 can be taken as the reference respectively, except that only DL interruption will be considered in test environment and test requirement. Uplink initial BWP configuration and Uplink dedicated BWP configuration for SCell shall be N/A, SRS configuration for SCell shall be N/A, propagation condition shall follow R18 ATG test cases.

Proposal 4: For Interruption requirement at SRS antenna port switching, the test in Clause A.6.5.2.3 and A.6.5.2.4 TS38.133 can be taken as the reference respectively, except that:

- only DL interruption will be considered in test environment and test requirement

- Uplink initial BWP configuration and Uplink dedicated BWP configuration for SCell shall be N/A

- SRS configuration for SCell shall be N/A

- propagation condition shall follow R18 ATG test cases

SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle

Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle

SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle

For normal SCell activation and deactivation, Direct SCell activation at SCell addition, fast SCell activation, and SCell activation with valid L3 measurement results test case, the existing test case defined in Clause A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, A.6.5.3.4, A.6.5.3.10, A.6.5.3.11 and A.6.5.3.14 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases.

Proposal 5: For normal SCell activation and deactivation, Direct SCell activation at SCell addition, fast SCell activation, and SCell activation with valid L3 measurement results test case, the existing test case defined in Clause A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, A.6.5.3.4, A.6.5.3.10, A.6.5.3.11 and A.6.5.3.14 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases.

Direct SCell activation at handover with known SCell in FR1

In legacy, the scenario ‘handover without SCell change’ is tested. However, we think it is not the common scenario in ATG, therefore, we propose to update the test scenario as:

At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Proposal 6: Update the test scenario for Direct SCell Activation at Handover test to:

- At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

- Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

- Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

For TRS based SCell Activation of SSB-less SCell, the existing test case defined in Clause A.6.5.3.15 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases and an additional applicability rule shall be added that only applicable for UE support one common Rx beam.

Proposal 7: For TRS based SCell Activation of SSB-less SCell, the existing test case defined in Clause A.6.5.3.15 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases and an additional applicability rule shall be added that only applicable for UE support one common Rx beam.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: For Event triggered reporting on SCC with deactivated SCell test, define two separate test cases as below:

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (together with interruptions during measurements on deactivated NR SCC in FR1 )

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (neighbour cell measurement skipping is indicated)

Proposal 2: For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the propagation condition shall follow R18 ATG test cases. PCell shall be continuously scheduled in DL

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2, and include the interruption requirement

Proposal 3: For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

- No highSpeedMeasCA-Scell-r17 indication configured

- the DRX configuration shall be OFF

- the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

- the measCycleSCell shall be configured as 160ms

- the neighbour cell measurement skipping indication shall be configured

- the propagation condition shall follow R18 ATG test cases

- The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2.

Proposal 4: For Interruption requirement at SRS antenna port switching, the test in Clause A.6.5.2.3 and A.6.5.2.4 TS38.133 can be taken as the reference respectively, except that:

- only DL interruption will be considered in test environment and test requirement

- Uplink initial BWP configuration and Uplink dedicated BWP configuration for SCell shall be N/A

- SRS configuration for SCell shall be N/A

- propagation condition shall follow R18 ATG test cases

Proposal 5: For normal SCell activation and deactivation, Direct SCell activation at SCell addition, fast SCell activation, and SCell activation with valid L3 measurement results test case, the existing test case defined in Clause A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, A.6.5.3.4, A.6.5.3.10, A.6.5.3.11 and A.6.5.3.14 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases.

Proposal 6: Update the test scenario for Direct SCell Activation at Handover test to:

- At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

- Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

- Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Proposal 7: For TRS based SCell Activation of SSB-less SCell, the existing test case defined in Clause A.6.5.3.15 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases and an additional applicability rule shall be added that only applicable for UE support one common Rx beam.

- Reference

- R4-2512122, WF on RRM requirements for NR_ATG_enh, ZTE, CMCC