3GPP TSG-RAN WG4 Meeting #116bis		R4-2513686

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.7.4

Source: 	CMCC

Title: 			 Simulation results for Rel-19 ATG

Document for:	Discussion

- Introduction

Based on the agreed simulation assumption approved in last meetings [1], we give our simulation results and observations.

- Simulation assumptions and results for CA

Simulation assumption

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30kHz/5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100MHz FDD: 15kHz/5, 10, 15, 20, 25, 30, 35, 40, 45, 50MHz |
| MIMO layer and Antenna configuration |  |  | Rank 1: 2T2R 2T4R |
| Propagation condition and antenna correlation |  |  | AWGN+220Hz for FDD 15kHz AWGN+500Hz for TDD 30kHz |
| MCS |  |  | 64QAM Table 1 MCS 22 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 30kHz SCS and 5MHz bandwidth: 1/AL2 Others: 1/AL8 |
| Number of HARQ Processes |  |  | 4,As defined in Table 5.2A-2, TS38.101-4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3, TS38.101-4 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases where the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. PUCCH format 3 for cases where the number of ACK/NACK to be transmitted on single PUCCH is more than 2. |

Simulation results for FDD 15kHz SCS

| Antenna config | Bandwidth/MHz | SNR at 70% TP/dB |
| --- | --- | --- |
| 2T2R | 5 | 10.3 |
|  | 10 | 10.2 |
|  | 15 | 10.1 |
|  | 20 | 10.1 |
|  | 25 | 10.2 |
|  | 30 | 10.3 |
|  | 35 | 10.1 |
|  | 40 | 10.1 |
|  | 45 | 10.1 |
|  | 50 | 10.3 |
| 2T4R | 5 | 7.3 |
|  | 10 | 7.2 |
|  | 15 | 7.1 |
|  | 20 | 7.0 |
|  | 25 | 7.2 |
|  | 30 | 7.3 |
|  | 35 | 7.1 |
|  | 40 | 7.1 |
|  | 45 | 7.1 |
|  | 50 | 7.3 |

Simulation results for TDD 30kHz SCS

| Antenna config | Bandwidth/MHz | SNR at 70% TP/dB |
| --- | --- | --- |
| 2T2R | 5 | 10.0 |
|  | 10 | 10.2 |
|  | 15 | 10.3 |
|  | 20 | 10.2 |
|  | 25 | 10.0 |
|  | 30 | 10.3 |
|  | 40 | 10.0 |
|  | 50 | 10.2 |
|  | 60 | 10.1 |
|  | 80 | 10.3 |
|  | 90 | 10.2 |
|  | 100 | 10.3 |
| 2T4R | 5 | 7.0 |
|  | 10 | 7.2 |
|  | 15 | 7.3 |
|  | 20 | 7.2 |
|  | 25 | 7.0 |
|  | 30 | 7.3 |
|  | 40 | 7.0 |
|  | 50 | 7.2 |
|  | 60 | 7.1 |
|  | 80 | 7.3 |
|  | 90 | 7.1 |
|  | 100 | 7.1 |

- Simulation assumptions and results for DL-MIMO

Simulation assumption

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD pattern |  |  | N/A | 7D1S2U S=6D:4G:4U |
| Correlation matrix and antenna configuration |  |  | 2T2R XP-Low 2T4R XP-Low | 2T2R XP-Low 2T4R XP-Low |
| Rank |  |  | 2 | 2 |
| MCS |  |  | 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 | 64QAM Table 1 MCS 22 256QAM Table 2 MCS 24 |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Simulation results

| Duplex mode | Antenna configuration | MCS | SNR at 70% TP/dB |
| --- | --- | --- | --- |
| FDD | 2T2R | 64QAM Table 1 MCS 22 | 14.3 |
|  |  | 256QAM Table 2 MCS 24 | 22.8 |
|  | 2T4R | 64QAM Table 1 MCS 22 | 11.3 |
|  |  | 256QAM Table 2 MCS 24 | 19.8 |
| TDD | 2T2R | 64QAM Table 1 MCS 22 | 14.2 |
|  |  | 256QAM Table 2 MCS 24 | 22.9 |
|  | 2T4R | 64QAM Table 1 MCS 22 | 11.1 |
|  |  | 256QAM Table 2 MCS 24 | 19.9 |

- Simulation assumptions and results for UL-MIMO

Simulation assumption

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| Duplex mode |  | FDD | TDD |
| SCS and bandwidth |  | 15kHz/5MHz | 30kHz/10MHz |
| Channel model |  | AWGN+200Hz | AWGN+500Hz |
| TDD pattern |  | N/A | 7D1S2U, S=6D:4G:4U |
| Correlation matrix and antenna configuration |  | 2T2R XP-Low |  |
| Rank |  | 2 |  |
| MCS |  | 64QAM Table 1 MCS 20 |  |
| Transform precoding |  | CP-OFDM |  |
| Mapping Type |  | Type A |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | {0} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type | A |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |

Simulation results

| Duplex mode | Antenna configuration | MCS | SNR at 70% TP/dB |
| --- | --- | --- | --- |
| FDD | 2T2R | 64QAM Table 1 MCS 22 | 12.5 |
| TDD | 2T2R | 64QAM Table 1 MCS 22 | 12.8 |

- Reference

- R4-2512602, Way Forward for [116][326] NR_ATG_enh_demod, ZTE Corporation, CMCC