3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513690

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.12.2

Source: 	National Taiwan University

Title: 	Cross-Frequency Channel Correlation Modelling for Inter-Frequency RSRP Prediction

Document for:	Approval

Introduction

In this contribution, we share our view on inter-frequency measurement prediction.

Discussion

Inter-Frequency Received Power Channel Modeling

In order to properly evaluate inter-frequency RSRP prediction to define the proper requirement that can reflect the field performance, we need a precise channel model to capture the correlations of channels across different frequencies. Reviewing the existing channel model in TR 38.901, we found that center frequency is taken into account in:

- Pathloss modeling, e.g.,

- Path distribution related parameters in Table 7.5-6, e.g.,

| Scenarios |  | UMi - Street Canyon |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | mlgDS | -0.24 log10(1+ fc) - 7.14 | -0.24 log10(1+ fc) - 6.83 | -6.62 |
|  | slgDS | 0.38 | 0.16 log10(1+ fc) + 0.28 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | mlgASD | -0.05 log10(1+ fc) + 1.21 | -0.23 log10(1+ fc) + 1.53 | 1.25 |
|  | slgASD | 0.41 | 0.11 log10(1+ fc) + 0.33 | 0.42 |

- Antenna array location/velocity related impact through λ0 in clause 7.5 in step 11, e.g.,

However, besides the above factors, the phase changes on the propagation through the channel (besides antenna array impact) are different across different central frequencies, and this can contribute to the correlation across channels on different frequencies. Such impact is missing from current channel modeling methodology in TR 38.901 for both system level simulations, and link level simulations.

Proposal 1: Study how to capture cross-frequency correlation of phase and other impact on the propagation channel.

Conclusion

Proposal 1: Study how to capture cross-frequency correlation of phase and other impact on the propagation channel.