# 3GPP TSG-RAN WG4 Meeting #116bis		  R4-2513695

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.5.2

Source:		Qualcomm Incorporated

Title:			UE demodulation requirements for 5G broadcast over GSO

Document for:	Approval

# 1  Introduction

The core part of WI on new LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite was competed in RAN#109 [1] [2].

Following WID in [3], RAN4 will start the performance part from RAN4#116bis meeting with the following objectives for UE demodulation requirements.

RAN4 to specify the following performance requirements related to above core part objectives: - Specify related UE demodulation requirements - Reuse existing UE PMCH demodulation requirements for 10MHz of terrestrial LTE-based 5G broadcast

In this paper, we provide our views on UE demodulation requirements for 5G broadcast over GSO.

# 2  PMCH demodulation requirements

## 2.1 Test parameters

Proposal 1: The following test parameters can be used to define PMCH demodulation requirements.

- SCS: 15kHz

- Channel bandwidth: 10MHz

- Modulation: Rank 1 with QPSK, Rank 1 with 16QAM

- Antenna configuration: 1x2 low

- BLER test point: 1% of BLER.

- Other test parameters in Table 1

Table 1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Bandwidth |  | MHz | 10MHz |
| Note 1: . |  |  |  |

## 2.2 FMC

Proposal 2: The following fixed reference channel in Table 2 for 15kHz subcarrier spacing for MBMS dedicated cell can be used for 5G broadcast over GSO UE demodulation requirements definition.

Table 2 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.x.1 | R.x.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

## 2.3	Propagation condition

MBSFN channel model specified in Table B.2.6.3-1 of TS 36.101 is used as propagation condition for 5G terrestrial broadcast demodulation requirements definition. While MBSFN propagation condition is not applicable for GSO 5G broadcast over GSO. Therefore, the channel model for 5G broadcast over GSO demodulation requirements needs to be discussed and defined.

Proposal 3: RAN4 to use the following NTN-TDLC (DS=5ns) channel model parameters as the propagation condition for 5G broadcast over GSO demodulation requirements definition.

Table 3: Delay profiles for 5G broadcast over GSO channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table 4 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

Table 5: Channel model parameters for 5G broadcast over GSO channel models

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |

# 3. Conclusion

In summary, we have following proposals.

Proposal 1: The following test parameters can be used to define PMCH demodulation requirements.

- SCS: 15kHz

- Channel bandwidth: 10MHz

- Modulation: Rank 1 with QPSK, Rank 1 with 16QAM

- Antenna configuration: 1x2 low

- BLER test point: 1% of BLER.

- Other test parameters in Table 1

Table 1: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Bandwidth |  | MHz | 10MHz |
| Note 1: . |  |  |  |

Proposal 2: The following fixed reference channel in Table 2 for 15kHz subcarrier spacing for MBMS dedicated cell can be used for 5G broadcast over GSO UE demodulation requirements definition.

Table 2 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.x.1 | R.x.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Proposal 3: RAN4 to use the following NTN-TDLC (DS=5ns) channel model parameters as the propagation condition for 5G broadcast over GSO demodulation requirements definition.

Table 3: Delay profiles for 5G broadcast over GSO channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table 4 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

Table 5: Channel model parameters for 5G broadcast over GSO channel models

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |

# 4  References

- RP-251943, Status Report for New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, RAN#109, September 2025

- RP-252885, Summary for WI New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, RAN#109, September 2025

- RP-251419, Revised WID: New LTE band for 5G broadcast for region 3 utilizing geosynchronous satellite, RAN#108, June, 2025.