3GPP TSG-RAN WG4 Meeting #116bis	R4- 2513696

# Prague, Czech Republic, Oct. 13 – 17, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for SBFD

Agenda item:	4.19.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on the RACH requirements described in WF[1].

Discussion

Issue 2-1-2: Requirements for RACH - Agreements - Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase.

Regarding RACH procedure, the random access is specified in clause 8 of TS38.213 and the control of the RACH transmission is specified in clause 5.1 of TS38.321. There are two types of procedures for the random access, the 4-step RA type, and the 2-step RA type. So, the requirements for random access procedures are described in clause 6.2.2.2 and 6.2.2.3 of TS38.133 for the 4-step RA type and 2-step RA type, respectively. During Rel-19 SBFD, some random access related issues have been discussed and made agreements as below in RAN2.

RAN2#127bis RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. RAN2#129bis Random access in SBFD - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported. RAN2#130 Random access in SBFD - Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. - For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. - The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available. RAN2#131 Random access in SBFD - RAN2 assume that when CFRA indicates SBFD RO, the RACH resources for the same RO type is provided for CBRA. FFS if any spec changes is needed. - For RACH configuration Option 2, all the RRC configured parameters are re-initialized after RO type switching. - For RACH Configuration Option 1, sbfd-RACHSingleConfig-preambleReceivedTargetPower is re-initialized after RO type switching. - Can discuss in the RRC CR review the configuration restriction (if needed) for preambleTransMax - For RACH configuration Option 2, PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are re-initialized after RO type switching. - For both RACH configuration Option 1 and RACH configuration Option 2, PREAMBLE_POWER_RAMPING_COUNTER is not re-initialized after RO type switching. - Can further check the other UE variables in the CR review. - For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two values of preamble power ramping steps is added. Exact change to the MAC spec can be further discussed in the CR review. - Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured. - For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

Detail procedures for RACH procedure of SBFD might be described in RAN2 document. So, RAN4 document need not to repeat same thing. However, adding a brief explanation would make the document more accessible to reader.

Proposal 1: RAN4 to consider adding following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

- The SBFD-aware UE is allowed to switch from additional RACH occasions to legacy RACH occasions or vice versa, after certain (configured) number of times of RACH attempt in RACH occasions.

Summary

In this contribution, we provide our views on the RACH requirements described in WF[1].

Proposal 1: RAN4 to consider adding following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

- The SBFD-aware UE is allowed to switch from additional RACH occasions to legacy RACH occasions or vice versa, after certain (configured) number of times of RACH attempt in RACH occasions.

Reference

[1] R4-2512133, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon