3GPP TSG-RAN WG4 Meeting #116bis	R4-2513704

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on core requirements maintenance for Rel-19 NES

Agenda Item:	4.21.1

Document for:	Discussion

- Introduction

The core part of Rel-19 NES was completed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on maintenance issues need to be addressed.

- Discussion

In last RAN1 meeting, this is checking point for RAN4 to decide whether to include smaller value than 25 for FR2 which is the number of OD-SSB burst. The already agreed values are shown as follows.

Agreement - Confirm the value range of od-ssb-nrofBurst for both FR1 and FR2 - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}. - Note: It is up to RAN4 whether to include smaller value than 25 for FR2.

Based on the applicable scenarios in RAN1/RAN4, following cases are considered:

- Deactivated SCell measurement (Scenario 2)

- SCell activation (Scenario 2A)

- After SCell is activated (Scenario 3B)

  - L1 measurement

  - L3 measurement

Firstly, for Scenario 3B, since only parameter updating is allowed for an already triggered OD-SSB, and the OD-SSB is not allowed to be deactivated for case#1 (i.e., NO AO-SSB), there is no sense to update the value of od-ssb-nrofBurst to a smaller one.

For Scenario 2 deactivated SCell measurement, the most critical case is measurement in fast measurement window (i.e., with minimum number of SSB bursts). At least 24 samples are needed for measurement, and more samples are needed for Cell search before measurement.

For Scenario 2A, for known SCell activation, only one SSB burst is needed which is not target scenario for OD-SSB. For unknown SCell activation, multiple samples are assumed for AGC/search/L1-RSRP/TF tracking. Thus, we fail to see the case that smaller value than 25 is needed.

Observation 1: Based on the applicable scenarios for OD-SSB, there is no significant need to introduce smaller value than 25 for od-ssb-nrofBurst for FR2.

Proposal 1: RAN4 confirm that there is no need to include smaller value than 25 for FR2 for od-ssb-nrofBurst, and provide the feedback to RAN1/2.

The drafted LS can be found in Annex.

Another issue which has been pending for many meeting between RAN2 and RAN4 is whether to introduce OD-SSB MAC CE triggered MR. The issue was both identified in RAN2 and RAN4, and after several meetings discussion, RAN2 concluded that it is up to RAN4 to make the decision in June RAN2 meeting, which is shown as follows:

RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

In last RAN4 meeting, RAN4 does not have much discussion on the technical details, and only reach following agreements.

Issue 1-4-4: OD-SSB based L3 reporting Agreement The discussion of the issue shall not impact the RAN4 core part completion of the WI.

Considering that this will have RAN2 impacts if RAN4 agree to introduce the method, it is proposed to conclude this issue in this meeting to leave sufficient time for subsequent RAN2 work.

Proposal 2: RAN4 to conclude on whether to introduce OD-SSB MAC CE triggered MR in this meeting to leave sufficient time for subsequent RAN2 work if needed.

In the following part, we provide though analysis on aspects that companies shown concerns on in previous meeting.

What is the benefit to have OD-SSB MAC CE triggered MR?

As discussed in previous meeting, for power saving purpose (both UE and gNB), gNB can configure a large MeasCellSCell to UE, and transmit long period OD-SSB. This will result in a very long measurement period. Take FR2 as example, the most extremely case is 30 seconds (1280ms*24). It means the last measurement is performed 30 seconds before. Thus, RAN4 have quite many discussions and agreed to introduce fast measurement window. Thus, gNB can triggered OD-SSB and UE will perform fast measurement accordingly to create a fresh measurement result. And this result is essential to gNB for subsequent determination (e.g., whether to activate/delete the SCell).

Fig.1

Observation 3: Since fast measurement window is introduced upon OD-SSB activation, the fresh measurement results is expected for NW.

Whether legacy L3 report can work?

Some companies commented during the discussion that there is no need to introduce addition MR triggering and legacy L3 report can be relied on. As shown in Fig.2, based on legacy L3 report method, what gNB can do to guarantee that UE will report a result after fast measurement, is to config an Event with very low RSRP threshold. But it has following drawbacks:

- Due to the artificially low event threshold, UE will keep reporting before OD-SSB is triggered for case#2, which will waste UL resource. And UE maybe not trigger the report after fast measurement since it event is already triggered before.

- gNB can not tell whether the event triggered report is based on newly measured results in fast measurement window or previous measurement.

Fig.2

Observation 3: Legacy L3 report method can not serve the purpose (i.e., report fresh result after fast measurement): UE may not trigger the report since the event maybe triggered before by artificially low threshold.

Based on analysis above, it is necessary to introduce OD-SSB activation MAC CE triggered L3 report for deactivated SCell measurement. Also, the workload of RAN2 is manageable as the framework of SCell activation triggered report could be leveraged.

Observation 4: The workload of RAN2 is manageable as the framework of SCell activation triggered report could be leveraged.

Proposal 3: RAN4 agree to introduce OD-SSB MAC CE triggered measurement reported and ask RAN2 to introduce corresponding signals. The reporting is optionally supported by UE.

- Conclusion

Observation 1: Based on the applicable scenarios for OD-SSB, there is no significant need to introduce smaller value than 25 for od-ssb-nrofBurst for FR2.

Proposal 1: RAN4 confirm that there is no need to include smaller value than 25 for FR2 for od-ssb-nrofBurst, and provide the feedback to RAN1/2.

Proposal 2: RAN4 to conclude on whether to introduce OD-SSB MAC CE triggered MR in this meeting to leave sufficient time for subsequent RAN2 work if needed.

Observation 3: Since fast measurement window is introduced upon OD-SSB activation, the fresh measurement results is expected for NW.

Observation 3: Legacy L3 report method can not serve the purpose (i.e., report fresh result after fast measurement): UE may not trigger the report since the event maybe triggered before by artificially low threshold.

Observation 4: The workload of RAN2 is manageable as the framework of SCell activation triggered report could be leveraged.

Proposal 3: RAN4 agree to introduce OD-SSB MAC CE triggered measurement reported and ask RAN2 to introduce corresponding signals. The reporting is optionally supported by UE.

References

[1] R4-2512327	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Annex

3GPP TSG-RAN WG4 Meeting #116bis

Prague, Czech, Oct. 13-17, 2025

Title:	LS on value range of od-ssb-nrofBurst for Rel-19 NES

Release:	Release 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:	TSG RAN WG2

Contact person:

1	Overall description

RAN4 has discussed whether to include smaller value than 25 for od-ssb-nrofBurst for FR2 in RAN4#116bis meeting, and RAN4 reach consensus that there is need to include smaller value than 25 in Rel-19.

RAN4 respectfully asks RAN1 to take above agreements into consideration.

2	Actions

To TSG RAN1

ACTION: 	RAN4 respectfully asks RAN1 to take above agreements into consideration.

3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #117	17 – 21 November 2025			Dallas, US

TSG-RAN WG4 Meeting #118	09 – 13 February 2026			Gothenburg, SE