3GPP TSG-RAN WG4 Meeting #116bis	R4-2513727

# Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Low Band CA operation

Agenda item:	4.8.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on maintenance issues for low band CA via switching between FDD and SDL CC based on the WF [1].

Discussion

SDL SCell activation related requirement

Activation of Switching Pattern

Agreement: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

RAN4 agreed to introduce TLBCA which is additional processing time for applying switching pattern, and the value is FFS. Since switching pattern is applied with MAC-CE based and RRC based direct SCell activation, no need to add long processing time to apply switching pattern if MAC-CE and RRC processing delay were considered. So, if TLBCA is needed, 1ms is enough.

- Proposal 1: For SCell activation procedure, 1ms can be considered for TLBCA if needed.

After receiving MAC-CE based SCell activation or RRC based direct SCell activation in slot n, UE should apply switching pattern after n+(THARQ+3ms+ TLBCA)/slot-length or n+(TRRC_Process + TLBCA)/slot-length. If the time slot of n+(THARQ+3ms+ TLBCA)/slot-length or n+(TRRC_Process + TLBCA)/slot-length is middle of SDL SCell on duration according to switching pattern, UE should switch to SCell at the next switching period (switching gap for FDD  SDL).

- Proposal 2: For SCell activation procedure, UE should switch to SCell at the next switching period if UE is on SCell after n+(THARQ+3ms+ TLBCA)/slot-length or n+(TRRC_Process + TLBCA)/slot-length

Deactivation of Switching Pattern

Agreement: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

As similar to activation of switching pattern discussion, if TLBCA is needed, 1ms is enough.

- Proposal 3: For SCell deactivation procedure, 1ms can be considered for TLBCA if needed

After UE completed the SCell deactivation procedure, the semi-static switching pattern should not be applied immediately since no SCell DL signal is transmitted. Therefore, unlike activation of switching pattern for the SCell activation procedure, UE switches to PCell immediately if the time slot for compeleting SCell deactivation is on SDL SCell on duration.

- Proposal 4: For SCell deactivation procedure, UE switches to PCell immediately if UE is on SCell after completion of SCell deactivation.

SCell activation delay requirements

RAN4 has defined SCell activation requirements only when the SDL SCell reference signals to be measured for SCell activation are fully overlapped with the SDL SCell active periods based on the semi-static switching pattern. However, RAN4 needs to consider a fractional case where {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern. For this case, principle of Kp scaling factor could be applied to SCell activation delay.

- Proposal 5: RAN4 to define SCell activation delay requirements for fractional case where {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, and the principle of Kp scaling factor could be applied to SCell activation delay.

Other SCell SDL requirement

BFD/CBD requirements for SDL SCell

Issue 1-3-6: BFD/CBD requirements for SDL SCell - Proposal 1 (CATT): The BFD/CBD measurement requirement for SDL SCell in LB CA via switching can be defined based on the legacy requirement with the number of measurement sample being updated. The new measurement sample equals to the legacy value plus X, where X is the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern. - Proposal 2 (Apple, HW): - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - New P_LB factor is used to replace the P factor in the existing BFD/CBD requirement, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P_LB= Ntotal_LB/ Navailable_LB - Ntotal_LB is total number of BFD-RS or CBD-RS in W window. - Navailable_LB is number of available BFD-RS or CBD-RS in W window. - Proposal 3 (LGE): RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered. - Proposal 4 (vivo, MTK): L1 measurement requirements for BFD/CBD are not defined for SDL SCell via switching. - Proposal 5 (Ericsson): For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG. [Agreement]: - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of BFD-RS or CBD-RS in W window. - Navailable is number of available BFD-RS or CBD-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage for P3.

Regarding BFD/CBD for SCell, a UE evaluate DL beam quality of serving cell utilizing RS set in q0. If every the DL beam quality of RS in q0 are below Qout_LR, physical layer sends BFI (beam failure indication) to higher layer. If the certain number of BFI is occurred within cerain time window, the UE can declare beam failure, and try to transmit LRR (link recovery request via PRACH or PUCCH if schedulingRequestIDForBFR is configured). After transmit LRR, the UE can transmit MAC-CE providing index of BFD-RS and CBD-RS for a corresponding SCell.

One issue is that switching pattern is not considered in the requirement for PUCCH transmission with LRR. In 8.5.9 (requirements for beam failure receovery in SCell), UE shall be capable of transmit PUCCH with an LRR within a period of T, where T= T1xCeil((T2+D)/T1) after BFR is triggered, in which T1 is periodicity of PUCCH, T2 is CBD-RS evaluation period, and D=2ms. In this equation, if the occasion of the PUCCH is located on SDL carrier, the UE cannot meet the requirement. Potential solution can be that the UE transmits PUCCH at the first available PUCCH occasion with LRR on FDD carrier.

- Proposal 6: RAN4 to consider BFR requirements for SCell in 8.5.9 as follows:

  - The UE shall be capable of trnamit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D

    - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time

Another issue is whether to fallback to PCell when BFR is triggered in SDL SCell. In order to perform beam failure recovery process properly and avoid additional delay due to switching pattern, it would be helpful for the UE to pause the switching pattern during link recovery, and resume the switching pattern when beam failure recovery is completed.

In the previous meeting we have proposed not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered. However, it seems that the NW may not know the BFR triggered by UE side before providing LRR. In this sense, we suggest not to apply switching pattern until link recovery is completed after LRR is provided for SDL SCell.

For instance, as illustrated in Figure 1, if a UE is configured by schedulingRequestIDForBFR, and beam failure is triggered in SDL SCell, the UE can perform CBD evaluation, and needs to provide PUCCH followed by PUSCH MAC-CE. In order to perform beam recovery procedure faster, the UE can assume that switching pattern is paused until beam failure recovery is completed. During SCell link recovery procedure, the UE would stay on FDD carrier.

- Proposal 7: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after LRR (link recovery request) is provided for SDL SCell

Figure 1 Example of SCell BFR for LB CA

BWP switching for SDL SCell

Issue 1-3-9: BWP switching for SDL SCell - Proposal 1 (Ofinno): - Clarify the UE behavior by adding the following text in clause 8.6.2: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which: - is available after the completion of the BWP switching and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior by adding the following text in clause 8.6.3: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell. [Agreement]: FFS this issue in maintenance stage.

When the UE performs active Bandwidth Part (BWP) switching, a certain delay is incurred before the new BWP becomes operational. During this delay period, performing carrier switching simultaneously may not be feasible, especially when the UE operates with a single RF chain. Therefore, it is reasonable to suspend carrier switching during the active BWP switching delay. For example, even if the switching pattern requires a transition from PCell to SCell, the UE may continue with BWP switching on the PCell. Once the UE ready to receive PDSCH or transmit PUSCH at the first DL or UL slot on the newly activated BWP, carrier switching can be resumed. After switching pattern is resumed, the UE may follow the intended switching pattern accordingly. In conclusion, we agree on the principle of the proposal, but wording can be refined.

- Proposal 8: Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows:

  - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs

FDD PCell related requirement

RLM/BFD requirement for PCell

Issue 1-4-1: RLM requirement for PCell Agreement: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

Regarding RLM for PCell, when the downlink quality measured by the UE across all resources configured as RLM-RS falls below the threshold Qout, the UE's physical layer may report an out-of-sync condition to higher layers. Upon declaration of out-of-sync, the UE shall initiate link recovery procedures, which may include performing initial access or requesting RRC reconfiguration, or cell reselection. When the UE is in out-of-sync stage, the link between NW and UE is no longer stable. Furthermore, it is highly likely that the reception quality of the SDL SCell is also degraded or unreliable. Therefore, rather than continuing with carrier switching, it is more appropriate for the UE to initiate link recovery procedures on the PCell to restore connectivity.

- Proposal 9: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until radio link recovery is completed after all resources configured as RLM-RS falls below the threshold Qout

Regarding BFD/CBD for PCell, as same logic stated in BFD/CBD requirement for SDL SCell, the UE needs to transmit PRACH, and perform RACH procedure. It is not helpful to keep switching to SDL SCell when beam failure is triggered in PCell. As co-located scenario is assumed, beam failure in PCell would degrade the performance of SCell as well as PCell. Thefore, as illustrated in Figure 2, if BFR is triggered for PCell, not to apply switching pattern until link recovery is completed after PRACH is provided for FDD PCell.

- Proposal 10: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after PRACH is provided for FDD PCell

Figure 2 Example of PCell BFR for LB CA

Conclusion

In this contribution, we provide our views on maintenance issues for low band CA via switching between FDD and SDL CC, and we propose

- Proposal 1: For SCell activation procedure, 1ms can be considered for TLBCA if needed.

- Proposal 2: For SCell activation procedure, UE should switch to SCell at the next switching period if UE is on SCell after n+(THARQ+3ms+ TLBCA)/slot-length or n+(TRRC_Process + TLBCA)/slot-length

- Proposal 3: For SCell deactivation procedure, 1ms can be considered for TLBCA if needed

- Proposal 4: For SCell deactivation procedure, UE switches to PCell immediately if UE is on SCell after completion of SCell deactivation.

- Proposal 5: RAN4 to define SCell activation delay requirements for fractional case where {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, and the principle of Kp scaling factor could be applied to SCell activation delay.

- Proposal 6: RAN4 to consider BFR requirements for SCell in 8.5.9 as follows:

  - The UE shall be capable of trnamit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D

    - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time

- Proposal 7: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after LRR (link recovery request) is provided for SDL SCell

- Proposal 8: Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows:

  - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs

- Proposal 9: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until radio link recovery is completed after all resources configured as RLM-RS falls below the threshold Qout

- Proposal 10: RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after PRACH is provided for FDD PCell

Reference

[1] R4-2512119, “WF on RRM requirements for NR_LBCA_Sw” Apple