3GPP TSG-RAN WG4 Meeting #116bis	R4-2513728

# Prague, Czech, Oct. 13-17, 2025

Source:	LG Electronics

Title:	Discussion on performance requirements for Low Band CA via switching

Agenda item:	6.3.2

Document for:	Discussion

Introduction

In the last RAN#109, the core requirements for LB CA via switching have been finalized, and the work plan [1] for performance part was agreed. In this contribution, we provide our views on test cases for LB CA via switching.

Discussion

The core requirements for LB CA via switching have been defined

- Radio link monitoring (SSB and CSI-RS based)

- Interruptions during measurements on deactivated SCC

- SCell activation/deactivation requirement

- Direct SCell activation at SCell addition

- Link recovery (SSB and CSI-RS based)

- TCI state switch delay (MAC-CE and RRC based)

- Pathloss RS switch delay (MAC-CE based)

- Intra-frequency measurement without gap

- L1-RSRP measurement reporting (SSB and CSI-RS based)

- L1-SINR measurement reporting (SSB and CSI-RS based)

, and these requirements were defined only for the case where fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern. Therefore, RAN4 needs to introduce test cases when fraction of measurement RS are not available based on the semi-static switching pattern except for test case of interruption requirements for deactivated SCell measurement.

- Proposal 1: All test cases for LB CA via switching should be verified when fraction of measurement RS are not available based on the semi-static switching pattern, except for test case of interruption requirements for deactivated SCell.

  - E.g., SSB/SMTC periodicity 20msec and switching pattern{20msec SCell; 20msec PCell}

For LB CA, since the tests are to verify the UE’s functional operation according to the switching pattern, there is no need to test for all cases. Therefore, we propose test cases to be defined for LB CA as below

- Radio link monitoring (SSB and CSI-RS based)

- Interruptions during measurements on deactivated SCC

- SCell activation/deactivation requirement

- Intra-frequency measurement without gap

- Proposal 2: To verify the UE’s functional operation according to the switching pattern, consider following test cases with non-DRX

  - Radio link monitoring (SSB and CSI-RS based)

  - Interruptions during measurements on deactivated SCC

  - SCell activation/deactivation requirement

  - Intra-frequency measurement without gap

  - Open to other test cases

For test paratemer, existing FR1 with 15kHz SCS test parameters could be baseline and switching pattern that partillay overlaps the measurement RS should be added.

- Proposal 3: For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline, and switching pattern configuration should be partially overlapped with measurement RS.

Conclusion

In this contribution, we provide our iews on test cases for low band CA via switching, and we propose

- Proposal 1: All test cases for LB CA via switching should be verified when fraction of measurement RS are not available based on the semi-static switching pattern, except for test case of interruption requirements for deactivated SCell.

  - E.g., SSB/SMTC periodicity 20msec and switching pattern{20msec SCell; 20msec PCell}

- Proposal 2: To verify the UE’s functional operation according to the switching pattern, consider following test cases with non-DRX

  - Radio link monitoring (SSB and CSI-RS based)

  - Interruptions during measurements on deactivated SCC

  - SCell activation/deactivation requirement

  - Intra-frequency measurement without gap

  - Open to other test cases

- Proposal 3: For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline, and switching pattern configuration should be partially overlapped with measurement RS.

Reference

[1] RP-2509990, “Work plan of performance part for R19 NR_LBCA_Sw RRM” Apple, TELUS