3GPP TSG-RAN WG4 # 116-bis	R4-2513764

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	7.9.3

Source:	MediaTek Inc.

Title:	Discussion on training and interoperability for two-sided AIML models

Document for:	Discussion

# Introduction

This contribution provides our views on newly introduced work item of Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2.

# Discussion

Newly introduced work item of AI/ML for NR air interface Phase 2 provides the normative support for the CSI feedback enhancement (two-sided model) use case as well as standards-based UE data collection. Work item objectives for RAN4 are defined as follows. [1]

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In the following sections we discuss our views on standardized models and datasets with several observations and proposals.

## 2.1 Discussion of standardized models

One of the main objectives of this work item is to define standardized reference encoder and test decoder models.

The Rel‑19 study item did not consider an implementation‑friendly model‑parameter format; it used full precision. When designing the reference encoder and decoder models, we must ensure they are practical to implement. Therefore, the format of the 3GPP reference encoder’s and test decoder’s model parameters must be studied and agreed. This decision has clear implications for compression performance and memory requirements.

Observation #1: Model parameter quantization affects compression performance and memory requirements.

Proposal #1: Study and agree on the format to use for the model parameters of the reference encoder and test decoder.

Proposal #2: We propose to use 8-bit precision parameters for the reference encoder as a starting point, with 16-bit precision as a fallback.

Proposal #3: We propose to use 8-bit precision parameters for the test decoder as a starting point, with 16-bit precision as a fallback.

Before we consider model backbones, we first need to understand the constraints. In practical implementations, FLOPs and memory budgets may differ significantly between UE and TE, and these should be considered separately. First, we need to agree on the upper bounds for FLOPs and parameter count for the reference encoder and the test decoder, respectively. With these constraints agreed, we can proceed to design and agree on the reference encoder and test decoder backbones. The Rel-19 study showed that the encoder and decoder backbones can differ. However, for simplicity, we propose using the same backbone for both the encoder and the decoder.

Observation #2: Practical FLOPs and memory budgets may differ significantly between UE and TE and should be considered separately.

Observation #3: Backbones with the same complexity and memory footprint can achieve different levels of compression performance.

Proposal #4: As a first step, study and agree on implementation‑friendly target complexity and memory budgets for the reference encoder and test decoder backbones.

Proposal #5: We propose defining an upper bound of 8 million FLOPs per inference for the reference encoder.

Proposal #6: We propose defining an upper bound of 500,000 parameters for the reference encoder.

Proposal #7: We propose seeking feedback from TE vendors to define the upper bounds on FLOPs and parameter count.

Proposal #8: As a second step, agree on the reference encoder and test decoder backbones, considering the agreed target complexity and memory from the first step.

Proposal #9: We propose using the same backbone for both the encoder and the decoder.

To support multiple configurations, either multiple models must be defined for different configurations, or scalability mechanisms must be defined within a single model. Methods such as padding/truncation and adaptation layers for handling different input/output dimensions were studied by RAN1 in the Rel‑18 AI/ML for Air Interface SI. It was shown that the padding/truncation approach offers advantages in model size and FLOPs compared with the adaptation‑layer approach, with comparable performance. Therefore, we propose using padding/truncation as the scalability mechanism for handling different input/output dimensions. Furthermore, various solutions for handling different latent‑message sizes (CSI payload) were studied by RAN1 in the Rel‑18 AI/ML for Air Interface SI, without a clear recommended solution. Therefore, we need to discuss and analyze which solution should be adopted by RAN4. The supported scalability should cover all agreed test‑case configurations. Our intention is to define a single scalable reference encoder and a single scalable test decoder.

Observation #4: The padding/truncation approach has advantages in terms of model size and FLOPs compared to adaptation layer approach with comparable performance.

Observation #5: Various solutions for handling of different latent message sizes (CSI payload) has been studied by RAN1 in Rel-18 AI/ML for Air Interface SI.

Proposal #10: We propose to define a reference encoder model which achieves scalability through zero-padding approach.

Proposal #11: We propose to define a test decoder model which achieves scalability through truncation approach.

Proposal #12: We propose to first analyze and agree on what solution to adopt for latent message scalability.

Proposal #13: We propose to define a reference encoder and a test decoder which supports the agreed method of handling various latent message sizes that covers all the agreed test case configurations which affects payload size.

## 2.2 Discussion of standardized datasets

One of the main objectives of this work item is to define standardized dataset that is used for training the reference encoder and the test decoder.

First, we need to define the dataset content. In the Rel‑19 RAN4 study, the dataset consisted of eigenvectors of the strongest layer. The existing PMI reporting requirements favor Rank 2 configurations, as this is usually a good indicator of PMI feedback quality. Therefore, if the dataset content is agreed to remain eigenvectors, it should be extended to cover the two strongest layers. Because an eigenvector representation omits some channel information, there have been proposals to use raw channel data in the datasets. Nevertheless, we prefer to retain an eigenvector‑based dataset unless there is strong justification for collecting full raw channel data instead.

Observation #6: Raw channel data depends on the Tx and Rx dimension.

Observation #7: Eigenvector data depends on the Tx and the number of layers.

Proposal #14: We propose to define the dataset content as eigenvectors up to Rank 2.

Second, we need to define the scenarios to be covered in the datasets. To test generalization, we need to define different antenna array shapes (N1, N2) for different tests. We propose maintaining separate datasets for different configurations to maximize the implementation’s ability to generalize. When training the reference encoder and the test decoder, we propose combining all datasets for training. We also propose agreeing on the total target size of each dataset and the suggested contribution size from each company.

Observation #8: Different configurations may affect performance, e.g., different antenna array shape (N1, N2).

Proposal #15: We propose to create separate datasets corresponding to each configuration used in the agreed test case configurations.

Proposal #16: We propose to train reference encoder and test decoder using a combined dataset that covers all the agreed test case configurations.

Proposal #17. We propose to define minimum total dataset target size as 500,000 samples.

Proposal #18. We propose to define minimum dataset target size from each company as 100,000 samples.

Third, we need to define the methodology for generating the dataset. In the Rel‑19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data. SLS‑based datasets tend to generalize well and can be assumed to cover LLS scenarios adequately, but not vice versa. Therefore, we propose generating the dataset solely using SLS configured with the CDL channel model.

Observation #9: In the Rel‑19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data.

Proposal #19: We propose using SLS with the CDL channel model to generate the dataset.

Lastly, we need to define methodologies for verifying dataset correctness. In the Rel‑19 study, datasets provided by some companies were found to differ significantly from most other datasets. We propose three metrics as potential verification measures.

Observation #10: In the Rel‑19 SI, datasets provided by some companies were found to differ significantly from most other datasets.

Proposal #20: We propose to study and agree on dataset‑related metrics and statistics to ensure dataset correctness.

Proposal #21: We propose to study the following metrics: power spectral entropy (PSE), frequency coherence (FC), and spatial coherence (SP).

# Summary

In this paper we provided our views to training and interoperability for two-sided AIML models topics. The following observations and proposals are made.

Observation #1: Model parameter quantization affects compression performance and memory requirements.

Proposal #1: Study and agree on the format to use for the model parameters of the reference encoder and test decoder.

Proposal #2: We propose to use 8-bit precision parameters for the reference encoder as a starting point, with 16-bit precision as a fallback.

Proposal #3: We propose to use 8-bit precision parameters for the test decoder as a starting point, with 16-bit precision as a fallback.

Observation #2: Practical FLOPs and memory budgets may differ significantly between UE and TE and should be considered separately.

Observation #3: Backbones with the same complexity and memory footprint can achieve different levels of compression performance.

Proposal #4: As a first step, study and agree on implementation-friendly target complexity and memory budgets for the reference encoder and test decoder backbones.

Proposal #5: We propose defining an upper bound of 8 million FLOPs per inference for the reference encoder.

Proposal #6: We propose defining an upper bound of 500,000 parameters for the reference encoder.

Proposal #7: We propose seeking feedback from TE vendors to define the upper bounds on FLOPs and parameter count.

Proposal #8: As a second step, agree on the reference encoder and test decoder backbones, considering the agreed target complexity and memory from the first step.

Proposal #9: We propose using the same backbone for both the encoder and the decoder.

Observation #4: The padding/truncation approach has advantages in terms of model size and FLOPs compared to adaptation layer approach with comparable performance.

Observation #5: Various solutions for handling of different latent message sizes (CSI payload) has been studied by RAN1 in Rel-18 AI/ML for Air Interface SI.

Proposal #10: We propose to define a reference encoder model which achieves scalability through zero-padding approach.

Proposal #11: We propose to define a test decoder model which achieves scalability through truncation approach.

Proposal #12: We propose to first analyze and agree on what solution to adopt for latent message scalability.

Proposal #13: We propose to define a reference encoder and a test decoder which supports the agreed method of handling various latent message sizes that covers all the agreed test case configurations which affects payload size.

Observation #6: Raw channel data depends on the Tx and Rx dimension.

Observation #7: Eigenvector data depends on the Tx and the number of layers.

Proposal #14: We propose to define the dataset content as eigenvectors up to Rank 2.

Observation #8: Different configurations may affect performance, e.g., different antenna array shape (N1, N2).

Proposal #15: We propose to create separate datasets corresponding to each configuration used in the agreed test case configurations.

Proposal #16: We propose to train reference encoder and test decoder using a combined dataset that covers all the agreed test case configurations.

Proposal #17. We propose to define minimum total dataset target size as 500,000 samples.

Proposal #18. We propose to define minimum dataset target size from each company as 100,000 samples.

Observation #9: In the Rel-19 SI, it was shown that performance depends strongly on whether the dataset is created from SLS or LLS data.

Proposal #19: We propose using SLS with the CDL channel model to generate the dataset.

Observation #10: In the Rel-19 SI, datasets provided by some companies were found to differ significantly from most other datasets.

Proposal #20: We propose to study and agree on dataset-related metrics and statistics to ensure dataset correctness.

Proposal #21: We propose to study the following metrics: power spectral entropy (PSE), frequency coherence (FC), and spatial coherence (SP).

# References

[1] RP-252445, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm