3GPP TSG-RAN WG4 # 116-bis	R4-2513766

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	6.11.7

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirements for CSI prediction

Document for:	Discussion

# Introduction

In RAN4#116, RAN4 agreed to continue discussion on CSI reporting requirements for CSI prediction issues [1]. In RAN4#116, updated simulation assumptions for AI/ML CSI prediction were agreed [2]. In this contribution, we provide our views and simulation results on CSI reporting requirement framework for CSI prediction.

# Discussion

In this Chapter, we provide our views on CSI prediction requirement issues and share simulation results.

The way forward on simulation assumptions for CSI prediction was agreed at the previous meeting [2]. Table 1 lists the simulation parameters used. Compared with the agreed assumptions, we have limited our simulation scope to FDD with a 10 MHz bandwidth and a 15 kHz SCS. For Step‑1 and Step‑2, we provide simulation results for all previously used Doppler frequencies (20 Hz, 50 Hz, and 100 Hz) and both agreed TX configurations: 32, with (N1, N2) = (4, 4), and 16, with (N1, N2) = (4, 2). The Step‑1 and Step‑2 datasets were generated with a link‑level simulator using ideal channel estimation. For Step‑3, we provide simulation results for both agreed Doppler frequencies (20 Hz and 50 Hz) and one of the agreed TX configurations: 16, with (N1, N2) = (4, 2). The Step‑3 dataset was generated with a link‑level simulator using practical channel estimation. The training dataset consists of channel realizations for all agreed Doppler frequencies. The testing datasets consist of independent channel realizations for all agreed Doppler frequencies separately. The Step‑3 dataset includes a range of SNRs, and training is performed across the full SNR range.

Companies were encouraged to report the model complexity of their AI/ML CSI prediction models to facilitate alignment. For this evaluation, we used models with fewer than 10,000 parameters and requiring fewer than 2 MFLOPs per inference for both the 16‑Tx and 32‑Tx configurations.

Table 1: Simulation parameters for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 32 (N1, N2) = (4, 4) as defined in Rel-15 type1 16 (N1, N2) = (4, 2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Beam steering | Beam steering approach with dual cluster beams (TS38.101-4 Annex B.2.3.2.3A) |
| Doppler spread | 20Hz, 50Hz |
| Delay spread | 30ns |
| Channel estimation | Ideal DL channel estimation (Step-1, Step-2) / Practical channel estimation (Step-3) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Check the CSI prediction performance of raw channel per layer Check the CSI prediction performance with Rel-18 codebook quantization |
| KPI for Step-1 | Average of SGCS for intermediate results over all subbands per layer SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |
| KPI for Step-2 | Average of SGCS for intermediate results over all subbands per layer SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth |
| KPI for Step-3 | Throughput γ1: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI γ2: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold) Channel prediction SGCS1 and SGCS3 Sample and hold SGCS3 |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training and validation datasets for Step-1 and Step-2 | Set1: 16TX containing 20Hz Doppler ideal estimates Set2: 16TX containing 50Hz Doppler ideal estimates Set3: 16TX containing 100Hz Doppler ideal estimates Set4: 16TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) Set5: 32TX containing 20Hz Doppler ideal estimates Set6: 32TX containing 50Hz Doppler ideal estimates Set7: 32TX containing 100Hz Doppler ideal estimates Set8: 32TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) |
| Testing datasets for Step-1 and Step-2 | Set1: 16TX containing only 20Hz Doppler ideal estimates Set2: 16TX containing only 50Hz Doppler ideal estimates Set3: 16TX containing only 100Hz Doppler ideal estimates Set4: 32TX containing only 20Hz Doppler ideal estimates Set5: 32TX containing only 50Hz Doppler ideal estimates Set6: 32TX containing only 100Hz Doppler ideal estimates |
| Training and validation datasets for Step-3 | Set1: 16TX containing 20Hz and 50Hz Doppler practical estimates (mixed) |
| Testing datasets for Step-3 | Set1: 16TX containing only 20Hz Doppler practical estimates Set2: 16TX containing only 50Hz Doppler practical estimates |
| MCS for Step-3 | MCS13, MCS17, MCS19 |

In the following Table 2 we share our Step-1 and Step-2 average of SGCS for intermediate results over all subbands per layer for 4 different measures. Firstly, we have non-predicted (sample and hold) raw channel SGCS. Secondly, we have predicted raw channel SGCS. Thirdly, we have predicted Rel-18 quantized precoder SGCS2 where comparison is done to ground truth of unquantized SVD precoder. Finally, we have predicted Rel-18 quantized precoder SGCS3 where comparison is done to ground truth Rel-18 quantized precoder. Note, that Step-1 and Step-2 SGCS results have not changed compared to our previous discussion paper [3].

Table 2: Step-1 and Step-2 results of average of SGCS over all subbands per layer

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel sample and hold [SGCS per layer] | Channel prediction [SGCS per layer] | Rel-18 precoder prediction SGCS2 [SGCS per layer] | Rel-18 precoder prediction SGCS3 [SGCS per layer] |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 16TX | 20 | 20 | [0.795, 0.650] | [0.999, 0.999] | [0.907, 0.825] | [0.992, 0.985] |
|  | 50 | 50 | [0.325, 0.187] | [0.969, 0.944] | [0.892, 0.801] | [0.955, 0.922] |
|  | 100 | 100 | [0.301, 0.166] | [0.750, 0.572] | [0.774, 0.597] | [0.821, 0.675] |
|  | Mixed (20, 50, 100) | 20 | [0.795, 0.650] | [0.957, 0.923] | [0.879, 0.778] | [0.941, 0.898] |
|  |  | 50 | [0.325, 0.187] | [0.872, 0.777] | [0.822, 0.688] | [0.873, 0.781] |
|  |  | 100 | [0.301, 0.166] | [0.666, 0.456] | [0.677, 0.471] | [0.717, 0.533] |
| FDD 32TX | 20 | 20 | [0.814, 0.670] | [0.999, 0.998] | [0.869, 0.735] | [0.990, 0.982] |
|  | 50 | 50 | [0.391, 0.201] | [0.972, 0.946] | [0.853, 0.708] | [0.961, 0.924] |
|  | 100 | 100 | [0.309, 0.160] | [0.743, 0.561] | [0.774, 0.597] | [0.836, 0.690] |
|  | Mixed (20, 50, 100) | 20 | [0.814, 0.670] | [0.954, 0.911] | [0.844, 0.693] | [0.944, 0.894] |
|  |  | 50 | [0.391, 0.201] | [0.881, 0.770] | [0.797, 0.611] | [0.891, 0.787] |
|  |  | 100 | [0.309, 0.160] | [0.664, 0.444] | [0.673, 0.438] | [0.749, 0.563] |

From these results we can make the following observations.

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler, accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

In the following Table 3, we present SGCS1 and SGCS3 metrics for channel prediction and the SGCS3 metric for sample and hold, measured at the SNR closest to the 90% of maximum throughput point. We observe channel prediction gains over sample and hold in all tests. The SGCS measured gain is larger at 50 Hz Doppler than at 20 Hz.

Observation #4: In Step‑3, channel prediction shows gains over sample and hold in all tests when evaluated using SGCS.

Observation #5: In Step‑3, the SGCS measured gains of channel prediction over sample and hold are larger at 50 Hz than at 20 Hz.

Table 3: Step-3 results of average SGCS over all subbands per layer (SNR at 90% throughput, prediction window 1/5ms/5ms)

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel prediction SGCS1 [SGCS per layer] | Channel prediction SGCS3 [SGCS per layer] | Channel sample and hold SGCS3 [SGCS per layer] |
| --- | --- | --- | --- | --- | --- |
| FDD 16TX MCS-13 | Mixed (20, 50) | 20 | [0.839, 0.718] | [0.845, 0.745] | [0.699, 0.540] |
|  |  | 50 | [0.793, 0.669] | [0.777, 0.639] | [0.302, 0.192] |
| FDD 16TX MCS-17 | Mixed (20, 50) | 20 | [0.908, 0.837] | [0.884, 0,.802] | [0.719, 0.568] |
|  |  | 50 | [0.834, 0.737] | [0.799, 0.674] | [0.305, 0.194] |
| FDD 16TX MCS-19 | Mixed (20, 50) | 20 | [0.920, 0.860] | [0.891, 0.816] | [0.722, 0.573] |
|  |  | 50 | [0.844, 0.754] | [0.805, 0.682] | [0.305, 0.194] |

In the following Figures 1 to 3 and Tables 4 to 5 we share our PDSCH Rank 2 throughput simulation results using various MCS. We have simulated MCS13, MCS17 and MCS19 to find suitable and feasible operation point for testing.

We report the SNR required to achieve 90% relative throughput for both predicted PMI and sample‑and‑hold (legacy) PMI. We also report the throughput ratio γ1 between predicted PMI and random PMI; this ratio would serve as the PMI reporting requirement for the predicted PMI. In addition, we report the throughput ratio γ2 between predicted PMI and sample and hold PMI to quantify the prediction gain. Finally, for reference, we report the throughput ratio γ between sample and hold PMI and random PMI.

Figure 1: Step-3 throughput results of PDSCH Rank2 MCS13 in Dopplers 20Hz and 50Hz.

Figure 2: Step-3 throughput results of PDSCH Rank2 MCS17 in Dopplers 20Hz and 50Hz.

Figure 3: Step-3 throughput results of PDSCH Rank2 MCS19 in Dopplers 20Hz and 50Hz.

Table 4: Step-3 PDSCH Rank 2 results collection of SNR, γ1 and γ2. Prediction results are given for prediction window 1/5ms/5ms.

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction γ1 (vs. random PMI) | Prediction γ2 (vs. s&h) | Sample&Hold γ (vs. random PMI) |
| --- | --- | --- | --- | --- | --- | --- |
| 20Hz | 13 | 1.4 | 3.2 | 3.1 | 1.2 | 2.3 |
|  | 17 | 4.7 | 6.7 | 2.8 | 1.2 | 2.1 |
|  | 19 | 6.3 | 8.4 | 2.6 | 1.2 | 2.0 |
| 50Hz | 13 | 6.0 | 5.6 | 1.7 | 1.0 | 1.7 |
|  | 17 | 9.9 | 9.5 | 1.6 | 1.0 | 1.6 |
|  | 19 | 11.8 | 11.2 | 1.5 | 1.0 | 1.6 |

Table 5: Step-3 PDSCH Rank 2 results collection of SNR, γ1 and γ2. Prediction results are given for prediction window 1/5ms/10ms.

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction γ1(vs. random PMI) | Prediction γ2 (vs. s&h) | Sample&Hold γ (vs. random PMI) |
| --- | --- | --- | --- | --- | --- | --- |
| 20Hz | 13 | 0.3 | 3.2 | 3.9 | 1.4 | 2.3 |
|  | 17 | 3.5 | 6.7 | 3.6 | 1.4 | 2.1 |
|  | 19 | 5.0 | 8.4 | 3.2 | 1.5 | 2.0 |
| 50Hz | 13 | 5.0 | 5.6 | 1.9 | 1.1 | 1.7 |
|  | 17 | 8.8 | 9.5 | 1.7 | 1.1 | 1.6 |
|  | 19 | 10.7 | 11.2 | 1.7 | 1.1 | 1.6 |

In the Figures 1 to 3 and Tables 4 to 5 we present our PDSCH Rank 2 throughput simulation results under two different prediction assumptions. The prediction window 1/5ms/5ms corresponds to the prediction timing defined in the simulation assumptions, in which we predict for the next CSI‑RS occasion. The prediction window 1/5ms/10ms corresponds to the prediction timing in which we predict for the CSI‑RS occasion after next. We investigated whether prediction performance can be improved by optimizing the prediction timing.

According to TS38.214 (Section 5.2.1.4.2) [4], assuming periodic CSI-RS and 𝑁4=1, UE is expected to report a predicted PMI for slot interval [𝑙, 𝑙+𝑑−1], where value of 𝑑 is the CSI-RS period. In addition, slot 𝑙=𝑛+𝛿, where 𝑛 is the uplink slot in which the CSI is reported and the slot offset 𝛿∈{−𝑛𝐶𝑆𝐼_𝑟𝑒𝑓,0,1,2}. This leaves uncertainty to the position of the target slot applied for PMI prediction within the slot interval. Our results indicate that PMI prediction for CSI-RS slot coming 10 slots after the last observation CSI-RS is optimal (prediction window 1/5ms/10ms). For prediction window 1/5ms/5ms, the predicted PMI is not matching well with the slot interval in which PMI is used by gNB.

Observation #6: Current RAN1 specification TS38.214 defines only a slot interval for which PMI is predicted while exact prediction slot is undefined.

Observation #7: For the test case, current RAN1 specification TS38.214 does not allow signalling the same five-slot period for PMI reporting that is used for applying the reported PMI at TX.

From presented results we can make the following observations.

Observation #8: Prediction gain measured in SNR varies between 1.8 to 2.1dB in Doppler 20Hz.

Observation #9: Prediction gain measured in γ1 varies between 2.6 to 3.1 in Doppler 20Hz.

Observation #10: There is no prediction gain in Doppler 50Hz.

Observation #11: Operation point of MCS13 in Doppler 20Hz is close to SNR 0dB.

Observation #12: Optimized prediction timing can improve performance approximately 1dB.

As the results show, there is no prediction gain at 50 Hz Doppler; therefore, we propose excluding 50 Hz from the PMI reporting requirements. Our assumption, consistent with previous Rel‑18 non‑AI prediction studies, is that very low and very high Doppler frequencies are not well suited to prediction, whereas an intermediate range is best suited for PMI prediction.

Proposal #1: We propose to introduce PMI reporting requirement with Doppler 20Hz.

Proposal #2: We propose not to introduce PMI reporting requirement with Doppler 50Hz.

However, when designing performance monitoring tests, low and high Doppler frequencies can be used to verify reporting conditions under which no prediction gain is expected.

We prefer to use a sufficiently high MCS when defining requirements to ensure robust SNR conditions during testing.

Proposal #3: We propose to use MCS17 or MCS19 for robust testing SNR.

# Conclusion

In this contribution we provide the view on the CSI reporting requirement framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

Observation #4: In Step-3, channel prediction shows gains over sample and hold in all tests when evaluated using SGCS.

Observation #5: In Step-3, the SGCS measured gains of channel prediction over sample and hold are larger at 50 Hz than at 20 Hz.

Observation #6: Current RAN1 specification TS38.214 defines only a slot interval for which PMI is predicted while exact prediction slot is undefined.

Observation #7: For the test case, current RAN1 specification TS38.214 does not allow signalling the same five-slot period for PMI reporting that is used for applying the reported PMI at TX.

Observation #8: Prediction gain measured in SNR varies between 1.8 to 2.1dB in Doppler 20Hz.

Observation #9: Prediction gain measured in γ1 varies between 2.6 to 3.1 in Doppler 20Hz.

Observation #10: There is no prediction gain in Doppler 50Hz.

Observation #11: Operation point of MCS13 in Doppler 20Hz is close to SNR 0dB.

Observation #12: Optimized prediction timing can improve performance approximately 1dB.

Proposal #1: We propose to introduce PMI reporting requirement with Doppler 20Hz.

Proposal #2: We propose not to introduce PMI reporting requirement with Doppler 50Hz.

Proposal #3: We propose to use MCS17 or MCS19 for robust testing SNR.

# Reference

- R4-2511890, WF on AIML for air interface for [116][131], Qualcomm inc.

- R4-2511796, WF on updated simulation assumptions for AI based CSI prediction, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

- R4-2509655, Discussion on CSI reporting requirement framework for CSI prediction, MediaTek inc.

- 3GPP TS 38.214, Physical layer procedures for data (Release 19), V19.1.0 (2025-09)