3GPP TSG-RAN WG4 # 116-bis	R4-2513767

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	6.12.3

Source:	MediaTek Inc.

Title:	Discussion on NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

In the previous RAN4#116 meeting NR MIMO Phase 5 was discussed and WF with several open issues was introduced for further work [1]. In this document we discuss remaining open issues with several observations and proposals. We also have separate simulation results document [2].

# Discussion

In work item description of NR MIMO Phase 5 there is the following RAN1 objective for CSI support for up to 128 CSI-RS antenna ports.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In work item description of NR MIMO Phase 5 there is the following RAN4 performance objective.

- Specify necessary performance requirements for the enhancements listed above.

In updated RAN1 UE features list for Rel-19 NR [3] there are several UE capabilities for CSI support for up to 128 CSI RS antenna ports. Note, support for each number of antenna ports has own dedicated capability.

UE capabilities of extended codebooks for CSI support for up to 128 CSI RS antenna ports - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-A - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-B - Enhanced Type-I MP codebook for [48, 64, 128] ports - Extended Rel-16 eType-II codebook for [48, 64, 128] Tx ports - Extended Rel-17 FeType-II codebook with [48, 64] Tx ports - Extended Rel-18 eType-II Doppler codebook for [48, 64, 128] Tx ports

In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook. From these codebooks Type-I SP codebook is the most deployed codebook in live networks by far. Correspondingly, in previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook. Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook. Therefore, we consider this as the most potential use case. We see that defining only PMI requirements for new Rel-19 CSI codebooks is sufficient.

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

New enhanced and extended codebooks introduce variants with different number of antenna ports. Available options are 48, 64, and 128 antenna ports, except FeType-II codebook does not support 128 antenna ports.

Table 1: RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources | 2-35 |
| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |
| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |

Table 1 lists the main parts of RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A. Here we can see that the baseline capability for Enhanced Type-I SP codebook is with 64 antenna ports. In addition, Enhanced Type-I SP codebook can support 48 and/or 128 antenna ports. This similar principle applies also to other Rel-19 Enhanced/Extended codebooks with 48 or more antenna ports.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

## 2.1 General scope

In this Section we discuss open issues of general scope.

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19 - Interested companies are welcomed to provide simulation results for next meeting

We observe that the market is dominated by Type-I SP codebook solutions, and the potential of new eTypeII solutions is limited. Therefore, we see little benefit in introducing new requirements for the eTypeII-r19 codebook.

Proposal #1: We propose to not consider eTypeII-r19 codebook for PMI requirements.

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19 Way forward: - FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19

We observe that the market is dominated by Type-I SP codebook solutions, and the potential of new eTypeII-based codebook solutions is limited. Therefore, we see little benefit in introducing new requirements for the typeII-Doppler-r19 codebook.

Proposal #2: We propose to not consider typeII-Doppler-r19 codebook for PMI requirements.

Issue 1-1-3: Clarify if UE demodulation/CSI reporting requirements should be specified for SRS port grouping Proposals: - Option 1: RAN4 to discuss to reuse an existing3-layer, 4Rx demodulation requirement with a simple set up for SRS port grouping feature. - Option 2: RAN4 should not introduce UE demodulation/CSI reporting requirements for SRS port grouping - Option 3: Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19 Way forward: - Encourage supporters for introducing SRS port grouping related test requirements to provide more details on the test setup.

In the previous meeting, it was noted that there is no precedent in RAN4 for specifying UE demodulation requirements for SRS-based precoders. Furthermore, it was proposed to potentially reuse existing UE demodulation requirements for the new SRS port grouping scheme to avoid additional simulation work.

Observation #6: There is no precedent in RAN4 to have UE demodulation requirements for SRS based precoders.

We observe that if RAN4 were to introduce new requirements for SRS-based precoders, it should first define the behaviour of test equipment. Additionally, with agreed-upon precoding processing, real performance simulations should be conducted. Typically, RAN4’s efforts to define test equipment precoder processing, such as in the Rel-17 FeType-II codebook, have faced challenges in reaching consensus. Therefore, we have a pessimistic view regarding the introduction of UE demodulation requirements for SRS-based precoders.

Proposal #3: We propose to not consider UE demodulation/CSI reporting requirements for SRS port grouping.

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT Proposals: For new CSI reporting for time/frequency/phase offsets: - Option 1: RAN4 should not introduce CSI reporting requirements for UE reporting enhancements for CJT - Option 2: RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test. For PMI reporting: - Option 1: Further discussion on introducing new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation. Way forward: - Encourage supporters for introducing CJT related CSI reporting test requirements to provide testable test setup.

In the previous meeting, it was proposed to further study a test scenario with two TRPs using a non-ideal or non-synchronized backhaul for Coherent Joint Transmission (CJT). In this scenario, the UE would report various channel properties, such as time offset, frequency offset, and phase offset. Furthermore, these reported offsets would be pre-compensated at the transmitter, and the corresponding performance could then be measured and compared to non-compensated transmission.

We see that implementing such a test scenario would be unnecessarily complex. We consider it sufficient to directly measure the reporting accuracy of the listed channel properties, as is done in the RRM requirements. Therefore, we propose not to introduce CSI reporting requirements for UE reporting enhancement for CJT.

Proposal #4: We propose to not consider CSI reporting requirements for UE reporting enhancements for CJT.

## 2.2 Test assumptions for typeI-SinglePanel-r19

In this Section we discuss open issues of test assumptions for typeI-SinglePanel-r19.

Issue 1-2-1 Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: (Huawei) γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. Way forward: - FFS based on simulation results provided in following meetings

We observe that our simulated gamma values range from 15 to 36. These are likely unreliable and unstable because the random-precoding reference operated at an extremely high BLER. Therefore, rather than using these high gamma values directly as requirements, we suggest identifying an alternative solution. We consider the proposed Option 1 as a potential candidate to address this issue. In addition, we propose defining requirements at the SNR corresponding to 90% of the maximum throughput under PMI follow operation. We recommend comparing these two alternatives to determine which yields better-aligned results.

Observation #7: Our simulated gammas vary between 15 to 36.

Proposal #5: We see Option 1 as possible solution.

Proposal #6: We propose to consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed.

Issue 1-2-4: MCS level for 64 CSI-RS ports Agreement: - Start simulation work with both MCS20 and MCS22. Down select further based on simulation results.

In our simulations, with updated simulation assumptions we still observe lower and more feasible gamma values with the MCS22 configuration. Additionally, the SNR operating point would be more feasible with MCS22. Therefore, we believe that MCS22 would result in a more robust test configuration.

Proposal #7: We propose to use MCS22 (Table 1) for typeI-SinglePanel-r19 requirements with 64 ports.

Issue 1-2-5: ZP-CSI-RS and CSI-IM configurations for 64 CSI-RS ports Way forward: - Reuse existing Rel-15 TypeI 32Tx port PMI test

In the existing Rel-15 Type I 32Tx-port PMI test, the ZP-CSI-RS and CSI-IM configurations use OFDM symbol 9. For the NZP-CSI-RS in the CSI configuration, we propose using OFDM symbols 2, 3, 5, 6, 9, 10, 12, and 13. Therefore, if we adopt the proposed NZP-CSI-RS configuration, we need to reallocate the ZP-CSI-RS and CSI-IM to different symbols. We propose using OFDM symbol 7 for ZP-CSI-RS and CSI-IM.

Proposal #8: We propose to use symbol 7 for ZP-CSI-RS and CSI-IM.

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
| --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,7) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |

Issue 1-2-6: NZP-CSI-RS configurations for 64 CSI-RS ports Proposals: - Option 1: Proposed NZP-CSI-RS configuration NZP CSI-RS; CSI-RS resource 1 and 2; CSI-RS resource Type; Aperiodic. Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 17,(2, 4, 6, 8). CSI-RS resource 1; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (1, 7). CSI-RS resource 2; First OFDM symbol in the PRB used for CSI-RS (l0, l1); Row 17, (5, 12). CSI-RS resource 1 and 2; CSI-RS interval and offset; slot; Not configured. aperiodicTriggeringOffset; 0. - Other options are not precluded

Under the common test parameters for CSI tests, the CSI-RS for tracking is configured on OFDM symbols 4 and 8. For the NZP-CSI-RS in the CSI configuration, we propose using OFDM symbols 2, 3, 5, 6, 9, 10, 12, and 13 to avoid overlap with the default TRS configuration.

Proposal #9: We propose to use symbols 2, 3, 5, 6, 9, 10, 12 and 13 for NZP CSI-RS.

| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
| --- | --- | --- | --- | --- |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17,(2, 5) | Row 17,(9, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |

Issue 1-2-10: Scaling factors for MIMO correlation matrices Proposals: - Option 1: 0.00026 for 64 (8,4,2) × 2, 0.00050 for 64 (8,4,2) × 4, 0.00053 for 128 (16,4,2) × 2, 0.00101 for 128 (16,4,2) × 4 - Option 2: RAN4 to use scaling factor a=0.00010 for both 64 CSI-RS ports - Option 3: do not use scaling factors - Other options are not precluded

Correlation matrices are positive semi-definite by definition. The diagonal loading scaling factor 'a' has been defined in TS 38.101-4 Annex B only for the purpose of keeping MIMO correlation matrices positive semi-definite after truncating the element-wise values to four decimal digits. These numerical matrices have then been printed in the spec for a few cases (but not all) with relatively small numbers of TX/RX antennas.

As the number of TX-antennas increases further, it is not possible or beneficial to print the numerical values of correlation matrices into spec. The composite matrix size for 64TX-4RX MIMO channel is 256x256, and for 128TX-8RX the size is 1024x1024. Furthermore, different N1xN2 gNB array geometries result in different numerical matrices. Since the numerical values will not be printed, there is no need for truncating. Thus, we should not instruct people to do truncation.

As the matrix size increases, it is not efficient or numerically stable to apply the correlations by using the huge composite correlation matrix at once. Luckily, correlations specified by 3GPP are separately defined for different matrix sub-dimensions (as α1, α2, β, and γ). Each sub-correlation can be applied one sub-dimension at a time by using smaller correlation matrices that are easy to decompose. Actually, specifying diagonal loading on top of the huge composite matrix gives a wrong impression of how to efficiently implement correlation generation. Thus, we believe that specifying scaling factors is not only unnecessary, but it also gives an incorrect message.

Proposal #10: We propose not to specify or use scaling factors for MIMO correlation matrices.

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports? Way forward: - If introduced, 128 ports for TDD only. Introduce applicability rules as if 128 ports cases could pass, UE can skip corresponding 64 ports test.

At the last meeting, there was no clear indication of a need to introduce requirements for 128‑port CSI‑RS. We encourage proponents to provide further justification. At this stage, we consider it sufficient to define requirements assuming a minimum capability of 64 ports.

Proposal #11: We propose to not consider 128 ports for typeI-SinglePanel-r19 requirements.

Issue 1-2-16: Row configuration of CSI-RS resources for 128 CSI-RS ports Proposals: - Option 1: RAN4 to use row#18 of Table 7.4.1.5.3-1 for 128 port CSI-RS transmission.

We are fine to RAN4 to use row#18 of Table 7.4.1.5.3-1 for 128 port CSI-RS transmission if requirements are introduced.

Proposal #12: We are fine to RAN4 to use row#18 of Table 7.4.1.5.3-1 for 128 port CSI-RS transmission if requirements are introduced.

Issue 1-2-17: typeI-codebookMode-r19 for 128 CSI-RS ports Proposals: - Option 1: Mode A

We are fine to use Mode A in typeI-codebookMode-r19 for 128 CSI-RS ports if requirements are introduced.

Proposal #13: We are fine to use Mode A in typeI-codebookMode-r19 for 128 CSI-RS ports if requirements are introduced.

# Conclusion

In this contribution we provide the view on the NR MIMO Phase 5 demodulation and CSI requirements issues. The following observations and proposals are made:

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

Proposal #1: We propose to not consider eTypeII-r19 codebook for PMI requirements.

Proposal #2: We propose to not consider typeII-Doppler-r19 codebook for PMI requirements.

Observation #6: There is no precedent in RAN4 to have UE demodulation requirements for SRS based precoders.

Proposal #3: We propose to not consider UE demodulation/CSI reporting requirements for SRS port grouping.

Proposal #4: We propose to not consider CSI reporting requirements for UE reporting enhancements for CJT.

Observation #7: Our simulated gammas vary between 15 to 36.

Proposal #5: We see Option 1 as possible solution.

Proposal #6: We propose to consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed.

Proposal #7: We propose to use MCS22 (Table 1) for typeI-SinglePanel-r19 requirements with 64 ports.

Proposal #8: We propose to use symbol 7 for ZP-CSI-RS and CSI-IM.

Proposal #9: We propose to use symbols 2, 3, 5, 6, 9, 10, 12 and 13 for NZP CSI-RS.

Proposal #10: We propose not to specify or use scaling factors for MIMO correlation matrices.

Proposal #11: We propose to not consider 128 ports for typeI-SinglePanel-r19 requirements.

Proposal #12: We are fine to RAN4 to use row#18 of Table 7.4.1.5.3-1 for 128 port CSI-RS transmission if requirements are introduced.

Proposal #13: We are fine to use Mode A in typeI-codebookMode-r19 for 128 CSI-RS ports if requirements are introduced.

# Reference

- R4-2512527, Way Forward for [116][327] NR_MIMO_demod, Samsung

- R4-2513768, Simulation results of NR MIMO Phase 5 demodulation and CSI requirements, MediaTek

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, inc.