3GPP TSG RAN WG4 #116bis	R4-2513860

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:		4.8.2

Source:				Ofinno

Title:					Analysis of RRM Requirements for Low Band CA via Switching

Document for:		Discussion

# Introduction

The core part of Rel-19 WI on Low Band Carrier Aggregation via Switching was completed at RAN#109 (September) [1]. The Big CR containing the RRM core requirements was agreed in [2], and the corresponding requirements were introduced in TS 38.133. Due to lack of time some of the issues related to the RRM core requirements were postponed for the maintenance phase as captured in the approved WI [2].

In this contribution we provide our views on some of these open issues on RRM core requirements.

# Active BWP switching delay requirements

According to the RAN4 WF [2], the impact of the low band CA switching pattern (switchingPattern-r19) on active BWP switching requirements was postponed for discussion during the maintenance stage:

Issue 1-3-9: BWP switching for SDL SCell - Proposal 1 (Ofinno): - Clarify the UE behavior by adding the following text in clause 8.6.2: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which: - is available after the completion of the BWP switching and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior by adding the following text in clause 8.6.3: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell. [Agreement]: FFS this issue in maintenance stage.

The requirements on the active BWP switching delay for single CC and multiple CCs are specified in clause 8.6 of TS 38.133.

The requirements on the DCI and timer-based BWP switching delay in clause 8.6.2 and RRC based BWP switch delay in clause 8.6.3 are also applicable to the UE configured in carrier aggregation (CA) as described in the introductory clause 8.6.1. Therefore, the requirements in clause 8.6.2 and clause 8.6.3 are also applicable to the UE configured with a low band CA switching pattern (switchingPattern-r19) in the CA operation.

However, as analysed in the following sections a clarification is needed in the active BWP switching delay requirements for PCell and SCell to account for the unavailability of the slots due to low band CA switching pattern (switchingPattern-r19).

## DCI and timer-based BWP switching delay

DCI-based BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n.

In response to the DCI command to switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

DCI-based dormant BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based dormant BWP switching delay when the BWP switch (to a dormant BWP) is performed on a single CC (e.g., SDL SCell) as follows:

For DCI-based BWP switch, if the new BWP is a dormant BWP, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive CSI-RS (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL slot occurs right after a time duration of TdormantBWPswitchDelay which starts from the beginning of DL slot n.

In response to the DCI command to switch to the dormant BWP on the SDL SCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the FDD PCell during the remaining part of the dormant BWP switch delay (TdormantBWPswitchDelay).

Timer-based BWP switching delay:

Clause 8.6.2 of TS 38.133 also defines requirements on the timer-based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

In response to bwp-InactivityTimer expiry triggering the switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

However, in all the above BWP switching scenario, the UE can only receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on a serving (e.g., PCell) cell provided that the slots are also assigned to that serving cell. When configured with switchingPattern-r19, a situation may arise that the “first slot” after the completion of the BWP in a serving cell is not available in that serving cell and is instead assigned/allocated to the other serving cell. In this case the UE may not be capable of transmitting PDSCH or receiving PUSCH in the first slot on the serving cell whose BWP is switched. To clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.2:

“If the UE is configured with switchingPattern-r19 then the first DL or UL slot in the DCI or timer-based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which:

is available after the completion of the BWP switching and

is assigned to the same serving cell on which the active BWP switching occurs.”

- Observation 1: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 2: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., switchingPattern-r19), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 1: Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the same serving cell on which the active BWP switching occurs.

## RRC based BWP switching delay

Clause 8.6.3 of TS 38.133 defines requirements on the RRC based BWP switching delay when the BWP switch is performed on a single CC (applicable to only SpCell) as follows:

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n

In response to RRC message to switch BWP on the FDD PCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the SDL SCell during the remaining part of the RRC-based active BWP switch delay.

When the UE is configured with switchingPattern-r19, then a situation similar to the one described in section 3.1 (for DCI and timer-based BWP switching) may arise in that the “first slot” after the completion of the BWP in the FDD PCell is not available in the FDD PCell, i.e., available in the SDL SCell.

Therefore, to clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.3:

“If the UE is configured with switchingPattern-r19then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.”

- Observation 3: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the RRC-based BWP switching delay on an FDD PCell, a first DL or UL slot may not be available in the FDD PCell.

- Proposal 2: Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the FDD PCell.

# RLM and BFD requirements

According to the RAN4 WF [2], the discussion on the applicability of the switching pattern (switchingPattern-r19) when RLM or BFD is triggered to the PCell was deferred to the maintenance stage:

Issue 1-4-1: RLM requirement for PCell [Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

The two issues (RLM and BFD) are separately analyzed below:

## RLM

The UE triggers radio link failure (RLF) and start RLF timer (T310) upon receiving N310 number of consecutive out sync (OOS) indications from the lower layers. The UE stops the RLF timer (T310) upon receiving N311 number of consecutive in-sync (IS) indications from the lower layers.

Due to the switching pattern, the OOS and IS evaluation periods are extended depending on the periodicity of the switching pattern. To reduce the IS evaluation period during the RLF (i.e., when T310 is running), it is desirable that the UE does not miss the RLM-RS of the PCell overlapping with time slots assigned to the SDL. To achieve this objective, the switching pattern can be suspended allowing the UE to stay on the PCell and receive the RLM-RS for fast recovery from the RLF. If the UE stops the RLF timer then the UE can resume the switching pattern.

- Proposal 3: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310).

- Proposal 4: The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310).

## BFD

The UE triggers the beam failure detection (BFD) when the downlink radio link quality exceeds a threshold corresponding to 10% block error rate (BLER) of a hypothetical PDCCH transmission. The UE estimates the downlink radio link quality over a BFD evaluation period.

Due to the switching pattern, the BFD evaluation period is extended depending on the periodicity of the switching pattern. To initiate the beam failure recovery the UE may transmit a random access. During the beam failure recovery procedure, the UE monitors new beam on the PCell. Therefore, to speed up the beam failure recovery, the UE should not miss the reference signal (RS) overlapping with time slots assigned to the SDL. To achieve this objective, upon triggering the BFD the UE can suspend the switching pattern enabling the UE to stay on the PCell and receive the RS for fast beam failure recovery. Upon detecting the new beam on the PCell, the UE can resume the switching pattern PCell. However, the details of the time when the UE shall resume the switching pattern can be further discussed.

- Proposal 5: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD).

- Proposal 6: The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS.

# Conclusion

In this contribution we have analyzed RRM requirements related to the active BWP switching delay when the UE is configured with low band CA switching pattern (switchingPattern-r19). Based on this analysis our observations and proposals are as follows:

DCI or timer-based BWP switching:

- Observation 1: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 2: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., switchingPattern-r19), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 1: Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the same serving cell on which the active BWP switching occurs.

RRC based BWP switching:

- Observation 3: Due to low band CA switching pattern (i.e., switchingPattern-r19), after the RRC-based BWP switching delay on an FDD PCell, a first DL or UL slot may not be available in the FDD PCell.

- Proposal 2: Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3:

  - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching, and

    - is assigned to the FDD PCell.

RLM and BFD requirements:

- Proposal 3: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310).

- Proposal 4: The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310).

- Proposal 5: The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD).

- Proposal 6: The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS.

# References

- RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS (Rapporteur).

- R4-2509499, “Big CR for R19 NR_LBCA_Sw RRM”, Apple.

- R4-2512119, “WF on RRM requirements for NR_LBCA_Sw”, Moderator (Apple).

- R4-2509049, “Topic summary for [116][206] NR_LBCA_Sw”, Moderator (Apple).