3GPP TSG-RAN WG4 Meeting #116bis		                		  R4-2513894

Prague, Czech Republic, Oct. 13-17, 2025

Source:	ZTE Corporation, Sanechips

Title:	Study of RAN4 impacts for RRM measurement prediction

Agenda Item:	7.12.3

Document for:	Approval

- Introduction

In RAN plenary #109 meeting, a revised WID[1] for Rel-20 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this WID are listed as follows.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this document, we will provide some views on requirements impacts based on RRM measurement prediction from the RAN4’s perspective.

- Discussion

On RRM measurement prediction, we provide our views on RRM impact on RRM measurement prediction use-case in the follow.

First, we should make a down selection of the ground truth definition for FR1. During the SI, two options have been captured in the TR and RAN4 need to do the dicision in WI. Based on the agreement made in RAN4#114 meeting, we should analyse the advantages and disadvantages for Alt 1 and Alt 2 from the follow aspects.

RAN4#114 Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

For this issue, we should discuss the definition of ground truth in FR1 and decide whether it can be reported or ideal. We have agreement in RAN4#112bis that consider conducted testing as a beseline option for testing in FR1. Based on this, we can use the transmitted power as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test. However, consider the mobility feature, fading channel is more suitable than AWGN and TE does not know when UE perform the prediction then the timing should be considered due to the time instance for predicted L3 RSRP and ground truth L3 RSRP needs to be aligned. In addition, the number of sample for L1 filtering is up to UE implementation, the margin should be considered when using different L1 filtering and L3 filtering rather than a defined L1 filtering and L3 filtering. If we use Alt 1, referring to the existing methods for FR1 conducted testing, use the transmitted power as baseline for the definition on ground truth of L3-RSRP as the definiation of ground truth, some margin should be considered compared to legacy due to L1/L3 filtering, timing, channal condition and so on. In our view, it’s hard to reach consensus on the value of margin.

Therefore, if down selection is needed in this meeting, we prefer to use Alt 2, which is more feasible for the dynamic channel conditions. For Alt 2, Ground truth for FR1 will be based on the reported measurement value under certain conditions and it is also used in FR2 according to AI BM. For this definition, we should consider how to avoid UE cheating and the solution can wait the progress in R19 AI interface beam management test.

Proposal 1: Prefer to use Alt 2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Futher discussion is needed on how to avoid UE cheating.

Issue 2-1: Impacts of measurement error on prediction accuracy Agreement: RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy: - As a baseline: - For BB error, use link level simulation to generate L3-RSRP difference as baseband error. - For RF error model, use a truncated Gaussian distribution. - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

- As to the issue for measurement error on prediction accuracy, we have reached consensus that RF and baseband errors on the measurements would impact the prediction accuracy. Based on this, we have discussed the error model for baseband error and RF error. To avoid duplicate discussions, the progress made in AI/ML BM are reused  as soon as possible. That’s to say, on the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface.

- At present, we have determined the baseline of the error model, so how to add the error model to the existing RAN2 simulation model is a problem we need to further consider. In R19 AI BM, the similar issue has been discussed and reached consensus. Therefore, the same logic can be reused when we consider how to add measurement error, including BB error and RF error in R20 AI mobility.

- During the discussion in R19 AI BM, there is a method to add the measurement error into SLS simulation, and the LLS simulation are considered to acquire baseband error. In RAN2, the SLS simulation without error has been done in R19 for AI mob, and we can use the SLS simulation from RAN2 as baseline to generate the training dataset and train the AI prediction model. Based on this, the way to add measurement error is AI BM can be used as baseline.

- In SLS simulation, SINR of each Tx beam is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 9dB which is widely used in FR1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

- RF error is modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3). A random RF error is generated for all the beams of the same frequency layer per SMTC.

- Proposal 2: Use system level channel model to generate the training dataset and train the AI prediction model, which is align with RAN2 simulation in R19.

- Proposal 3:For the simulation on AI mobility, the way to add measurement error in AI BM can be used as baseline.

- Where to add measurement errors is also a question we need to consider. Currently, there are two ways: the first is to not consider measurement errors to the model input during model training and only add measurement errors to the model input during prediction. Another approach is to incorporate measurement errors to the model input during both training and prediction. The prediction performance under these two methods is different, so we need to discuss and determine which data to add measirement errors on.

- Proposal 4: RAN4 should discuss and determine which data to add measirement errors on.

- Errors are added to the model input during the prediction

- Errors are added to the model input during both the training and prediction

Issue 2-6: Impacts from observation and prediction windows - Candidate Options: - Option 1: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed - Option 2: For intra-frequency intra-cell prediction scenarios, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed. - Option 3: In addition to OW, PW, MRRT and UE speed, RAN to also study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B

Based on the RAN2’s agreement, we should define the prediction accuray of the cell-level measurement prediction, and RSRP difference to the actual measurement should be defined. Meanwhile, measurement reduction rate is also considered as one KPI in RAN2 and it can be depotitized compared to measurement prediction accuracy.

RAN2#126 => Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured RAN2#130 The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered izln WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

RAN2 has defined measurement reduction for intra-frequency scenario, including MRRT and MRRS. Measurement reduction rate in temporal domain (MRRT) is the ratio between skipped measurement time instances and total measurement time instances, and measurement reduction rate in spatial domain (MRRS) is tha ratio between skipped beams to be measured and total beams to be measured. In our understanding, these two indicators can be used to evaluate the benefits brought by AI models in mobilioty.

According to the WID, spatial domain is not considered in Rel-20, and RAN4 only need to study the impacts of MRRT. As mentioned by other company before, only know the information of MRRT is useless because different skipping patterns of the same MRRT have different impacts on the prediction accuracy. In SI phase, even RAN2 didn’t consider measurement error in simulation, the trend of some configurations on prediction accuracy can be as a reference for RAN4. During RAN2’s discussion, they found that the prediction accuracy is affected by OW length, MRRT, and UE speed for intra-frequency temporal domain case B. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.

Observation 1: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy.

Proposal 5: Regarding temporal domain measurement prediction B, study the impact of measurement skipping pattern on prediction accuracy for the same MRRT.

In R19, RAN4 has an agreement that consider the impacts on prediction accuracy due to OW, PW, MRRT and UE speed and so on. When we discuss the impact on RRM requirements due to the length of OW and/or PW, several factors should be considered, such as L3 filtering method, prediction by L1/L3-RSRP, different patterns for prediction window and observation window, and so on. For L3 filtering method, sliding and non-sliding is different for the definiation of requirements. If sliding method is used, the prediction result can be obtained by just one additional sample, however, more than one new samples are needed if non-sliding method is used. Similar situations can also occur in different sub-use cases. For prediction based on L1-RSRP (sub-use case1 and 3), the required observation window length and prediction window length are different from those predicted by L3-RSRP(sub-use case2). In addition, there are differences in the lengths of OW and PW for the two different patterens proposed for RAN2 in temporal domain measurement prediction case B.

Intra-frequency temporal domain case B-To be discussed in Phase 2: In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) after measurement result(s) in previous prediction window is/are skipped. Example 1 of intra-frequency temporal domain case B Example 2 of intra-frequency temporal domain case B

For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.Therefore, how to study the mpacts on prediction accuracy due to OW, PW, MRRT and UE speed should be considered due to there are numerous possible scenarios/combinations. From our side, there are many configuration and the combination of various configurations can create a large number of possible scenarios. RAN4 needs to discuss whether/how to make down selection on these scenarios to analyse the impacts on prediction accuracy.

Proposal 6: For intra-frequency intra-cell prediction, RAN4 needs to discuss whether/how to make down selection on numerous scenarios to analyse the impacts on prediction accuracy.

For frequency-domain prediction, there is no need to measure the MO on another frequency layer, which means the use of GAP is eliminated and may even affect the calculation of CSSF, thereby changing the measurement delay.

When analyzing the impact of AI prediction on RRM measurement, in addition to the issue of prediction accuracy, it is also necessary to consider the impact on measurement latency. With the help of AI tools, it may not be necessary to perform actual measurements at the time when UE measurements were previously required. For intra-frequency temporal domain prediction caseB, it can be observed that UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC cannot be directly used in the measurement delay requirements as legacy. For frequency-domain prediction, there is no need to measure the MO on another frequency layer, which means the use of GAP is eliminated. Meanwhile, for inter-frequency inter-cell (frequency domain), not all configured MOs need to be measurement, just need to consider the actually measurement frequency layers when we define measurement delay requirements. Thus, the CSSF assumptions need to be updated.

Proposal 7: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are reduced, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

As to the LCM, RAN4 should LCM-related core requirements to support the LCM functionality management , if any. In traditional discussions, Life Cycle Management (LCM) typically encompasses several key stages during the measurement process as follow:

- Measurement Configuration: The network (gNB/eNB) sends measurement configuration signaling to the UE, specifying how measurements should be performed.

- Measurement Activation: The network issues an activation command, or certain trigger conditions are met, prompting the UE to begin executing the measurement task.

- Measurement Execution: The UE collects measurement data periodically or on demand as instructed.

- Measurement Reporting: According to the configured criteria, such as periodic intervals, event triggers, or threshold crossings, the UE reports the measurement results back to the network.

- Measurement Deactivation/Release: The network sends a deactivation command or releases the measurement context, which ends the measurement process

Acccording to the WID, RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management for UE sided model and NW sided model for RRM measurement prediction. Meanwhile, the similar issus on LCM framework is also discussed in RAN4 under Rel-19 AI/ML for NR air interface WI. Therefore, we think it is premature to discuss the LCM in RAN4 AI Mob, and perfer to wait more progress made by RAN2 or AI/ML for NR air interface WI.

Observation 2: RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management in Rel-20, it is premature to discuss the LCM-related core requirements in RAN4 AI Mobility.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Proposal 1: Prefer to use Alt 2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Futher discussion is needed on how to avoid UE cheating.

- Proposal 2: Use system level channel model to generate the training dataset and train the AI prediction model, which is align with RAN2 simulation in R19.

- Proposal 3:For the simulation on AI mobility, the way to add measurement error in AI BM can be used as baseline.

- Proposal 4: RAN4 should discuss and determine which data to add measirement errors on.

- Errors are added to the model input during the prediction

- Errors are added to the model input during both the training and prediction

Observation 1: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy.

Proposal 5: Regarding temporal domain measurement prediction B, study the impact of measurement skipping pattern on prediction accuracy for the same MRRT.

Proposal 6: For intra-frequency intra-cell prediction, RAN4 needs to discuss whether/how to make down selection on numerous scenarios to analyse the impacts on prediction accuracy.

Proposal 7: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are reduced, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

Observation 2: RAN2 will discuss and specify signalling and protocol aspects to enable LCM functionality management in Rel-20, it is premature to discuss the LCM-related core requirements in RAN4 AI Mobility.

- Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital