3GPP TSG RAN WG4 Meeting #116-bis	R4-2513913

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			7.6.4.2

Source:	vivo

Title:	Genernal discussion on RRM impact for HD-FDD for Ku band with FR1 numerology

Document for:	Discussion

- Introduction

In RAN#109 meeting, the WID [1] for Enhanced Requirements for NR NTN and IoT NTN Phase 2 was approved. The part impacting RRM is the HD-FDD for NTN Core Part, which aims to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies. The corresponding description of objectives is copied as follows:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

Among them, in the first phase, the focus is on the Ku band with FR1 numerology. In this contribution, we initially provide our views on the RRM impacts for this part based on the WID above.

- Discussion

2.1 Requirements related to HD-FDD introduced by RedCap NTN

In R19 NR NTN phase 3, we defined the relevant requirements for UEs capable of (e)RedCap NTN for FR1. This includes how to adapt the existing HD-FDD aspects of RedCap to the NTN scenario. Based on discussions, the impacts on HD-FDD have been defined in the following requirements:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state (reuse redcap)

- RRC Connection Release with Redirection for RedCap UE with Satellite Access (reuse redcap)

- Minimum requirement for CSI-RS based RLM/BFD

- Scheduling availability of UE performing RLM/BFD/CBD

- Minimum requirement for L1 indication

- Scheduling availability of UE performing intra-frequency/inter-frequency measurements in HD-FDD bands on FR1

- Requirements Applicability for UE Rx-Tx time difference measurements

Regarding the details defined for the impact of the above requirements on HD-FDD, the RAN4 conclusions are as follows:

For 1 and 2, the existing HD-FDD requirements for RedCap are reused.

For 4 and 7, corresponding requirement definitions are established based on the following conclusions.

#### Issue 1-3-1: The impact of HD-FDD on scheduling availability of UE related to CSI-RS and PRS Background • When UE indicates CollisionHandlingOfHDFDDOperation, RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH

For 5, RAN4 define the corresponding requirements based on the following conclusions.

#### Issue 1-3-2: The impact of HD-FDD on minimum requirement for L1 indication for RLM and LR online agreement < Agreement >: - For the impact of HD-FDD on minimum requirement for L1 indication for RLM in clause 8.1B.6 and LR in clause 8.5B.4, the legacy requirements in TN RedCap and applicable conditions can be reused. - For each RS configuration, at least one RS sample must fall with DL occasion within an indication period. - If the above condition is not met, no requirement shall be applied. - Not capture it in the spec.

For 3, the requirement applicability rule is defined as follows.

- there is no collision between the CSI-RS resources for RLM/BFD measurements and UL transmission when network indicates ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision, or - in case of collision between the CSI-RS resources for RLM/BFD measurements and UL transmission except PRACH, and ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision is not indicated by networknetwork doesn’t indicates ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision. - there is no collision between the CSI-RS resources for RLM/BFD measurements and PRACH.

For 6, it is defined according to the following rules.

#### Issue 1-3-4: The impact of HD-FDD on intra/inter measurements online agreement Agreement: - For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, regardless of the network indication.

2.2 RRM requirement on HD-FDD VSATs with Ku-band with FR1 numerology

According to the WID, specific UE types have been defined, with VSAT types 2, 3, 5, and 6 specified in TS38.101-5. In addition, a new VSAT type is introduced, which is an HD-FDD Mobile VSAT with small antennas (e.g., 20x20 cm).

For the requirements defined for UEs capable of (e)RedCap NTN for FR1 mentioned above, since they are agnostic to both bands and VSAT types, the requirements can generally be reused as is.

Proposal 1: RRM requirements for HD-FDD VSATs operating in Ku-band with FR1 numerology. The following HD-FDD-related requirement applicability conditions defined for (e)RedCap NTN for FR1 can be reused as a baseline:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state (reuse redcap)

- RRC Connection Release with Redirection for RedCap UE with Satellite Access (reuse redcap)

- Minimum requirement for CSI-RS based RLM/BFD

- Scheduling availability of UE performing RLM/BFD/CBD

- Minimum requirement for L1 indication

- Scheduling availability of UE performing intra-frequency/inter-frequency measurements in HD-FDD bands on FR1

- Requirements Applicability for UE Rx-Tx time difference measurements

2.3 Where and how to capture RRM requirements for HD-FDD VSATs with Ku-band

Regarding the HD-FDD requirements for FR1 NTN, currently, requirements are only defined for (e)RedCap NTN. There are no relevant requirements for other non-(e)RedCap NTN UEs. For UEs capable of (e)RedCap NTN for FR1, a new section has been specifically introduced in Rel-19 to capture the corresponding requirements. If we now consider defining RRM requirements for HD-FDD only for a specific UE type and for the Ku-band with FR1 numerology, one approach is to add additional descriptions in the existing NR NTN section to capture the RRM requirements for HD-FDD VSATs, specifying that this only applies to the UE type defined in the WID. Furthermore, for the HD-FDD-related requirement applicability conditions defined in (e)RedCap NTN for FR1, the requirements can be directly referenced to the corresponding sections to avoid redundant and repetitive descriptions.

Alternatively, if, based on discussions, it is determined that at least for the Ku-band with FR1 numerology, the requirements defined by RAN4 are agnostic to both the band and VSAT types, then the requirements will not include additional applicability rules restricting them to specific UE types and bands.

Proposal 2：RAN4 to discusse how to capture HD-FDD related requirements and applicability rule of requirements

- Option 1: To add additional descriptions in the existing NR NTN section to capture the HD-FDD related RRM requirements for VSAT, specifying that this only applies to the UE types defined in the WID

- Option 2: the requirements will not include additional applicability rules restricting them to specific UE types and bands if the requirements defined are agnostic to both the band and VSAT types

- Summary

Proposal 1: RRM requirements for HD-FDD VSATs operating in Ku-band with FR1 numerology. The following HD-FDD-related requirement applicability conditions defined for (e)RedCap NTN for FR1 can be reused as a baseline:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state (reuse redcap)

- RRC Connection Release with Redirection for RedCap UE with Satellite Access (reuse redcap)

- Minimum requirement for CSI-RS based RLM/BFD

- Scheduling availability of UE performing RLM/BFD/CBD

- Minimum requirement for L1 indication

- Scheduling availability of UE performing intra-frequency/inter-frequency measurements in HD-FDD bands on FR1

- Requirements Applicability for UE Rx-Tx time difference measurements

Proposal 2：RAN4 to discusse how to capture HD-FDD related requirements and applicability rule of requirements for VSAT

- Option 1: To add additional descriptions in the existing NR NTN section to capture the HD-FDD related RRM requirements for VSAT, specifying that this only applies to the UE types defined in the WID

- Option 2: the requirements will not include additional applicability rules restricting them to specific UE types and bands if the requirements defined are agnostic to both the band and VSAT type

- References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Qualcomm Incorporated