3GPP TSG-RAN WG4 Meeting #116bis				                   R4-2513938

Prague, Czech, Oct. 13-17, 2025

Source:	Sony

Title:	RF requirement for (e)Redcap with 3MHz CBW

Agenda item:	7.4.3.1

Document for:	Approval

# Background

The WI Supporting 3MHz UE CBW for (e)Redcap has been approved in RAN#109 [1], with the following objects:

In this contribution, we provide our view on the RF requirements of (e)Redcap devices.

# Spec impact in NR UE RF

To support the 3MHz UE CBW for Rel-17 Redcap and Rel-18 eRedcap, the spec impact needs to be analyzed. Considering the 3MHz has been supported by normal NR UEs, the main issues remain to identify the gap in terms of RF requirements between NR UE and (e)Redcap devices.

From system parameters aspect, as the channel raster and sync raster have been defined for normal NR UEs, (e)Redcap devices can follow the same approach as for another bandwidth. Therefore, there is no impact on the system parameters in our understanding. The only changes required is delete the sentence that 3MHz is not applicable to (e)Redcap device in Section 5.3I.

Proposal 1: Allowing 3MHz CBW by deleting the sentence in 5.3I that 3MHz is not applicable to (e)Redcap device.

From Tx requirement aspect, there is no specific treatment of (e)Redcap other than applicable power class, which is anyway not related to the UE CBW. Still, RAN4 may need to confirm the 3MHz CBW will be applied to both PC2 and PC3.

Proposal 2: RAN4 confirms no other Tx requirement impact other than confirm that the 3MHz CBW will be applied to both PC2 and PC3.

Eventually, for the Rx requirement, at least eh REFSENS for (e)Redcap device with 3MHz CBW is missing. Therefore, the main spec impact to support 3MHz CBW for (e)Redcap would be specifying the REFSENS requirement, for (e)Redcap with 2Rx, 1Rx, and HD-FDD.

Observation 1. The main impact of supporting 3MHz CBW for (e)Redcap would be specifying the REFSENS requirement, for (e)Redcap with 2Rx, 1Rx, and HD-FDD.

# REFSENS for Redcap with 3MHz

## 2Rx FD-FDD

For Redcap with 2Rx, with other bands, the same REFSENS as normal NR devices are reused with applicable bandwidth. For the example bands that are considered in the WID, e.g., n100, n106, n85, n26, n28, the REFSENS of 3MHz. Therefore, for 2Rx FD-FDD, the same reference as normal NR UE can be re-used, which results in:

Proposal 3: Re-use the 3MHz REFSENS as normal NR UE for Redcap UEs for 2Rx FD-FDD:

| Operation band | 3MHz REFSENS with HD-FDD with 2Rx |
| --- | --- |
| n26 | -99.7 |
| n28 | -100.2 |
| n85 | -99.2 |
| n100 | -102.2 |
| n106 | -99.2 |

## Single Rx

For single Rx, the the following REFSENS degradation factors are adopted since Rel-17 Redcap:

It is understood that such a value is highly dependent on the implementations and the band specific antenna and component performance. For sake of simplicity, the single RX allowance for Redcap in Rel-17 was based on the LTE cat1bis value.  For 3MHz, the same sensitivity allowance as 5MHz can be considered.

Proposal 4: Using the same sensitivity allowance value as 5MHz, e.g., 2.5 dB for single antenna port with 3MHz.

## HD-FDD

The REFSENS of HD-FDD was derived based on the FD-FDD REFSEN with some tightening factors considering the lower insertion loss in the front end.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

Since the HD-FDD REFSENS with 5MHz only defined for n26 and n28 so far, it is hard to scale the 3MHz HD-FDD REFENSE directly from the 5MHz CBW for all example bands. Therefore, we proposed to adopt same principle as shown above to determine the HD-FDD tightening factor and then apply this tightening factor on top of the 3MHz REFSENS with HD-FDD with 2Rx in proposal 2 to determine the REFSENS value of HD-FDD with 3MHz UE CBW. Based on this principle, we proposed the following tightening factor:

Proposal 5: Adopt the following HD-FDD tightening factor and REFSENS values for HD-FDD Redcap with 3MHz CBW:

| Operating band | HD-FDD Tightening factor | 3MHz REFSENS with HD-FDD with 2Rx |
| --- | --- | --- |
| n26 | 0.8 | -100.5 |
| n28 | 0.8 | -101 |
| n85 | 0.8 | -100 |
| n100 | 0 | -102.2 |
| n106 | 0.8 | -100 |

# REFSENS for eRedcap with 3MHz

For Rel-18 eRedcap that supports NOT reduced baseband bandwidth, the same REFSENS as Rel-17 Redcap is applied. For reduced baseband bandwidth eRedcap, for 15kHz SCS, the 5MHz REFSENS requirement applies to 5, 10, 15, and 20 MHz RF bandwidth as the PDSCH is limited to 5MHz regardless of the RF bandwidth. For 3MHz CBW, since it is only related with 15 kHz SCS, the REFSENS requirement of 3MHz from Redcap can be directly re-used for reduced baseband bandwidth eRedcap.

Proposal 6: Re-use the 3MHz REFSENS as Redcap for both reduced base band bandwidth and NOT reduced base band bandwidth eRedcap devices.

# Release independent of 3MHz for (e)Redcap

For 3MHz CBW support, it is proposed that it become mandatory from Rel-20 for (e)Redcap. In addition, considering the possibility that legacy devices may already be able to support 3MHz CBW, it is also benefited to allow the release independence of the feature to early releases. Since Redcap was specified in Rel-17, it is proposed to allow 3Mhz CBW support release independent from earlier release.

Proposal 7: 3MHz CBW for (e)Redcap should be mandatory from Rel-20.

Proposal 8: Allow 3MHz CBW for (e)Redcap release independent from Rel-17.

# Conclusion

In this contribution, we share our analysis on the spec impact on UE RF to support 3MHz CBW for (e)Redcap, where the following observations and proposals are given:

Observation 1. The main impact of supporting 3MHz CBW for (e)Redcap would be specifying the REFSENS requirement, for (e)Redcap with 2Rx, 1Rx, and HD-FDD.

Proposal 1: Allowing 3MHz CBW by deleting the sentence in 5.3I that 3MHz is not applicable to (e)Redcap device.

Proposal 2: RAN4 confirms no other Tx requirement impact other than confirm that the 3MHz CBW will be applied to both PC2 and PC3.

Proposal 3: Re-use the 3MHz REFSENS as normal NR UE for Redcap UEs for 2Rx FD-FDD:

| Operation band | 3MHz REFSENS with HD-FDD with 2Rx |
| --- | --- |
| n26 | -99.7 |
| n28 | -100.2 |
| n85 | -99.2 |
| n100 | -102.2 |
| n106 | -99.2 |

Proposal 4: Using the same sensitivity allowance value as 5MHz, e.g., 2.5 dB for single antenna port with 3MHz.

Proposal 5: Adopt the following HD-FDD tightening factor and REFSENS values for HD-FDD Redcap with 3MHz CBW:

| Operating band | HD-FDD Tightening factor | 3MHz REFSENS with HD-FDD with 2Rx |
| --- | --- | --- |
| n26 | 0.8 | -100.5 |
| n28 | 0.8 | -101 |
| n85 | 0.8 | -100 |
| n100 | 0 | -102.2 |
| n106 | 0.8 | -100 |

Proposal 6: Re-use the 3MHz REFSENS as Redcap for both reduced base band bandwidth and NOT reduced base band bandwidth eRedcap devices.

Proposal 7: 3MHz CBW for (e)Redcap should be mandatory from Rel-20.

Proposal 8: Allow 3MHz CBW for (e)Redcap release independent from Rel-17.

# References

- RP-252955	New WID: NR RRM enh Phase 6	Moderator (Apple)